InspectionReport

Asbestos Abatement Containment Integrity Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Polyethylene Sheeting Integrity

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all seams of the polyethylene sheeting (poly) properly overlapped (minimum 12 inches) and taped with asbestos-rated duct tape? | ☐ | ☐ | ☐ | ____________________ |
| Verify that the polyethylene sheeting is of adequate thickness (minimum 6-mil) and free from punctures, tears, or abrasions. | ☐ | ☐ | ☐ | ____________________ |
| Confirm critical barriers (e.g., doorways, windows, HVAC vents) are sealed with multiple layers of poly and taped securely. | ☐ | ☐ | ☐ | ____________________ |
| Is the floor completely covered with polyethylene sheeting, extending up the walls at least 12 inches, and properly sealed? | ☐ | ☐ | ☐ | ____________________ |
| Are walls covered with polyethylene sheeting to a height sufficient to contain all asbestos-containing materials (ACM)? | ☐ | ☐ | ☐ | ____________________ |
| Ensure poly sheeting is sealed tightly around the negative air machine exhaust duct to prevent air leakage. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all penetrations (pipes, conduits) through the poly for proper sealing with tape or caulk. | ☐ | ☐ | ☐ | ____________________ |
| Conduct a thorough visual inspection of all poly surfaces for any signs of damage or deterioration. | ☐ | ☐ | ☐ | ____________________ |
| Are sufficient quantities of appropriate repair materials (poly sheeting, tape) readily available on-site? | ☐ | ☐ | ☐ | ____________________ |
| Are doorways protected with an airlock system consisting of at least two layers of poly sheeting? | ☐ | ☐ | ☐ | ____________________ |

Negative Pressure System

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify that the negative pressure enclosure (NPE) maintains a minimum negative pressure of -0.02 inches of water column relative to surrounding areas. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the manometer or pressure gauge is functioning correctly and properly calibrated. | ☐ | ☐ | ☐ | ____________________ |
| Ensure the negative air machine (NAM) provides the required air changes per hour (ACH) as specified in the project plan. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the HEPA filter on the NAM for damage, proper seating, and loading. Replace if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Verify that the NAM exhaust is directed outside the building or to a location where re-entrainment is prevented. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the NAM is connected to a reliable power source and protected from power outages (e.g., backup generator). | ☐ | ☐ | ☐ | ____________________ |
| Test the NAM's alarm system to ensure it functions correctly in the event of pressure loss or equipment failure. | ☐ | ☐ | ☐ | ____________________ |
| Review the NAM maintenance log to ensure regular maintenance has been performed according to the manufacturer's recommendations. | ☐ | ☐ | ☐ | ____________________ |
| Observe air flow patterns within the containment to ensure proper direction towards the NAM intake. | ☐ | ☐ | ☐ | ____________________ |
| Are spare HEPA filters readily available on-site for immediate replacement? | ☐ | ☐ | ☐ | ____________________ |

Decontamination Unit (DCU)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the DCU is properly configured with a clean room, shower room, and equipment room, in that order. | ☐ | ☐ | ☐ | ____________________ |
| Ensure each chamber of the DCU has an airlock system to prevent the escape of asbestos fibers. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the shower provides adequate hot water (minimum 100°F) and cold water for effective decontamination. | ☐ | ☐ | ☐ | ____________________ |
| Verify the shower drainage system is functioning properly and connected to an appropriate wastewater disposal system. | ☐ | ☐ | ☐ | ____________________ |
| Ensure adequate supplies of soap, shampoo, and clean towels are available in the shower room. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that asbestos-contaminated clothing and equipment are properly bagged and labeled for disposal. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the DCU for cleanliness and proper maintenance. Clean regularly to prevent fiber accumulation. | ☐ | ☐ | ☐ | ____________________ |
| Are clear and concise decontamination procedures posted within the DCU? | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect the DCU structure for any damage or leaks that could compromise its effectiveness. | ☐ | ☐ | ☐ | ____________________ |
| Ensure adequate lighting is provided in all areas of the DCU for safe and effective use. | ☐ | ☐ | ☐ | ____________________ |

Air Monitoring and Clearance

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify that a written air monitoring plan is in place and being followed according to regulatory requirements. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that personal air sampling is being conducted for all employees potentially exposed to asbestos fibers. | ☐ | ☐ | ☐ | ____________________ |
| Ensure area air samples are collected outside the containment to monitor for potential fiber release. | ☐ | ☐ | ☐ | ____________________ |
| Verify that air samples are being analyzed by a qualified laboratory using Phase Contrast Microscopy (PCM) or Transmission Electron Microscopy (TEM). | ☐ | ☐ | ☐ | ____________________ |
| Review air sample results to ensure employee exposures are below the Permissible Exposure Limit (PEL) and Excursion Limit. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that clearance air monitoring is performed after abatement is complete and before the containment is removed. | ☐ | ☐ | ☐ | ____________________ |
| Verify that clearance air monitoring meets the required criteria (e.g., <0.01 f/cc by TEM) before containment removal. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that all air monitoring records are properly maintained and readily available for review. | ☐ | ☐ | ☐ | ____________________ |
| Verify that air sampling equipment is properly calibrated and maintained. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that a competent person is overseeing the air monitoring program. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________