InspectionReport

Asphalt Paving Machine & Road Crew Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Mobile Equipment (Paver & Rollers)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Paver and tandem roller backup alarms are functioning and audible above ambient noise levels (min. 85 dBA). | ☐ | ☐ | ☐ | ____________________ |
| Paver horn is operational and distinct. | ☐ | ☐ | ☐ | ____________________ |
| Roller operator has clear, unobstructed visibility, or a spotter is used in blind spots. | ☐ | ☐ | ☐ | ____________________ |
| Screed is free from defects (cracks, excessive wear) that could cause uneven asphalt distribution. | ☐ | ☐ | ☐ | ____________________ |
| Hopper safety devices (e.g., locking pins, support arms) are in place and functioning correctly when the hopper is raised for maintenance. | ☐ | ☐ | ☐ | ____________________ |
| Emergency stop buttons on the paver are clearly marked, accessible, and tested before each shift. | ☐ | ☐ | ☐ | ____________________ |
| Roller seatbelts are present, undamaged, and used by the operator. | ☐ | ☐ | ☐ | ____________________ |
| No visible hydraulic fluid, fuel, or oil leaks are present on the paver or rollers. | ☐ | ☐ | ☐ | ____________________ |
| Paver tires are properly inflated and free from cuts, bulges, or excessive wear. | ☐ | ☐ | ☐ | ____________________ |
| Roller drums are clean and free of excessive asphalt buildup that could affect compaction. | ☐ | ☐ | ☐ | ____________________ |
| Paver headlights and taillights are functioning properly, especially for nighttime operations. | ☐ | ☐ | ☐ | ____________________ |
| Roller parking brake effectively holds the machine on an incline. | ☐ | ☐ | ☐ | ____________________ |

Site Conditions & Traffic Control

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| A site-specific traffic control plan is in place, communicated to all workers, and effectively separates workers from public traffic (MUTCD compliant). | ☐ | ☐ | ☐ | ____________________ |
| Warning signs (e.g., 'Hot Asphalt', 'Construction Ahead') are clearly visible and properly placed. | ☐ | ☐ | ☐ | ____________________ |
| Pedestrian barriers are in place to prevent unauthorized access to the work zone. | ☐ | ☐ | ☐ | ____________________ |
| Adequate lighting is provided for nighttime or low-light conditions (minimum 5 foot-candles). | ☐ | ☐ | ☐ | ____________________ |
| Clear escape routes are identified and maintained in case of emergencies. | ☐ | ☐ | ☐ | ____________________ |
| Work area is free from tripping hazards (e.g., tools, debris, materials). | ☐ | ☐ | ☐ | ____________________ |
| Slopes adjacent to the paving area are stable and properly graded to prevent collapses. | ☐ | ☐ | ☐ | ____________________ |
| Underground utilities have been located and marked before excavation or paving. | ☐ | ☐ | ☐ | ____________________ |
| Workers maintain a safe distance from operating rollers and pavers (minimum 10 feet unless directly involved in the operation). | ☐ | ☐ | ☐ | ____________________ |
| Clear communication methods (e.g., hand signals, radios) are established and understood by all workers. | ☐ | ☐ | ☐ | ____________________ |
| Asphalt and other materials are stored in designated areas, away from traffic and pedestrian routes. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| All workers wear high-visibility safety apparel (ANSI/ISEA 107 Class 2 or 3) appropriate for the work environment. | ☐ | ☐ | ☐ | ____________________ |
| Workers wear safety glasses with side shields to protect against asphalt splash and debris. | ☐ | ☐ | ☐ | ____________________ |
| Workers handling hot asphalt wear heat-resistant gloves. | ☐ | ☐ | ☐ | ____________________ |
| Workers wear steel-toe boots that meet ASTM F2413 standards. | ☐ | ☐ | ☐ | ____________________ |
| Workers wear long-sleeved shirts and pants to protect against asphalt burns and UV exposure. | ☐ | ☐ | ☐ | ____________________ |
| Workers wear hard hats that meet ANSI Z89.1 standards. | ☐ | ☐ | ☐ | ____________________ |
| Respiratory protection (e.g., N95 mask) is available and used when asphalt fume exposure exceeds permissible exposure limits (PELs). | ☐ | ☐ | ☐ | ____________________ |
| Workers are encouraged to apply sunscreen with an SPF of 30 or higher. | ☐ | ☐ | ☐ | ____________________ |
| PPE is clean, in good condition, and properly fitted. | ☐ | ☐ | ☐ | ____________________ |
| An eyewash station is readily available in case of asphalt splash in the eyes. | ☐ | ☐ | ☐ | ____________________ |

Heat Illness Prevention

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| An adequate supply of cool, potable water (at least 1 quart per worker per hour) is readily available. | ☐ | ☐ | ☐ | ____________________ |
| Shade is available for workers to take breaks during hot weather. | ☐ | ☐ | ☐ | ____________________ |
| A heat acclimatization program is in place for new workers or those returning after an absence. | ☐ | ☐ | ☐ | ____________________ |
| Work-rest cycles are implemented to reduce heat exposure during peak heat hours. | ☐ | ☐ | ☐ | ____________________ |
| A buddy system is in place to monitor workers for signs of heat illness. | ☐ | ☐ | ☐ | ____________________ |
| Workers are trained on the signs and symptoms of heat illness and how to respond. | ☐ | ☐ | ☐ | ____________________ |
| Workers are encouraged to drink water frequently, even if they don't feel thirsty. | ☐ | ☐ | ☐ | ____________________ |
| Workers with pre-existing medical conditions are closely monitored for heat stress. | ☐ | ☐ | ☐ | ____________________ |
| Electrolyte replacement drinks are available to workers. | ☐ | ☐ | ☐ | ____________________ |
| Ambient temperature and humidity are monitored to assess heat stress risk. | ☐ | ☐ | ☐ | ____________________ |

Documentation & Training

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Material Safety Data Sheets (MSDS) for asphalt and other chemicals are readily available. | ☐ | ☐ | ☐ | ____________________ |
| Daily equipment inspection logs are completed and maintained. | ☐ | ☐ | ☐ | ____________________ |
| The traffic control plan is documented and readily available on site. | ☐ | ☐ | ☐ | ____________________ |
| A competent person is designated and trained to conduct site safety inspections. | ☐ | ☐ | ☐ | ____________________ |
| Workers are trained in basic first aid and CPR. | ☐ | ☐ | ☐ | ____________________ |
| Equipment operators are trained and certified to operate the specific equipment they are using. | ☐ | ☐ | ☐ | ____________________ |
| Workers are trained on hazard communication and chemical safety. | ☐ | ☐ | ☐ | ____________________ |
| An emergency contact list is posted and readily accessible. | ☐ | ☐ | ☐ | ____________________ |
| Regular safety meeting minutes are documented and available for review. | ☐ | ☐ | ☐ | ____________________ |
| Incident reporting procedures are established and followed. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________