InspectionReport

High-Rise Facade Cleaning Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Davit Arm and Anchor Point Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the davit arm certified by a qualified engineer within the last 12 months? (Review certification documentation) | ☐ | ☐ | ☐ | ____________________ |
| Perform a visual inspection of the davit arm for any signs of corrosion, cracks, or deformation. | ☐ | ☐ | ☐ | ____________________ |
| Verify the davit arm's load rating is sufficient for the intended load, including personnel, equipment, and a safety factor of at least 5:1. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the davit arm's mounting points for secure attachment to the building structure. Check for loose bolts, damaged welds, or compromised structural integrity. | ☐ | ☐ | ☐ | ____________________ |
| Is the anchor point certified by a qualified engineer within the last 12 months? (Review certification documentation) | ☐ | ☐ | ☐ | ____________________ |
| Conduct a visual inspection of the anchor point for any signs of corrosion, cracks, or deformation. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the anchor point's load rating is sufficient for the intended load, including personnel, equipment, and a safety factor of at least 5:1. | ☐ | ☐ | ☐ | ____________________ |
| Ensure the anchor point is readily accessible and free from obstructions that could hinder its use. | ☐ | ☐ | ☐ | ____________________ |
| Verify the davit arm rotates freely and smoothly without any binding or excessive force. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the davit arm's locking mechanism to ensure it securely locks the arm in the desired position. | ☐ | ☐ | ☐ | ____________________ |
| Verify the anchor point is located in a position that minimizes swing fall hazards. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the anchor point is clearly labeled with its load rating and date of last inspection. | ☐ | ☐ | ☐ | ____________________ |

Rope Descent System (RDS) Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect the working rope and safety line for cuts, abrasions, fraying, chemical damage, or excessive wear along their entire length. | ☐ | ☐ | ☐ | ____________________ |
| Verify the rope diameter meets the manufacturer's specifications and is compatible with the descent control device. | ☐ | ☐ | ☐ | ____________________ |
| Inspect rope terminations (knots, splices, swaged fittings) for proper construction and security. | ☐ | ☐ | ☐ | ____________________ |
| Examine the descent control device for proper function, smooth operation, and any signs of damage or wear. | ☐ | ☐ | ☐ | ____________________ |
| Verify the backup device (e.g., rope grab) is properly installed and functions correctly, arresting a fall immediately. | ☐ | ☐ | ☐ | ____________________ |
| Inspect carabiners for proper locking mechanism function, gate closure, and any signs of damage or deformation. Ensure they are auto-locking and double-locking. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the full-body harness for cuts, abrasions, tears, or chemical damage. Verify proper fit and adjustment. | ☐ | ☐ | ☐ | ____________________ |
| Ensure rope protectors are in place to prevent abrasion at edges and corners. | ☐ | ☐ | ☐ | ____________________ |
| Verify the rope length is sufficient to reach the lowest intended work point with adequate slack. | ☐ | ☐ | ☐ | ____________________ |
| Confirm proper rope management techniques are in place to prevent entanglement or snagging. | ☐ | ☐ | ☐ | ____________________ |
| Verify all components of the RDS are compatible with each other and meet the manufacturer's specifications. | ☐ | ☐ | ☐ | ____________________ |
| Review the rope log to ensure the rope has not exceeded its service life or been subjected to any damaging events. | ☐ | ☐ | ☐ | ____________________ |

Public Protection Measures

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Ensure adequate barricades are in place to prevent pedestrian access to the work area below. | ☐ | ☐ | ☐ | ____________________ |
| Verify warning signs are prominently displayed to alert pedestrians of overhead work. | ☐ | ☐ | ☐ | ____________________ |
| Inspect debris netting for proper installation and integrity to prevent falling objects from reaching the ground. | ☐ | ☐ | ☐ | ____________________ |
| Confirm a designated spotter is in place to monitor the work area and warn pedestrians of potential hazards. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all tools and equipment are properly tethered to prevent them from falling. | ☐ | ☐ | ☐ | ____________________ |
| Verify clear communication protocols are in place between the workers and the spotter. | ☐ | ☐ | ☐ | ____________________ |
| Define a clear drop zone and ensure all personnel are aware of its boundaries. | ☐ | ☐ | ☐ | ____________________ |
| Assess wind conditions and suspend operations if winds exceed safe limits (typically 25 mph). | ☐ | ☐ | ☐ | ____________________ |
| Review the weather forecast for any potential hazards (e.g., thunderstorms, high winds) and adjust work plans accordingly. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the emergency plan is in place and all personnel are familiar with its procedures. | ☐ | ☐ | ☐ | ____________________ |
| Ensure a fully stocked first-aid kit is readily available on site. | ☐ | ☐ | ☐ | ____________________ |
| Monitor pedestrian traffic flow and adjust barricades or spotter positioning as needed. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect hard hats for cracks, dents, or other damage. Ensure they meet ANSI Z89.1 standards. | ☐ | ☐ | ☐ | ____________________ |
| Verify safety glasses or goggles are worn to protect against debris and chemical splashes. Ensure they meet ANSI Z87.1 standards. | ☐ | ☐ | ☐ | ____________________ |
| Ensure appropriate gloves are worn to protect against cuts, abrasions, and chemical exposure. Check for tears or punctures. | ☐ | ☐ | ☐ | ____________________ |
| Verify workers are wearing slip-resistant footwear with ankle support. Check for worn soles or damaged uppers. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the full-body harness for cuts, abrasions, tears, or chemical damage. Verify proper fit and adjustment. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the lanyard for cuts, abrasions, or damage to the snap hooks. Ensure it is properly connected to the harness and anchor point. | ☐ | ☐ | ☐ | ____________________ |
| Verify all tools are properly tethered to prevent them from falling. Check the tether for damage or wear. | ☐ | ☐ | ☐ | ____________________ |
| Ensure workers are wearing appropriate clothing for the weather conditions (e.g., rain gear, thermal layers). | ☐ | ☐ | ☐ | ____________________ |
| Verify PPE is clean and properly maintained. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all PPE fits properly and is comfortable for the worker. | ☐ | ☐ | ☐ | ____________________ |
| Confirm all workers have been trained on the proper use and maintenance of their PPE. | ☐ | ☐ | ☐ | ____________________ |
| If required, ensure appropriate respiratory protection is worn and properly fitted. (e.g., for chemical cleaning). | ☐ | ☐ | ☐ | ____________________ |

Documentation and Training

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify a Job Hazard Analysis (JHA) has been completed and reviewed with all workers. | ☐ | ☐ | ☐ | ____________________ |
| Confirm a rescue plan is in place and all workers are trained on its procedures. | ☐ | ☐ | ☐ | ____________________ |
| Verify a competent person is designated on site to oversee the work and ensure safety compliance. | ☐ | ☐ | ☐ | ____________________ |
| Review training records to ensure all workers are properly trained in fall protection, rope descent systems, and hazard communication. | ☐ | ☐ | ☐ | ____________________ |
| Inspect equipment inspection logs to ensure all equipment has been inspected and is in good working order. | ☐ | ☐ | ☐ | ____________________ |
| Ensure Material Safety Data Sheets (MSDS) are readily available for all chemicals used on site. | ☐ | ☐ | ☐ | ____________________ |
| Verify all required permits (e.g., street closure permits) are obtained and displayed. | ☐ | ☐ | ☐ | ____________________ |
| Confirm a communication plan is in place to address emergencies and routine communication. | ☐ | ☐ | ☐ | ____________________ |
| Verify a site-specific safety plan is in place and addresses the unique hazards of the location. | ☐ | ☐ | ☐ | ____________________ |
| Ensure an emergency contact list is readily available with phone numbers for emergency services, supervisors, and building management. | ☐ | ☐ | ☐ | ____________________ |
| Confirm a daily safety briefing is conducted before work begins to review hazards and safety procedures. | ☐ | ☐ | ☐ | ____________________ |
| Ensure equipment manuals are available on site for reference. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________