InspectionReport

Trenching & Excavation Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Pre-Excavation Assessment & Planning

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a 'Competent Person' (as defined by OSHA 1926.650(b)) identified and present on site? | ☐ | ☐ | ☐ | ____________________ |
| Have all underground utilities (gas, electric, water, sewer) been located and marked prior to excavation (811 One-Call system)? | ☐ | ☐ | ☐ | ____________________ |
| Has a 'Competent Person' classified the soil type (Type A, B, or C) according to OSHA 1926 Appendix A? | ☐ | ☐ | ☐ | ____________________ |
| Is the protective system (sloping, shoring, trench box) designed by a registered professional engineer if deeper than 20 feet, or if conditions warrant? | ☐ | ☐ | ☐ | ____________________ |
| Are safe means of egress (ladder, ramp, stairs) provided every 25 feet of lateral travel in the trench? | ☐ | ☐ | ☐ | ____________________ |
| Is there a system in place to control water accumulation in the trench (e.g., pumps, drainage)? | ☐ | ☐ | ☐ | ____________________ |
| Is atmospheric testing conducted before entry if a hazardous atmosphere (e.g., low oxygen, toxic gases) could reasonably be expected? | ☐ | ☐ | ☐ | ____________________ |
| Is a daily inspection of the excavation, protective systems, and adjacent areas conducted by the 'Competent Person'? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency rescue procedures and equipment (e.g., confined space rescue) readily available? | ☐ | ☐ | ☐ | ____________________ |
| Is there a site-specific excavation safety plan readily available and understood by all workers? | ☐ | ☐ | ☐ | ____________________ |

Trench Shield (Trench Box) Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the trench shield (trench box) properly certified for the depth and soil conditions? | ☐ | ☐ | ☐ | ____________________ |
| Is the trench shield free from damage (e.g., cracks, bends, corrosion) that could compromise its structural integrity? | ☐ | ☐ | ☐ | ____________________ |
| Is the trench shield installed according to the manufacturer's specifications and OSHA requirements? | ☐ | ☐ | ☐ | ____________________ |
| Is the trench shield placed so that the bottom edge extends at least 18 inches below the top of the excavation? | ☐ | ☐ | ☐ | ____________________ |
| Are voids between the trench shield and the excavation wall properly backfilled to prevent soil movement? | ☐ | ☐ | ☐ | ____________________ |
| Is safe access and egress provided to and from the trench shield? | ☐ | ☐ | ☐ | ____________________ |
| Are lifting lugs on the trench shield in good condition and properly rated for the weight of the box? | ☐ | ☐ | ☐ | ____________________ |
| Are manufacturer's specifications for the trench box readily available on site? | ☐ | ☐ | ☐ | ____________________ |
| Are trench box panels securely pinned or connected per manufacturer's instructions? | ☐ | ☐ | ☐ | ____________________ |
| Is there visual proof (photos/video) that the trench box is installed correctly before crews enter? | ☐ | ☐ | ☐ | ____________________ |

Spoil Pile Management & Site Conditions

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the spoil pile located at least 2 feet (0.6 meters) from the edge of the excavation? | ☐ | ☐ | ☐ | ____________________ |
| Is the spoil pile stable and prevented from sliding or rolling back into the excavation? | ☐ | ☐ | ☐ | ____________________ |
| Are surface encumbrances (e.g., rocks, trees, debris) that could create a hazard removed or supported? | ☐ | ☐ | ☐ | ____________________ |
| Is equipment or machinery prevented from operating near the edge of the excavation unless precautions are taken to prevent cave-ins? | ☐ | ☐ | ☐ | ____________________ |
| Are warning systems (e.g., barricades, signs) in place to alert workers and the public to the excavation hazard? | ☐ | ☐ | ☐ | ____________________ |
| Are adjacent structures (buildings, roadways) protected from the effects of the excavation? | ☐ | ☐ | ☐ | ____________________ |
| Are weather conditions (rain, freezing temperatures) monitored for their potential impact on soil stability? | ☐ | ☐ | ☐ | ____________________ |
| Is water being effectively removed from the excavation to prevent soil saturation and instability? | ☐ | ☐ | ☐ | ____________________ |
| Is adequate traffic control in place to protect workers from vehicular traffic? | ☐ | ☐ | ☐ | ____________________ |
| Is ground vibration from nearby equipment or traffic monitored to prevent soil destabilization? | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all workers wearing properly fitted hard hats (ANSI Z89.1)? | ☐ | ☐ | ☐ | ____________________ |
| Are all workers wearing appropriate eye protection (ANSI Z87.1) for the hazards present (e.g., flying debris, chemical splash)? | ☐ | ☐ | ☐ | ____________________ |
| Are all workers wearing high-visibility clothing (ANSI 107) if exposed to vehicular traffic? | ☐ | ☐ | ☐ | ____________________ |
| Are all workers wearing appropriate foot protection (ANSI Z41) for the hazards present (e.g., puncture hazards, falling objects)? | ☐ | ☐ | ☐ | ____________________ |
| Is respiratory protection (NIOSH approved) required and used if a hazardous atmosphere is present? | ☐ | ☐ | ☐ | ____________________ |
| Are workers wearing appropriate gloves for the tasks being performed (e.g., cut-resistant, chemical-resistant)? | ☐ | ☐ | ☐ | ____________________ |
| Is hearing protection available and used if noise levels exceed permissible exposure limits (PEL)? | ☐ | ☐ | ☐ | ____________________ |
| Are fall protection harnesses inspected before each use for damage or defects? | ☐ | ☐ | ☐ | ____________________ |
| Is all PPE maintained in good condition and replaced when damaged or worn? | ☐ | ☐ | ☐ | ____________________ |
| Have all workers been trained on the proper use, care, and limitations of their PPE? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________