InspectionReport

CNC Machining Center & Lathe Shop Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Machine Guarding and Safety Devices

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify all interlocked guards are functioning correctly, preventing machine operation when open. Test each guard daily. | ☐ | ☐ | ☐ | ____________________ |
| Inspect spindle guards for damage (cracks, dents, missing parts) that could compromise protection from rotating parts and ejected material. | ☐ | ☐ | ☐ | ____________________ |
| Test all E-stop buttons to ensure immediate machine shutdown. Verify proper labeling and accessibility of E-stops. | ☐ | ☐ | ☐ | ____________________ |
| If equipped, confirm light curtains are operational and properly positioned to prevent access to hazardous areas during machine operation. Test response time. | ☐ | ☐ | ☐ | ____________________ |
| For machines with two-hand controls, verify simultaneous operation is required to initiate and maintain machine function. Check for bypass mechanisms. | ☐ | ☐ | ☐ | ____________________ |
| If applicable, ensure lathe chucks have a key interlock system or self-ejecting key to prevent accidental machine starts with the key in place. | ☐ | ☐ | ☐ | ____________________ |
| Confirm fixed barrier guards are of adequate height and construction to prevent reaching over or around hazardous areas (minimum 6 feet recommended). | ☐ | ☐ | ☐ | ____________________ |
| Verify guards are designed to prevent easy adjustment or removal without tools, ensuring they remain in place during operation. | ☐ | ☐ | ☐ | ____________________ |
| If equipped with presence-sensing device initiation (PSDI), ensure proper setup and daily testing according to manufacturer's specifications. | ☐ | ☐ | ☐ | ____________________ |
| Inspect for any signs of interlock bypass or tampering. Implement procedures to prevent unauthorized modification of safety devices. | ☐ | ☐ | ☐ | ____________________ |

Coolant Mist and Air Quality

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify coolant mist collectors are operating effectively, with no visible mist escaping into the work area. Check filter pressure and airflow. | ☐ | ☐ | ☐ | ____________________ |
| Review the mist collector filter maintenance schedule and confirm filters are replaced or cleaned according to manufacturer's recommendations. | ☐ | ☐ | ☐ | ____________________ |
| Assess general ventilation in the shop to ensure adequate air exchange and removal of coolant mist and fumes. Use smoke tubes to visualize airflow. | ☐ | ☐ | ☐ | ____________________ |
| Check coolant concentration levels to ensure they are within the manufacturer's recommended range. Improper concentration can increase mist generation. | ☐ | ☐ | ☐ | ____________________ |
| Verify the coolant type is compatible with the materials being machined and the machine's components to prevent corrosion or other issues. | ☐ | ☐ | ☐ | ____________________ |
| Conduct periodic air quality monitoring for coolant mist and other contaminants to ensure compliance with OSHA permissible exposure limits (PELs). | ☐ | ☐ | ☐ | ____________________ |
| Inspect mist collector ductwork for leaks, damage, or obstructions that could reduce its effectiveness. | ☐ | ☐ | ☐ | ____________________ |
| Verify the coolant sump is cleaned regularly to prevent bacterial growth and reduce mist generation. Follow proper disposal procedures for used coolant. | ☐ | ☐ | ☐ | ____________________ |
| Confirm operators are trained on proper coolant handling, mist control techniques, and the use of mist collection equipment. | ☐ | ☐ | ☐ | ____________________ |
| Ensure the mist collector is properly grounded to prevent static electricity buildup, especially in areas with flammable coolants. | ☐ | ☐ | ☐ | ____________________ |

Chip Management and Housekeeping

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the chip conveyor is functioning correctly and effectively removing chips from the machine. Check for jams or obstructions. | ☐ | ☐ | ☐ | ____________________ |
| Ensure chip containers are properly labeled, emptied regularly, and stored in a designated area away from walkways and exits. | ☐ | ☐ | ☐ | ____________________ |
| Inspect chips for sharp edges or burrs and implement procedures for safe handling and disposal to prevent cuts and abrasions. | ☐ | ☐ | ☐ | ____________________ |
| Maintain clean and dry floors around machines to prevent slips, trips, and falls. Clean up coolant spills and chip accumulation immediately. | ☐ | ☐ | ☐ | ____________________ |
| Ensure tools are stored in designated areas and organized to prevent tripping hazards and accidental contact with sharp edges. | ☐ | ☐ | ☐ | ____________________ |
| Use compressed air safely for chip removal, with appropriate PPE (eye protection) and at a reduced pressure (below 30 psi) to prevent injury. | ☐ | ☐ | ☐ | ____________________ |
| Verify operators are using appropriate PPE (gloves, eye protection, etc.) when handling chips to prevent cuts, abrasions, and eye injuries. | ☐ | ☐ | ☐ | ____________________ |
| Implement procedures to prevent fire hazards associated with oily chips, including proper storage and disposal methods. | ☐ | ☐ | ☐ | ____________________ |
| Follow the manufacturer's recommended maintenance schedule for the chip conveyor to ensure its continued safe and efficient operation. | ☐ | ☐ | ☐ | ____________________ |
| Ensure walkways and access points around machines are clear of chips, tools, and other obstructions to allow for safe passage. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify all operators and personnel in the area are wearing appropriate eye protection (safety glasses with side shields or face shields) at all times. | ☐ | ☐ | ☐ | ____________________ |
| Ensure operators are using appropriate gloves for the tasks being performed, protecting against cuts, abrasions, and chemical exposure. | ☐ | ☐ | ☐ | ____________________ |
| Verify operators are wearing appropriate foot protection (safety shoes) to protect against falling objects and sharp materials. | ☐ | ☐ | ☐ | ____________________ |
| Ensure hearing protection (earplugs or earmuffs) is readily available and used in areas where noise levels exceed 85 dBA. | ☐ | ☐ | ☐ | ____________________ |
| Confirm operators are wearing appropriate clothing (no loose clothing, jewelry, or long hair) that could become entangled in rotating machinery. | ☐ | ☐ | ☐ | ____________________ |
| Inspect PPE for damage or wear and tear. Replace damaged PPE immediately. | ☐ | ☐ | ☐ | ____________________ |
| Verify operators have been trained on the proper use, care, and maintenance of PPE. | ☐ | ☐ | ☐ | ____________________ |
| If respiratory protection is required (e.g., for specific coolant types), ensure a comprehensive respiratory protection program is in place, including fit testing and medical evaluations. | ☐ | ☐ | ☐ | ____________________ |
| Provide designated storage areas for PPE to keep it clean and readily accessible. | ☐ | ☐ | ☐ | ____________________ |
| Enforce PPE requirements consistently and address any non-compliance immediately. | ☐ | ☐ | ☐ | ____________________ |

Electrical Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify all machines and electrical equipment are properly grounded to prevent electrical shock hazards. Test grounding continuity. | ☐ | ☐ | ☐ | ____________________ |
| Inspect power cords and cables for damage (fraying, cuts, exposed wires) and replace damaged cords immediately. | ☐ | ☐ | ☐ | ____________________ |
| Ensure electrical panels are accessible and clear of obstructions (minimum 3 feet of clearance). | ☐ | ☐ | ☐ | ____________________ |
| Verify electrical panels are properly labeled with circuit identification. | ☐ | ☐ | ☐ | ____________________ |
| Ensure lockout/tagout procedures are followed during maintenance and repair of machines to prevent accidental startup. | ☐ | ☐ | ☐ | ____________________ |
| Use Ground Fault Circuit Interrupters (GFCIs) for portable electrical tools and equipment used in wet or damp locations. | ☐ | ☐ | ☐ | ____________________ |
| Ensure only qualified personnel perform electrical work on machines and equipment. | ☐ | ☐ | ☐ | ____________________ |
| Conduct an arc flash hazard assessment and implement appropriate safety measures, including PPE and training. | ☐ | ☐ | ☐ | ____________________ |
| Minimize the use of extension cords and ensure they are properly sized for the load and not used as permanent wiring. | ☐ | ☐ | ☐ | ____________________ |
| Verify the emergency power-off switch is easily accessible and functioning correctly to de-energize equipment in an emergency. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________