InspectionReport

Drywall Finishing & Stilts Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Pre-Work Site Conditions

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Has a thorough floor sweep been completed to remove all debris, tools, and materials from the work area? (Documented floor sweep log available?) | ☐ | ☐ | ☐ | ____________________ |
| Are all open wall cavities and floor holes properly covered or barricaded to prevent accidental falls? (Covers secured and clearly marked?) | ☐ | ☐ | ☐ | ____________________ |
| Is there adequate lighting (minimum 5 foot-candles) throughout the work area to ensure clear visibility and prevent tripping hazards? (Measure light levels with a meter.) | ☐ | ☐ | ☐ | ____________________ |
| Are all pathways and access routes free from obstructions, ensuring safe movement while on stilts? (Minimum 3 feet wide?) | ☐ | ☐ | ☐ | ____________________ |
| Are environmental controls in place to manage dust and debris, such as ventilation or dust collection systems? (Verify system functionality.) | ☐ | ☐ | ☐ | ____________________ |
| Are drywall sheets and other materials properly staged to minimize clutter and potential tripping hazards? (Stacked securely and away from pathways?) | ☐ | ☐ | ☐ | ____________________ |
| Have all electrical cords and equipment been inspected for damage and proper grounding? (No frayed wires or missing ground prongs?) | ☐ | ☐ | ☐ | ____________________ |
| Is the general housekeeping in the area acceptable? (Waste containers available and used?) | ☐ | ☐ | ☐ | ____________________ |
| Are fire extinguishers readily accessible and fully charged? (Inspected within the last year?) | ☐ | ☐ | ☐ | ____________________ |
| Are emergency exits clearly marked and unobstructed? (Ensure clear egress routes.) | ☐ | ☐ | ☐ | ____________________ |

Drywall Stilts Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all stilt straps and buckles in good condition, showing no signs of wear, fraying, or damage? (Replace damaged straps immediately.) | ☐ | ☐ | ☐ | ____________________ |
| Do the stilt springs function smoothly and provide adequate support? (Check for binding or stiffness.) | ☐ | ☐ | ☐ | ____________________ |
| Are the footplates securely attached to the stilts and provide a stable platform? (Check for loose screws or bolts.) | ☐ | ☐ | ☐ | ____________________ |
| Does the height adjustment mechanism function properly and lock securely at the desired height? (Test locking mechanism.) | ☐ | ☐ | ☐ | ____________________ |
| Is the stilt foot grip in good condition and providing adequate traction? (Clean or replace worn grips.) | ☐ | ☐ | ☐ | ____________________ |
| Is the user's weight within the stilt's rated weight capacity? (Verify stilt rating and user weight.) | ☐ | ☐ | ☐ | ____________________ |
| Perform a stability test on the stilts before use. (Walk a short distance and check for wobbling or instability.) | ☐ | ☐ | ☐ | ____________________ |
| Is there a maintenance log for the stilts, documenting inspections and repairs? (Review log for recent maintenance.) | ☐ | ☐ | ☐ | ____________________ |
| Are all locking pins or mechanisms fully engaged and secured before use? (Double-check all locking points.) | ☐ | ☐ | ☐ | ____________________ |
| Are the stilt feet properly aligned and adjusted to the user's foot size? (Ensure a snug and comfortable fit.) | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a hard hat worn at all times in the work area? (ANSI Z89.1 compliant?) | ☐ | ☐ | ☐ | ____________________ |
| Is appropriate eye protection (safety glasses or goggles) worn to protect against dust and debris? (ANSI Z87.1 compliant?) | ☐ | ☐ | ☐ | ____________________ |
| Is a properly fitted respirator worn during sanding or other dust-generating activities? (NIOSH approved?) | ☐ | ☐ | ☐ | ____________________ |
| Are appropriate work boots worn, providing ankle support and slip resistance? (No open-toed shoes or sneakers.) | ☐ | ☐ | ☐ | ____________________ |
| Are gloves worn to protect hands from cuts, abrasions, and chemical exposure? (Appropriate for the task?) | ☐ | ☐ | ☐ | ____________________ |
| Is high-visibility clothing worn, especially in areas with mobile equipment or limited visibility? (ANSI 107 compliant?) | ☐ | ☐ | ☐ | ____________________ |
| Is hearing protection available and used in areas with high noise levels? (Exceeding 85 dBA?) | ☐ | ☐ | ☐ | ____________________ |
| Inspect all PPE for damage or defects before each use. (Replace damaged PPE immediately.) | ☐ | ☐ | ☐ | ____________________ |
| Ensure all PPE fits properly and is comfortable for the user. (Proper sizing and adjustment.) | ☐ | ☐ | ☐ | ____________________ |
| Has each worker received training on the proper use, care, and maintenance of their PPE? (Documented training records?) | ☐ | ☐ | ☐ | ____________________ |

Fall Protection Measures

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are guardrail systems in place around elevated work areas where stilts are used? (42 inches +/- 3 inches high?) | ☐ | ☐ | ☐ | ____________________ |
| If guardrails are not feasible, are personal fall arrest systems (PFAS) used, including a full-body harness, lanyard, and anchorage point? (Inspected before use?) | ☐ | ☐ | ☐ | ____________________ |
| Are anchorage points for PFAS capable of supporting at least 5,000 pounds per employee attached? (Engineered and certified?) | ☐ | ☐ | ☐ | ____________________ |
| Is the lanyard length appropriate to prevent contact with lower levels in case of a fall? (Calculate fall distance.) | ☐ | ☐ | ☐ | ____________________ |
| Is there sufficient fall clearance below the work area to prevent striking an object or lower level in the event of a fall? (Calculate required clearance.) | ☐ | ☐ | ☐ | ____________________ |
| Is there a rescue plan in place in case of a fall while using stilts? (Trained personnel and equipment available?) | ☐ | ☐ | ☐ | ____________________ |
| Are safety net systems used where other fall protection measures are not feasible? (Installed and maintained properly?) | ☐ | ☐ | ☐ | ____________________ |
| Are warning lines used to delineate areas where fall protection is required? (Placed at least 6 feet from the edge?) | ☐ | ☐ | ☐ | ____________________ |
| Are controlled access zones established to limit access to areas where fall hazards exist? (Clearly marked and enforced?) | ☐ | ☐ | ☐ | ____________________ |
| Have all workers received training on fall protection requirements and the proper use of fall protection equipment? (Documented training records?) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the full body harness before each use for any signs of damage, wear, or defects. (Remove from service if damaged.) | ☐ | ☐ | ☐ | ____________________ |

Documentation & Training

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are Safety Data Sheets (SDS) readily available for all chemicals and materials used on site? (Accessible to all employees?) | ☐ | ☐ | ☐ | ____________________ |
| Is there a written hazard communication program in place, outlining procedures for handling hazardous materials? (Review program annually?) | ☐ | ☐ | ☐ | ____________________ |
| Is a first-aid kit readily available and adequately stocked? (Trained personnel on site?) | ☐ | ☐ | ☐ | ____________________ |
| Is emergency contact information posted in a conspicuous location? (Including phone numbers for emergency services and site supervisors?) | ☐ | ☐ | ☐ | ____________________ |
| Are regular toolbox talks conducted to address specific safety hazards and reinforce safe work practices? (Documented attendance and topics?) | ☐ | ☐ | ☐ | ____________________ |
| Has a competent person been designated to inspect the job site and identify potential hazards? (Documented qualifications and responsibilities?) | ☐ | ☐ | ☐ | ____________________ |
| Are training records available for all employees who use drywall stilts, demonstrating competency in their safe operation? (Hands-on training and evaluation?) | ☐ | ☐ | ☐ | ____________________ |
| Is there a clear procedure for reporting incidents and near misses? (Encourage reporting without fear of reprisal.) | ☐ | ☐ | ☐ | ____________________ |
| Is there a site-specific safety plan that addresses the unique hazards of the project? (Reviewed and updated regularly?) | ☐ | ☐ | ☐ | ____________________ |
| Does the Lead Carpenter understand their responsibilities for ensuring site safety and compliance with regulations? (Documented understanding?) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________