InspectionReport

High-Rise Glass Glazing & Installation Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Vacuum Lifter Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect vacuum lifter pads for cuts, tears, or debris before each lift. Replace damaged pads immediately. | ☐ | ☐ | ☐ | ____________________ |
| Verify vacuum lifter battery is fully charged before starting work. Ensure sufficient charge for the day's planned lifts. | ☐ | ☐ | ☐ | ____________________ |
| Confirm vacuum gauge is functioning correctly and displays adequate vacuum pressure (minimum 80% of rated capacity) before lifting. | ☐ | ☐ | ☐ | ____________________ |
| Test audible and visual low-vacuum alarms before each use. Ensure alarms activate at the manufacturer's specified threshold. | ☐ | ☐ | ☐ | ____________________ |
| Verify the vacuum lifter's rated capacity exceeds the weight of the glass panel being lifted, with a safety factor of at least 2:1. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all vacuum hoses and connections for leaks or damage. Replace any compromised components immediately. | ☐ | ☐ | ☐ | ____________________ |
| Review vacuum lifter maintenance records to ensure regular inspections and servicing are up-to-date. | ☐ | ☐ | ☐ | ____________________ |
| Verify the emergency vacuum release mechanism is functional and easily accessible in case of power failure. | ☐ | ☐ | ☐ | ____________________ |
| Confirm all operators are trained and certified in the safe operation of the specific vacuum lifter model being used. | ☐ | ☐ | ☐ | ____________________ |
| Assess weather conditions (wind speed, rain) and halt glazing operations if conditions exceed the vacuum lifter's or glass panel's safe operating limits. | ☐ | ☐ | ☐ | ____________________ |

Wind Load & Environmental Conditions

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Determine maximum allowable wind speed for glazing operations based on glass size, vacuum lifter capacity, and site-specific conditions. Use an anemometer to measure wind speed. | ☐ | ☐ | ☐ | ____________________ |
| Monitor wind gust frequency and intensity. Suspend glazing operations if gusts exceed 75% of the maximum allowable wind speed. | ☐ | ☐ | ☐ | ____________________ |
| Review updated weather forecasts at least every 4 hours to anticipate changes in wind conditions and adjust work plans accordingly. | ☐ | ☐ | ☐ | ____________________ |
| Ensure glass panels are securely stored and protected from wind exposure before installation. Use appropriate bracing and tie-downs. | ☐ | ☐ | ☐ | ____________________ |
| Identify and mitigate potential wind obstructions (e.g., adjacent buildings) that could create localized wind turbulence. | ☐ | ☐ | ☐ | ____________________ |
| Monitor temperature extremes (high and low) that could affect vacuum lifter performance or glass panel integrity. Consult manufacturer specifications. | ☐ | ☐ | ☐ | ____________________ |
| Suspend glazing operations during heavy rain or snow to prevent slippage and reduced visibility. | ☐ | ☐ | ☐ | ____________________ |
| Monitor humidity levels, as high humidity can affect vacuum lifter performance. Consult manufacturer specifications. | ☐ | ☐ | ☐ | ____________________ |
| Verify a site-specific wind action plan is in place and communicated to all personnel involved in glazing operations. | ☐ | ☐ | ☐ | ____________________ |
| Ensure a clear emergency shutdown procedure is in place and understood by all personnel in case of sudden adverse weather conditions. | ☐ | ☐ | ☐ | ____________________ |

Edge Protection & Fall Prevention

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify perimeter guardrails are installed and properly secured on all open edges where glazing is taking place. Ensure guardrails meet OSHA height requirements (42 inches +/- 3 inches). | ☐ | ☐ | ☐ | ____________________ |
| Inspect safety nets for proper installation, adequate overlap, and sufficient clearance below. Ensure nets are free from damage or debris. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all glaziers are wearing properly fitted personal fall arrest systems (PFAS) with appropriate anchorage points. Inspect harnesses and lanyards for damage. | ☐ | ☐ | ☐ | ____________________ |
| Verify anchorage points for PFAS are certified to withstand a minimum of 5,000 pounds per employee attached or designed by a qualified person. | ☐ | ☐ | ☐ | ____________________ |
| Establish controlled access zones (CAZ) where guardrails or safety nets are not feasible. Clearly mark CAZ boundaries with warning lines or control lines. | ☐ | ☐ | ☐ | ____________________ |
| Ensure warning lines are erected at least 6 feet from the edge of the roof or working surface. Verify lines are flagged every 6 feet. | ☐ | ☐ | ☐ | ____________________ |
| Are 3.5-inch toe boards installed on all open sides to prevent tools and materials from falling (where applicable)? | ☐ | ☐ | ☐ | ____________________ |
| Provide edge protection (e.g., edge protectors, gloves) to prevent cuts and lacerations when handling glass panels. | ☐ | ☐ | ☐ | ____________________ |
| Establish and enforce a strict drop zone management plan to prevent tools and materials from falling to lower levels. | ☐ | ☐ | ☐ | ____________________ |
| Maintain good housekeeping practices to prevent slips, trips, and falls. Keep walkways clear of debris and obstructions. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify all personnel are wearing ANSI-approved hard hats in good condition. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all personnel are wearing ANSI Z87.1-approved safety glasses with side shields. | ☐ | ☐ | ☐ | ____________________ |
| Confirm glaziers are wearing cut-resistant gloves when handling glass panels. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all personnel are wearing high-visibility clothing (Class 2 or 3) that meets ANSI 107 standards. | ☐ | ☐ | ☐ | ____________________ |
| Verify all personnel are wearing steel-toed boots that meet ASTM F2413 standards. | ☐ | ☐ | ☐ | ____________________ |
| Inspect fall protection harnesses for proper fit, damage, and wear. Ensure D-rings are in good condition. | ☐ | ☐ | ☐ | ____________________ |
| Ensure personnel are wearing appropriate clothing for the weather conditions (e.g., rain gear, thermal layers). | ☐ | ☐ | ☐ | ____________________ |
| Encourage the use of sunscreen and hats to protect against sun exposure. | ☐ | ☐ | ☐ | ____________________ |
| Assess the need for respiratory protection based on site-specific conditions (e.g., dust, fumes). Provide appropriate respirators if required. | ☐ | ☐ | ☐ | ____________________ |
| Verify that PPE is properly maintained, cleaned, and stored when not in use. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________