InspectionReport

Commercial Kitchen Hood & Fire Suppression Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Hood Filter & Grease Accumulation

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are hood filters free from excessive grease buildup? Grease should not exceed 75% of filter capacity. | ☐ | ☐ | ☐ | ____________________ |
| Are hood filters correctly installed and securely in place, angled at 45 degrees? | ☐ | ☐ | ☐ | ____________________ |
| Are grease gutters and collection trays clean and free from accumulated grease? | ☐ | ☐ | ☐ | ____________________ |
| Is the grease level in the grease trap below the maximum allowable level (typically 25% capacity)? | ☐ | ☐ | ☐ | ____________________ |
| Is the grease trap free from leaks or damage? | ☐ | ☐ | ☐ | ____________________ |
| Are access panels on ductwork accessible and unobstructed for cleaning and inspection? | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect accessible ductwork for excessive grease buildup. Note any areas requiring professional cleaning. | ☐ | ☐ | ☐ | ____________________ |
| Is the exterior exhaust discharge free from obstructions and directing away from combustible materials? | ☐ | ☐ | ☐ | ____________________ |
| Are the lights illuminating the cooking surface clean and functional? | ☐ | ☐ | ☐ | ____________________ |
| Verify that the hood is constructed of approved materials (e.g., stainless steel). | ☐ | ☐ | ☐ | ____________________ |

Ansul System Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all Ansul nozzles free from grease, obstructions, and properly aimed at the cooking surfaces? | ☐ | ☐ | ☐ | ____________________ |
| Are nozzle caps intact and not damaged? | ☐ | ☐ | ☐ | ____________________ |
| Is the pressure gauge on the Ansul cylinder within the acceptable operating range (green zone)? | ☐ | ☐ | ☐ | ____________________ |
| Verify the hydrostatic test date on the Ansul cylinder. Is it within the required timeframe (typically every 12 years)? | ☐ | ☐ | ☐ | ____________________ |
| Are fusible links clean, undamaged, and properly positioned above cooking appliances? | ☐ | ☐ | ☐ | ____________________ |
| Is the manual pull station easily accessible and unobstructed? | ☐ | ☐ | ☐ | ____________________ |
| Test the manual pull station to ensure it activates the system (perform test with qualified technician). | ☐ | ☐ | ☐ | ____________________ |
| Inspect the system piping for any signs of damage, corrosion, or leaks. | ☐ | ☐ | ☐ | ____________________ |
| Verify that the system will automatically shut off gas or electrical power to appliances upon activation. | ☐ | ☐ | ☐ | ____________________ |
| Review the last inspection tag. Is the system within its required inspection interval (typically semi-annually)? | ☐ | ☐ | ☐ | ____________________ |

Electrical & Gas Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect electrical wiring near cooking equipment for damage, fraying, or exposed conductors. | ☐ | ☐ | ☐ | ____________________ |
| Ensure electrical wiring is properly protected in conduit and that the conduit is securely fastened. | ☐ | ☐ | ☐ | ____________________ |
| Check gas line connections for leaks using a soap solution. Look for bubbles. | ☐ | ☐ | ☐ | ____________________ |
| Verify the gas shut-off valve is easily accessible and functional. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all cooking appliances are properly grounded. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that extension cords are not used as permanent wiring for cooking equipment. | ☐ | ☐ | ☐ | ____________________ |
| Ensure electrical panels are accessible and free from obstructions. | ☐ | ☐ | ☐ | ____________________ |
| Verify that electrical panels are properly labeled. | ☐ | ☐ | ☐ | ____________________ |
| Test emergency lighting to ensure proper function in case of power outage. | ☐ | ☐ | ☐ | ____________________ |
| Ensure exit pathways are clear and unobstructed. | ☐ | ☐ | ☐ | ____________________ |

Documentation & Training

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are maintenance records for the hood and fire suppression system readily available? | ☐ | ☐ | ☐ | ____________________ |
| Is there a documented cleaning schedule for the hood, filters, and ductwork? | ☐ | ☐ | ☐ | ____________________ |
| Have employees been trained on the proper use of the fire suppression system and fire extinguishers? | ☐ | ☐ | ☐ | ____________________ |
| Is training documented for all employees on the use of portable fire extinguishers? | ☐ | ☐ | ☐ | ____________________ |
| Are records of fire drills maintained and up-to-date? | ☐ | ☐ | ☐ | ____________________ |
| Are previous inspection reports for the fire suppression system available for review? | ☐ | ☐ | ☐ | ____________________ |
| Is an emergency contact list posted in a visible location? | ☐ | ☐ | ☐ | ____________________ |
| Are Material Safety Data Sheets (MSDS) or Safety Data Sheets (SDS) readily available for all chemicals used in the kitchen? | ☐ | ☐ | ☐ | ____________________ |
| Verify that portable fire extinguishers have been inspected and certified within the last year. | ☐ | ☐ | ☐ | ____________________ |
| Are fire extinguishers readily accessible and properly mounted? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________