InspectionReport

Industrial Laundry & Dry Cleaning Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Steam System Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all steam valve seals inspected monthly for leaks using a calibrated ultrasonic leak detector? (Record dB levels) | ☐ | ☐ | ☐ | ____________________ |
| Are steam traps tested quarterly for proper functionality (e.g., temperature differential, visual inspection of condensate discharge)? | ☐ | ☐ | ☐ | ____________________ |
| Is insulation intact on all steam pipes and equipment to prevent contact burns? (Surface temperature < 140°F) | ☐ | ☐ | ☐ | ____________________ |
| Are pressure relief valves (PRVs) on steam boilers and pressure vessels inspected annually and tested every 3 years? (Documented inspection) | ☐ | ☐ | ☐ | ____________________ |
| Is the condensate return system free from leaks and blockages, ensuring efficient steam system operation? | ☐ | ☐ | ☐ | ____________________ |
| Is boiler water treatment program in place and followed to prevent scale buildup and corrosion? (Review chemical analysis reports) | ☐ | ☐ | ☐ | ____________________ |
| Are steam hoses inspected before each use for damage (cracks, bulges, abrasions) and properly rated for steam pressure and temperature? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency shutdown valves for the steam system clearly marked and easily accessible? | ☐ | ☐ | ☐ | ____________________ |
| Have all employees working with or near steam systems received documented training on safe operating procedures and emergency response? | ☐ | ☐ | ☐ | ____________________ |
| Are all steam pipes and equipment properly grounded to prevent static electricity buildup? | ☐ | ☐ | ☐ | ____________________ |

Dry Cleaning (PERC) Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is PERC vapor concentration monitored regularly (at least weekly) in the breathing zone using a calibrated monitoring device? (Results < 25 ppm TWA) | ☐ | ☐ | ☐ | ____________________ |
| Is the dry cleaning machine fully enclosed and properly ventilated to prevent PERC vapor leaks? | ☐ | ☐ | ☐ | ____________________ |
| Are PERC machines inspected daily for leaks (seals, hoses, connections) using a halogen leak detector? | ☐ | ☐ | ☐ | ____________________ |
| Is a spill containment system (e.g., berms, absorbent materials) in place to prevent PERC spills from reaching drains or the environment? | ☐ | ☐ | ☐ | ____________________ |
| Is PERC waste (sludge, filters) properly stored, labeled, and disposed of according to hazardous waste regulations? | ☐ | ☐ | ☐ | ____________________ |
| Are employees provided with and using appropriate PPE (gloves, respirators, eye protection) when handling PERC? | ☐ | ☐ | ☐ | ____________________ |
| Have all employees working with PERC received documented training on its hazards, safe handling procedures, and emergency response? | ☐ | ☐ | ☐ | ____________________ |
| Is a fire suppression system (e.g., CO2, FM-200) in place to protect the dry cleaning area in case of a fire? | ☐ | ☐ | ☐ | ____________________ |
| Is the dry cleaning machine maintained per manufacturer's recommendations, including regular filter changes and solvent recovery system checks? | ☐ | ☐ | ☐ | ____________________ |
| Is an emergency eyewash station readily accessible near the PERC handling area? | ☐ | ☐ | ☐ | ____________________ |

Laundry Operations & Ergonomics

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are laundry carts equipped with brakes and are they used on inclines to prevent runaway carts? | ☐ | ☐ | ☐ | ____________________ |
| Are laundry carts loaded within their rated weight capacity to prevent tipping and ergonomic strain? (Post weight limits clearly) | ☐ | ☐ | ☐ | ____________________ |
| Have employees received training on proper lifting techniques to minimize the risk of back injuries? (Documented training) | ☐ | ☐ | ☐ | ____________________ |
| Have ergonomic assessments been conducted for workstations involving repetitive tasks (e.g., folding, sorting) to identify and mitigate risk factors? | ☐ | ☐ | ☐ | ____________________ |
| Are work surfaces adjustable to accommodate different employee heights and prevent awkward postures? | ☐ | ☐ | ☐ | ____________________ |
| Are anti-fatigue mats provided in areas where employees stand for extended periods? | ☐ | ☐ | ☐ | ____________________ |
| Are mechanical assist devices (e.g., conveyors, lift tables) used to reduce manual lifting and carrying of heavy loads? | ☐ | ☐ | ☐ | ____________________ |
| Are laundry chutes properly maintained and equipped with safety interlocks to prevent accidental falls? | ☐ | ☐ | ☐ | ____________________ |
| Are laundry areas kept clean and free from clutter to prevent slips, trips, and falls? | ☐ | ☐ | ☐ | ____________________ |
| Are chemical injection systems properly calibrated and maintained to prevent over- or under-dosing of detergents and other chemicals? | ☐ | ☐ | ☐ | ____________________ |
| Are incompatible chemicals stored separately to prevent dangerous reactions? | ☐ | ☐ | ☐ | ____________________ |

Fire Prevention & Electrical Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are dryer lint traps cleaned daily to prevent lint buildup, which is a fire hazard? (Document cleaning schedule) | ☐ | ☐ | ☐ | ____________________ |
| Is there adequate clearance (3 feet) around electrical panels to allow for safe access and maintenance? | ☐ | ☐ | ☐ | ____________________ |
| Are extension cords used only temporarily and are they properly rated for the intended use? (No daisy-chaining) | ☐ | ☐ | ☐ | ____________________ |
| Are ground-fault circuit interrupters (GFCIs) installed in wet locations (e.g., near washing machines) to protect against electrical shock? | ☐ | ☐ | ☐ | ____________________ |
| Are fire extinguishers inspected monthly and serviced annually by a qualified technician? (Documented inspections) | ☐ | ☐ | ☐ | ____________________ |
| Are exit routes clearly marked and free from obstructions? | ☐ | ☐ | ☐ | ____________________ |
| Is emergency lighting functional and tested regularly to ensure adequate illumination during power outages? | ☐ | ☐ | ☐ | ____________________ |
| Are hot work permits required for welding, cutting, or grinding activities in the laundry facility? | ☐ | ☐ | ☐ | ____________________ |
| Are measures in place to control static electricity buildup, especially in areas where flammable solvents are used? | ☐ | ☐ | ☐ | ____________________ |
| Is electrical equipment inspected regularly for damage (frayed cords, loose connections) and repaired or replaced as needed? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________