InspectionReport

Supermarket Refrigeration & HVAC Leak Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Refrigerant Leak Detection & Alarm Systems

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify refrigerant alarm system is functional and calibrated per manufacturer's specifications. Test alarm response time. | ☐ | ☐ | ☐ | ____________________ |
| Confirm refrigerant sensors are correctly positioned in walk-in coolers and machine rooms, as per ASHRAE 15 guidelines. | ☐ | ☐ | ☐ | ____________________ |
| Ensure refrigerant alarms are audible throughout the store and connected to a central monitoring system. | ☐ | ☐ | ☐ | ____________________ |
| Inspect battery backup for refrigerant alarm system; verify it provides at least 24 hours of power in case of outage. | ☐ | ☐ | ☐ | ____________________ |
| Verify refrigerant type used matches the sensor calibration and system design specifications. Document refrigerant type. | ☐ | ☐ | ☐ | ____________________ |
| Review maintenance records for refrigerant alarm system; confirm annual calibration and functional testing are up-to-date. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the calibration date on each refrigerant sensor. Recalibrate if overdue according to manufacturer's recommendations. | ☐ | ☐ | ☐ | ____________________ |
| Confirm a written procedure exists for responding to refrigerant alarms, including evacuation protocols and emergency contact information. Post procedure clearly. | ☐ | ☐ | ☐ | ____________________ |
| Verify that a refrigerant usage log is maintained, documenting additions, removals, and leak repairs, as required by EPA regulations. | ☐ | ☐ | ☐ | ____________________ |
| Inspect refrigerant recovery equipment for proper functionality and certification. Ensure technicians are trained in its use. | ☐ | ☐ | ☐ | ____________________ |

Walk-in Cooler & Freezer Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Measure oxygen levels inside walk-in coolers using a calibrated oxygen meter. Ensure levels are above 19.5%. | ☐ | ☐ | ☐ | ____________________ |
| Inspect walk-in cooler door seals for damage or deterioration. Replace if compromised to prevent refrigerant leaks and maintain temperature. | ☐ | ☐ | ☐ | ____________________ |
| Examine evaporator coils for ice buildup, corrosion, and cleanliness. Clean coils as needed to maintain efficiency and prevent mold growth. | ☐ | ☐ | ☐ | ____________________ |
| Verify interior lighting in walk-in coolers is functional and provides adequate illumination for safe operation. | ☐ | ☐ | ☐ | ____________________ |
| Ensure emergency exit mechanisms inside walk-in coolers are functional and unobstructed. Test the release mechanism. | ☐ | ☐ | ☐ | ____________________ |
| Inspect floors inside walk-in coolers for ice buildup, spills, and damage. Address hazards to prevent slips and falls. | ☐ | ☐ | ☐ | ____________________ |
| Verify temperature monitoring systems are functional and accurate. Calibrate thermometers regularly. | ☐ | ☐ | ☐ | ____________________ |
| Inspect condensate drain lines for clogs or leaks. Ensure proper drainage to prevent water accumulation and mold growth. | ☐ | ☐ | ☐ | ____________________ |
| Confirm proper signage is in place, including warnings about low oxygen levels and emergency exit instructions. | ☐ | ☐ | ☐ | ____________________ |
| Verify proper air circulation within the walk-in cooler to prevent temperature stratification and ensure even cooling. | ☐ | ☐ | ☐ | ____________________ |

Condenser & HVAC Unit Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect condenser units for refrigerant leaks using an electronic leak detector. Pay close attention to connections and welds. | ☐ | ☐ | ☐ | ____________________ |
| Verify condenser fan operation; ensure fans are free from obstructions and operating at the correct speed. | ☐ | ☐ | ☐ | ____________________ |
| Inspect condenser coils for dirt, debris, and corrosion. Clean coils as needed to maintain efficiency. | ☐ | ☐ | ☐ | ____________________ |
| Examine electrical connections for tightness and corrosion. Ensure wiring is properly insulated and protected. | ☐ | ☐ | ☐ | ____________________ |
| Verify condenser units are securely mounted and properly supported to prevent vibration and damage. | ☐ | ☐ | ☐ | ____________________ |
| Inspect condensate drainage from HVAC units; ensure proper drainage to prevent water accumulation and mold growth. | ☐ | ☐ | ☐ | ____________________ |
| Ensure adequate access is maintained around condenser and HVAC units for maintenance and repairs. Clear obstructions. | ☐ | ☐ | ☐ | ____________________ |
| Inspect pressure relief valves on refrigeration systems to ensure they are not corroded or obstructed. Verify proper operation. | ☐ | ☐ | ☐ | ____________________ |
| Check compressor oil levels (if applicable) and ensure they are within the manufacturer's recommended range. | ☐ | ☐ | ☐ | ____________________ |
| Inspect vibration dampening pads or mounts on compressors and other equipment to ensure they are in good condition. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify technicians wear appropriate safety glasses or face shields when working on refrigeration systems. | ☐ | ☐ | ☐ | ____________________ |
| Ensure technicians wear chemical-resistant gloves when handling refrigerants or cleaning coils. | ☐ | ☐ | ☐ | ____________________ |
| Confirm technicians have access to and are trained in the use of appropriate respiratory protection (e.g., SCBA) when working in areas with potential refrigerant leaks or oxygen displacement. | ☐ | ☐ | ☐ | ____________________ |
| Verify technicians wear hearing protection when working near noisy equipment (e.g., compressors). | ☐ | ☐ | ☐ | ____________________ |
| Ensure technicians wear appropriate protective clothing (e.g., long sleeves, pants) to protect against skin contact with refrigerants or chemicals. | ☐ | ☐ | ☐ | ____________________ |
| Verify technicians wear safety shoes with slip-resistant soles when working in areas with potential slip hazards. | ☐ | ☐ | ☐ | ____________________ |
| Inspect PPE for damage or defects before each use. Replace damaged PPE immediately. | ☐ | ☐ | ☐ | ____________________ |
| Confirm technicians have received adequate training on the proper use, maintenance, and storage of PPE. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________