InspectionReport

Commercial Flat Roof Fall Protection Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

I. Pre-Work Planning & Documentation

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a site-specific Fall Protection Plan documented and readily available? (OSHA 1926.502(h)) | ☐ | ☐ | ☐ | ____________________ |
| Has a Competent Person been designated to supervise fall protection measures? (OSHA 1926.502(f)) | ☐ | ☐ | ☐ | ____________________ |
| Was a pre-work safety briefing conducted, covering fall hazards and control measures? (OSHA 1926.20(b)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are inspection records available for all fall protection equipment (harnesses, lanyards, anchor points)? (ANSI Z359.11) | ☐ | ☐ | ☐ | ____________________ |
| Have weather conditions (wind, rain, ice) been assessed for their impact on fall hazards? (OSHA 1926.501) | ☐ | ☐ | ☐ | ____________________ |
| Is a documented rescue plan in place in case of a fall incident? (OSHA 1926.502(d)(20)) | ☐ | ☐ | ☐ | ____________________ |
| Are relevant MSDS/SDS sheets available for all chemicals used on the roof? (OSHA 1910.1200) | ☐ | ☐ | ☐ | ____________________ |
| Are all site-specific hazards (e.g., power lines, asbestos) identified and communicated to workers? | ☐ | ☐ | ☐ | ____________________ |

II. Warning Line Systems & Safety Monitors

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are warning lines erected at least 6 feet from the roof edge? (OSHA 1926.502(f)(1)(ii)) | ☐ | ☐ | ☐ | ____________________ |
| Are warning lines flagged every 6 feet and capable of withstanding 16 pounds of force? (OSHA 1926.502(f)(1)(iii)) | ☐ | ☐ | ☐ | ____________________ |
| If mechanical equipment is used, are warning lines at least 10 feet from the edge? (OSHA 1926.502(f)(1)(ii)) | ☐ | ☐ | ☐ | ____________________ |
| Is the safety monitor competent and able to identify fall hazards? (OSHA 1926.502(k)) | ☐ | ☐ | ☐ | ____________________ |
| Can the safety monitor effectively communicate warnings to workers? (OSHA 1926.502(k)) | ☐ | ☐ | ☐ | ____________________ |
| Is the safety monitor free from other duties that could distract them from their safety responsibilities? (OSHA 1926.502(k)) | ☐ | ☐ | ☐ | ____________________ |
| Are warning lines clearly visible from all angles and in all lighting conditions? | ☐ | ☐ | ☐ | ____________________ |
| Are warning lines securely anchored to prevent displacement by wind or contact? | ☐ | ☐ | ☐ | ____________________ |
| Is the sag in the warning line minimal to prevent workers from tripping? | ☐ | ☐ | ☐ | ____________________ |

III. Personal Fall Arrest Systems (PFAS)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect harnesses for damage (cuts, abrasions, chemical exposure) before each use. (ANSI Z359.11) | ☐ | ☐ | ☐ | ____________________ |
| Inspect lanyards for frays, cuts, and proper functioning of snap hooks. (ANSI Z359.13) | ☐ | ☐ | ☐ | ____________________ |
| Verify anchor points can support 5,000 lbs per worker attached. (OSHA 1926.502(d)(15)) | ☐ | ☐ | ☐ | ____________________ |
| Are anchor points positioned to minimize swing fall hazards? (OSHA 1926.502(d)) | ☐ | ☐ | ☐ | ____________________ |
| Is there adequate fall clearance below the work area to prevent ground contact? (OSHA 1926.502(d)(16)) | ☐ | ☐ | ☐ | ____________________ |
| Are all components of the PFAS (harness, lanyard, anchor) compatible? (ANSI Z359) | ☐ | ☐ | ☐ | ____________________ |
| If using SRLs, do they retract and lock properly when tested? (ANSI Z359.14) | ☐ | ☐ | ☐ | ____________________ |
| Are harnesses properly fitted to each worker? (OSHA 1926.502) | ☐ | ☐ | ☐ | ____________________ |
| If using a horizontal lifeline, is it free from damage and properly tensioned? | ☐ | ☐ | ☐ | ____________________ |

IV. Skylight & Roof Opening Protection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all skylights protected by covers or guardrails capable of withstanding 200 lbs? (OSHA 1926.501(b)(4)(i)) | ☐ | ☐ | ☐ | ____________________ |
| Are skylight covers clearly marked with 'HOLE' or 'COVER' to prevent accidental stepping? (OSHA 1926.501(b)(4)(ii)) | ☐ | ☐ | ☐ | ____________________ |
| Are skylight covers securely fastened to prevent displacement? (OSHA 1926.501(b)(4)) | ☐ | ☐ | ☐ | ____________________ |
| Are all other roof openings (e.g., hatchways) guarded by railings or covers? (OSHA 1926.501(b)(4)) | ☐ | ☐ | ☐ | ____________________ |
| Are guardrails 42 inches (+/- 3 inches) in height with a mid-rail? (OSHA 1926.502(b)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Can guardrails withstand 200 pounds of force in any direction? (OSHA 1926.502(b)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Are toe boards installed where there is a risk of tools or materials falling on workers below? (OSHA 1926.502(j)) | ☐ | ☐ | ☐ | ____________________ |
| Are skylight covers made of durable material that will not break or crack under load? | ☐ | ☐ | ☐ | ____________________ |
| Are skylight covers highly visible, especially in low-light conditions? | ☐ | ☐ | ☐ | ____________________ |

V. General Site Conditions & Housekeeping

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are walking/working surfaces free of slip, trip, and fall hazards (debris, tools, hoses)? (OSHA 1926.25(a)) | ☐ | ☐ | ☐ | ____________________ |
| Are materials stored in a manner that does not create a fall hazard or obstruct walkways? (OSHA 1926.250(a)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are materials stored at least 6 feet away from roof edges? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Is there adequate lighting to safely perform work tasks? (OSHA 1926.56) | ☐ | ☐ | ☐ | ____________________ |
| Are ladders used for access properly secured and extended 3 feet above the landing? (OSHA 1926.1053(b)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are ladders inspected before each use for defects? (OSHA 1926.1053(b)(15)) | ☐ | ☐ | ☐ | ____________________ |
| Are extension cords in good condition and properly grounded? (OSHA 1926.405) | ☐ | ☐ | ☐ | ____________________ |
| Is a fire extinguisher readily available and inspected? (OSHA 1926.150(c)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Is a first-aid kit readily available and adequately stocked? (OSHA 1926.50(c)) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________