InspectionReport

High-Rise Scaffold Handover Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Base and Foundations

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are base plates used under all scaffold posts and are they adequately sized for the load? (OSHA 1926.451(c)(2)) | ☐ | ☐ | ☐ | ____________________ |
| If the scaffold is erected on soil, are mud sills used and properly sized to distribute the load? (OSHA 1926.451(c)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Are footings level, sound, rigid, and capable of carrying the maximum intended load without settling or displacement? (OSHA 1926.451(c)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are all locking pins and mechanisms on adjustable base plates properly engaged and secured? | ☐ | ☐ | ☐ | ____________________ |
| Assess ground conditions for signs of instability (water pooling, soft soil). | ☐ | ☐ | ☐ | ____________________ |
| Verify scaffold is plumb and level within acceptable tolerances (check with a level). | ☐ | ☐ | ☐ | ____________________ |
| If mobile, are caster brakes in good working order and engaged when stationary? | ☐ | ☐ | ☐ | ____________________ |
| If mobile, are casters locked to prevent movement during use? | ☐ | ☐ | ☐ | ____________________ |
| Confirm proper installation of diagonal bracing to prevent racking. | ☐ | ☐ | ☐ | ____________________ |
| Inspect vertical standards for damage (bends, dents, corrosion). | ☐ | ☐ | ☐ | ____________________ |

Scaffold Structure and Ties

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are scaffold ties installed at the correct vertical and horizontal spacing as per the scaffold design? (OSHA 1926.451(e)(6)(i)) | ☐ | ☐ | ☐ | ____________________ |
| Are scaffold ties properly secured to both the scaffold and the building structure? (OSHA 1926.451(e)(6)(ii)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect ties for any signs of damage or corrosion. | ☐ | ☐ | ☐ | ____________________ |
| Verify all frame connections are properly secured with appropriate locking devices. | ☐ | ☐ | ☐ | ____________________ |
| Ensure toprails are installed at the correct height (42 inches +/- 3 inches). (OSHA 1926.451(g)(4)(ii)) | ☐ | ☐ | ☐ | ____________________ |
| Ensure midrails are installed approximately halfway between the toprail and the platform surface. (OSHA 1926.451(g)(4)(iii)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect guardrails for damage (bends, breaks, corrosion). | ☐ | ☐ | ☐ | ____________________ |
| Are gaps between platform units and the uprights small enough to prevent falls? (OSHA 1926.451(b)(1)(i)) | ☐ | ☐ | ☐ | ____________________ |
| Do platform units overlap properly to prevent tripping hazards? | ☐ | ☐ | ☐ | ____________________ |
| Inspect platforms for damage (cracks, rot, corrosion). | ☐ | ☐ | ☐ | ____________________ |

Access and Egress

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is safe access provided to all scaffold platforms (ladders, stair towers)? (OSHA 1926.451(e)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are ladders set at the correct angle (4:1 ratio)? | ☐ | ☐ | ☐ | ____________________ |
| Do ladders extend at least 3 feet above the landing platform? (OSHA 1926.1053(b)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are ladders secured to prevent displacement? | ☐ | ☐ | ☐ | ____________________ |
| If stair towers are used, are they properly constructed and secured? | ☐ | ☐ | ☐ | ____________________ |
| Are stair railings installed on stair towers and in good condition? | ☐ | ☐ | ☐ | ____________________ |
| Is access to and from the scaffold free from obstructions? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency egress routes clearly identified and unobstructed? | ☐ | ☐ | ☐ | ____________________ |
| Inspect ladder rungs for damage, missing rungs, or slippery conditions. | ☐ | ☐ | ☐ | ____________________ |
| Ensure ladders are placed on stable and level surfaces. | ☐ | ☐ | ☐ | ____________________ |

Fall Protection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all workers using appropriate personal fall arrest systems (PFAS) when required? (OSHA 1926.451(g)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect harnesses for damage, wear, and proper fit. | ☐ | ☐ | ☐ | ____________________ |
| Inspect lanyards for cuts, abrasions, and proper functioning of snap hooks. | ☐ | ☐ | ☐ | ____________________ |
| Are anchorage points adequate and properly rated for fall arrest? (OSHA 1926.502(d)(15)) | ☐ | ☐ | ☐ | ____________________ |
| Is there sufficient fall clearance below the work platform to prevent striking a lower level? | ☐ | ☐ | ☐ | ____________________ |
| Are debris nets installed and properly maintained to catch falling objects? | ☐ | ☐ | ☐ | ____________________ |
| Are 3.5-inch toe boards installed on all open sides and ends of the platform? (OSHA 1926.451(h)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect toe boards for damage or missing sections. | ☐ | ☐ | ☐ | ____________________ |
| Are controlled access zones properly established and enforced? | ☐ | ☐ | ☐ | ____________________ |
| Are warning lines used correctly and at the appropriate distance from the edge? | ☐ | ☐ | ☐ | ____________________ |

Documentation and Tagging

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a scaffold design available and does it match the erected scaffold? (OSHA 1926.451(d)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Was the scaffold erected under the supervision of a Competent Person? (OSHA 1926.451(f)(7)) | ☐ | ☐ | ☐ | ____________________ |
| Is a valid inspection tag attached to the scaffold, indicating it is safe for use? (OSHA 1926.451(f)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Verify the inspection tag has not expired. | ☐ | ☐ | ☐ | ____________________ |
| If a Red Tag is present, ensure the scaffold is not in use and the deficiencies are being addressed. | ☐ | ☐ | ☐ | ____________________ |
| Is the maximum intended load capacity clearly posted on the scaffold? (OSHA 1926.451(f)(8)) | ☐ | ☐ | ☐ | ____________________ |
| Are any modifications to the scaffold approved by a Competent Person and documented? | ☐ | ☐ | ☐ | ____________________ |
| Verify that workers using the scaffold have received proper training. (OSHA 1926.454) | ☐ | ☐ | ☐ | ____________________ |
| Are daily scaffold inspection logs being maintained? | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all handover documentation is complete and accurate. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________