InspectionReport

Concrete Batch Plant Mixer Maintenance Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Confined Space Entry (Central Mixer Drum)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a Confined Space Entry Permit completed and signed by authorized personnel before entry into the central mixer drum? (OSHA 1910.146(e)) | ☐ | ☐ | ☐ | ____________________ |
| Has air quality testing been conducted inside the central mixer drum to verify oxygen levels (19.5-23.5%), absence of toxic gases (CO, H2S), and flammable vapors before entry? (OSHA 1910.146(c)(5)) | ☐ | ☐ | ☐ | ____________________ |
| Is a forced-air ventilation system in place and operating effectively to maintain safe air quality inside the central mixer drum during occupancy? (OSHA 1910.146(d)(4)) | ☐ | ☐ | ☐ | ____________________ |
| Is a trained attendant stationed outside the central mixer drum to monitor entrants and initiate rescue procedures if needed? (OSHA 1910.146(d)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Is a reliable communication system (e.g., two-way radio) established between the entrant and the attendant? (OSHA 1910.146(d)(5)) | ☐ | ☐ | ☐ | ____________________ |
| Is appropriate rescue equipment (e.g., retrieval system, harness, lifeline) readily available and in good working order? (OSHA 1910.146(d)(9)) | ☐ | ☐ | ☐ | ____________________ |
| Are emergency procedures, including contact information for emergency services, clearly posted and understood by all personnel? (OSHA 1910.146(d)(11)) | ☐ | ☐ | ☐ | ____________________ |
| Have all entrants received adequate training on confined space entry procedures, hazards, and rescue techniques? (OSHA 1910.146(g)) | ☐ | ☐ | ☐ | ____________________ |
| Has the central mixer drum been thoroughly cleaned and purged of residual concrete and materials before entry? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Is adequate lighting (explosion-proof if necessary) provided inside the central mixer drum? (OSHA 1910.146(d)(6)) | ☐ | ☐ | ☐ | ____________________ |

Lockout/Tagout (LOTO) Procedures

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a written Lockout/Tagout (LOTO) program in place and readily available? (OSHA 1910.147(c)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are all energy sources (electrical, mechanical, hydraulic, pneumatic) effectively isolated and locked out before maintenance begins on the central mixer, conveyor belts, and aggregate bins? (OSHA 1910.147(d)) | ☐ | ☐ | ☐ | ____________________ |
| Are LOTO devices (locks, tags) properly applied and clearly labeled with the employee's name and contact information? (OSHA 1910.147(c)(5)) | ☐ | ☐ | ☐ | ____________________ |
| Is energy isolation verified by attempting to start the equipment after LOTO is applied? (OSHA 1910.147(d)(6)) | ☐ | ☐ | ☐ | ____________________ |
| If multiple employees are working on the equipment, is a group LOTO procedure in place and followed? (OSHA 1910.147(f)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Have all authorized employees received training on LOTO procedures and their responsibilities? (OSHA 1910.147(c)(7)) | ☐ | ☐ | ☐ | ____________________ |
| Are contractors informed of and compliant with the plant's LOTO procedures? (OSHA 1910.147(c)(4)) | ☐ | ☐ | ☐ | ____________________ |
| Is LOTO documentation, including equipment-specific procedures, readily available and up-to-date? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Are procedures in place for the temporary removal of LOTO devices (e.g., for testing) and subsequent reapplication? (OSHA 1910.147(e)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Has all stored energy (e.g., hydraulic pressure, compressed air) been safely released or controlled before maintenance begins? (OSHA 1910.147(d)(5)) | ☐ | ☐ | ☐ | ____________________ |

Fall Protection (Aggregate Bins, Silos, Elevated Platforms)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is fall protection required for any work performed at heights of 6 feet or more? (OSHA 1926.501(b)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are guardrails with toeboards installed on all open sides of elevated platforms and walkways? (OSHA 1926.502(b)) | ☐ | ☐ | ☐ | ____________________ |
| Are personal fall arrest systems (harnesses, lanyards) inspected before each use for damage or defects? (OSHA 1926.502(d)(21)) | ☐ | ☐ | ☐ | ____________________ |
| Are anchor points for personal fall arrest systems capable of supporting 5,000 pounds per employee attached? (OSHA 1926.502(d)(15)) | ☐ | ☐ | ☐ | ____________________ |
| Is there sufficient fall clearance below the work area to prevent contact with lower levels in case of a fall? (OSHA 1926.502(d)(16)) | ☐ | ☐ | ☐ | ____________________ |
| Are ladders used to access elevated areas in good condition, properly secured, and extended at least 3 feet above the landing? (OSHA 1926.1053) | ☐ | ☐ | ☐ | ____________________ |
| Are safe access methods (e.g., ladders with cages, aerial lifts) provided for accessing aggregate bins and silos? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Have all employees working at heights received training on fall hazards and the proper use of fall protection equipment? (OSHA 1926.503) | ☐ | ☐ | ☐ | ____________________ |
| Is a rescue plan in place to promptly retrieve employees who have fallen while working at height? (OSHA 1926.502(d)(20)) | ☐ | ☐ | ☐ | ____________________ |
| Is work at height suspended during adverse weather conditions (e.g., high winds, rain, ice) that could increase the risk of falls? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |

Welding Safety (Mixer Repair)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are welders wearing appropriate personal protective equipment (PPE), including welding helmets with proper shade, gloves, and flame-resistant clothing? (ANSI Z49.1) | ☐ | ☐ | ☐ | ____________________ |
| Is a fire extinguisher readily available in the welding area? (NFPA 51B) | ☐ | ☐ | ☐ | ____________________ |
| Is a hot work permit required and obtained before welding or cutting operations begin? (NFPA 51B) | ☐ | ☐ | ☐ | ____________________ |
| Is adequate ventilation provided to remove welding fumes and gases from the work area? (ANSI Z49.1) | ☐ | ☐ | ☐ | ____________________ |
| Is the welding area properly shielded to protect other workers from arc flash and flying sparks? (ANSI Z49.1) | ☐ | ☐ | ☐ | ____________________ |
| Have all flammable materials been removed from the welding area or properly protected? (NFPA 51B) | ☐ | ☐ | ☐ | ____________________ |
| Is the welding equipment properly grounded to prevent electrical shock? (ANSI Z49.1) | ☐ | ☐ | ☐ | ____________________ |
| Are compressed gas cylinders (oxygen, acetylene) stored and handled safely, away from heat sources and secured to prevent tipping? (CGA P-1) | ☐ | ☐ | ☐ | ____________________ |
| Are welding cables inspected for damage or wear before each use? (ANSI Z49.1) | ☐ | ☐ | ☐ | ____________________ |
| Are all personnel in the welding area wearing appropriate eye protection (safety glasses or goggles)? (ANSI Z87.1) | ☐ | ☐ | ☐ | ____________________ |

Mobile Equipment (Loader, Skid Steer)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a pre-shift inspection of the loader or skid steer conducted and documented before operation? (OSHA 1926.602(a)(3)(i)) | ☐ | ☐ | ☐ | ____________________ |
| Are operators wearing seatbelts at all times while operating the loader or skid steer? (OSHA 1926.602(a)(2)(i)) | ☐ | ☐ | ☐ | ____________________ |
| Is the backup alarm on the loader or skid steer functioning properly? (OSHA 1926.602(a)(9)(ii)) | ☐ | ☐ | ☐ | ____________________ |
| Is the loader or skid steer equipped with a rollover protective structure (ROPS) and seatbelts? (OSHA 1926.1000) | ☐ | ☐ | ☐ | ____________________ |
| Are operators trained and authorized to operate the specific type of loader or skid steer being used? (OSHA 1926.20(b)(4)) | ☐ | ☐ | ☐ | ____________________ |
| Are pedestrian traffic routes clearly marked and separated from mobile equipment operating areas? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Are speed limits enforced for mobile equipment within the plant? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Is visibility maintained by ensuring clean windows and mirrors on the loader or skid steer? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Is the loader bucket or attachment properly secured and in good working condition? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Is the parking brake functioning correctly and used when the equipment is parked? (Best Practice) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________