InspectionReport

Confined Space Entry Permit Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

Jurisdiction: Adaptable. Covers OSHA 29 CFR 1910.146 (US) and UK HSE Confined Spaces Regulations 1997. This is a planning aid, not an executed permit. Apply only the section and thresholds for the governing jurisdiction; do not combine US and UK requirements as if they were one code.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Permit header

| Field | Entry |
| --- | --- |
| Permit number | ________________________________________________ |
| Date and time of issue | ________________________________________________ |
| Location of confined space | ________________________________________________ |
| Purpose of entry | ________________________________________________ |

Space identification and hazards

| Field | Entry |
| --- | --- |
| Space description (tank, vessel, pit, tunnel, duct) | ________________________________________________ |
| Hazardous atmosphere: known or potential (toxic gases, oxygen deficiency, explosive fumes) | ________________________________________________ |
| Engulfment potential (loose materials, liquids, soil) | ________________________________________________ |
| Internal configuration hazards (inwardly converging walls, sloping floors) | ________________________________________________ |
| Other recognized hazards (unguarded machinery, live electrical, extreme heat) | ________________________________________________ |

Atmospheric testing

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Oxygen content: between 19.5% and 23.5% | ☐ | ☐ | ☐ | ____________________ |
| Flammable gases and vapors: below 10% LEL | ☐ | ☐ | ☐ | ____________________ |
| Toxic air contaminants: tested based on historical contents | ☐ | ☐ | ☐ | ____________________ |
| Stratified testing: top, middle, and bottom of space | ☐ | ☐ | ☐ | ____________________ |
| Continuous monitoring during entry: Yes / No | ☐ | ☐ | ☐ | ____________________ |

Isolation and energy control

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Lockout/Tagout (LOTO): all mechanical, electrical systems locked out at source | ☐ | ☐ | ☐ | ____________________ |
| Blanking and blinding: pipes disconnected and capped | ☐ | ☐ | ☐ | ____________________ |
| Ventilation: mechanical forced-air ventilation in place, air intake from clean source | ☐ | ☐ | ☐ | ____________________ |

Personnel roles

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Entry Supervisor: name, signature, authorization | ☐ | ☐ | ☐ | ____________________ |
| Authorized Entrants: names listed | ☐ | ☐ | ☐ | ____________________ |
| Attendant (standby): name, location, never enters the space | ☐ | ☐ | ☐ | ____________________ |

Emergency rescue

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Rescue plan: non-entry rescue (full-body harness, retrieval line, tripod) | ☐ | ☐ | ☐ | ____________________ |
| Rescue team on standby: Yes / No | ☐ | ☐ | ☐ | ____________________ |
| Communication method between entrant and attendant | ☐ | ☐ | ☐ | ____________________ |
| First aid and CPR certified team member: Yes / No | ☐ | ☐ | ☐ | ____________________ |

PPE and equipment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Required PPE for entry (respirator, harness, gloves, eye protection) | ☐ | ☐ | ☐ | ____________________ |
| Gas detector / monitor: calibrated and functioning | ☐ | ☐ | ☐ | ____________________ |
| Communication equipment: functional | ☐ | ☐ | ☐ | ____________________ |

Permit closure

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| All entrants accounted for and exited | ☐ | ☐ | ☐ | ____________________ |
| Permit canceled by Entry Supervisor | ☐ | ☐ | ☐ | ____________________ |
| Time of permit closure | ☐ | ☐ | ☐ | ____________________ |
| Entry Supervisor signature | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________

References consulted

OSHA 29 CFR 1910.146 – Permit-required confined spaces

UK HSE – Confined Spaces Regulations 1997

UK HSE – Confined space entry (HSG100)

NFPA 350 – Standard for Safety and Health in Confined Spaces

Verify the current edition and local adoption before use.