InspectionReport

Grain Silo Entry Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Pre-Entry Permitting & LOTO

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Entry permit is completed, signed, and dated by authorized personnel before entry. (OSHA 1910.146(e)) | ☐ | ☐ | ☐ | ____________________ |
| Sweep auger and all mechanical equipment within the silo are completely locked out and tagged out. (OSHA 1910.272(g)(1)(i)) | ☐ | ☐ | ☐ | ____________________ |
| Electrical disconnects for all equipment are locked and tagged out at the main panel. (NFPA 70E) | ☐ | ☐ | ☐ | ____________________ |
| Atmospheric testing for oxygen levels, combustible gases, and toxic substances is completed and documented within the permit. (OSHA 1910.146(d)(5)) | ☐ | ☐ | ☐ | ____________________ |
| Communication system (two-way radio or hard-wired system) between entrant and attendant is tested and functional. (OSHA 1910.146(i)) | ☐ | ☐ | ☐ | ____________________ |
| A detailed rescue plan is documented and readily available, including contact information for emergency services. (OSHA 1910.146(d)(9)) | ☐ | ☐ | ☐ | ____________________ |
| Entry permit is posted at the entry point of the grain silo. (OSHA 1910.146(e)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Entrant and attendant have been briefed on the hazards, entry procedures, and emergency procedures. (OSHA 1910.146(h)) | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Full-body harness fits correctly and is inspected for damage (tears, cuts, corrosion) before each use. (ANSI Z359.11) | ☐ | ☐ | ☐ | ____________________ |
| Lifeline is in good condition, free from cuts, abrasions, and chemical damage. (ANSI Z359.15) | ☐ | ☐ | ☐ | ____________________ |
| Respirator (if required) is properly fit-tested and the user is trained on its use and limitations. (OSHA 1910.134) | ☐ | ☐ | ☐ | ____________________ |
| Appropriate dust mask (N95 or higher) is available and worn to minimize dust inhalation. (NIOSH Recommendations) | ☐ | ☐ | ☐ | ____________________ |
| Safety glasses or goggles are worn to protect eyes from dust and debris. (ANSI Z87.1) | ☐ | ☐ | ☐ | ____________________ |
| Appropriate footwear (steel-toed boots) is worn to protect feet from injury. (ANSI Z41) | ☐ | ☐ | ☐ | ____________________ |
| Clothing is close-fitting to prevent entanglement with equipment or grain. (General Safety Practice) | ☐ | ☐ | ☐ | ____________________ |
| Hearing protection is available and used if noise levels exceed 85 dBA. (OSHA 1910.95) | ☐ | ☐ | ☐ | ____________________ |

Silo Condition & Environment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Grain level is assessed to determine the extent of bridging or compaction. (Visual Inspection) | ☐ | ☐ | ☐ | ____________________ |
| Signs of bridging (uneven surfaces, voids) are identified and addressed before entry. (Visual Inspection) | ☐ | ☐ | ☐ | ____________________ |
| Dust accumulation on surfaces is assessed and controlled before and during entry. (NFPA 61) | ☐ | ☐ | ☐ | ____________________ |
| Ventilation is adequate to minimize dust concentration and maintain breathable air. (ACGIH Recommendations) | ☐ | ☐ | ☐ | ____________________ |
| Lighting is sufficient to provide clear visibility inside the silo. (OSHA 1926.56) | ☐ | ☐ | ☐ | ____________________ |
| Temperature and humidity are monitored to prevent heat stress or other adverse conditions. (NIOSH Recommendations) | ☐ | ☐ | ☐ | ____________________ |
| Entry point is clear of obstructions and provides safe access. (General Safety Practice) | ☐ | ☐ | ☐ | ____________________ |
| Any internal obstructions (ladders, supports) are identified and accounted for in the entry plan. (Visual Inspection) | ☐ | ☐ | ☐ | ____________________ |
| Grain condition (moisture, mold, foreign material) is assessed to identify potential hazards. (Visual Inspection) | ☐ | ☐ | ☐ | ____________________ |

Fall Protection & Rescue Equipment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Anchorage point for lifeline is rated for at least 5,000 lbs per person attached. (OSHA 1926.502(d)(15)) | ☐ | ☐ | ☐ | ____________________ |
| Lifeline is securely attached to the harness and the anchorage point. (ANSI Z359.15) | ☐ | ☐ | ☐ | ____________________ |
| Retrieval system (winch or tripod) is available and in good working order for emergency extraction. (OSHA 1910.146(k)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Rescue team is trained in grain engulfment rescue techniques and equipment operation. (OSHA 1910.146(k)) | ☐ | ☐ | ☐ | ____________________ |
| Rescue equipment (harnesses, ropes, cutting tools) is inspected before each use. (Manufacturer Recommendations) | ☐ | ☐ | ☐ | ____________________ |
| Confined space monitor is present and continuously monitoring atmospheric conditions. (OSHA 1910.146(i)) | ☐ | ☐ | ☐ | ____________________ |
| First-aid kit is readily available and stocked with necessary supplies. (ANSI Z308.1) | ☐ | ☐ | ☐ | ____________________ |
| A tagline is used to control the movement of the entrant within the silo. (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Cutting tools (e.g., grain rescue tubes) are available for creating a barrier around the engulfed person. (Grain Handling Safety Coalition Recommendations) | ☐ | ☐ | ☐ | ____________________ |

Combustible Dust Control

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| All potential ignition sources (open flames, sparks, hot surfaces) are removed from the silo area. (NFPA 61) | ☐ | ☐ | ☐ | ____________________ |
| Measures are in place to control static electricity (grounding, bonding). (NFPA 77) | ☐ | ☐ | ☐ | ____________________ |
| Housekeeping procedures are followed to minimize dust accumulation on surfaces. (NFPA 652) | ☐ | ☐ | ☐ | ____________________ |
| Dust collection system (if present) is functional and properly maintained. (NFPA 652) | ☐ | ☐ | ☐ | ____________________ |
| Explosion venting (if present) is clear of obstructions. (NFPA 68) | ☐ | ☐ | ☐ | ____________________ |
| Hot work permit is required for any activities that could generate sparks or heat. (OSHA 1910.252) | ☐ | ☐ | ☐ | ____________________ |
| Electrical equipment is rated for use in a Class II, Division 1 or 2 environment. (NEC Article 500) | ☐ | ☐ | ☐ | ____________________ |
| No smoking policy is strictly enforced in and around the grain silo. (General Safety Practice) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________