InspectionReport

Urban Building Demolition & Shoring Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Shoring System Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify shoring system load capacity matches or exceeds calculated loads from engineering survey (documented calculations available on site). | ☐ | ☐ | ☐ | ____________________ |
| Inspect all shoring members (posts, beams, bracing) for damage (cracks, bends, corrosion) before installation and daily. Replace damaged members immediately. | ☐ | ☐ | ☐ | ____________________ |
| Confirm vertical alignment of shoring posts using a plumb bob or laser level. Maximum allowable deviation: +/- 1/4 inch per 10 feet of height. | ☐ | ☐ | ☐ | ____________________ |
| Ensure shoring baseplates are in full and level contact with the supporting surface. Use shims as needed to achieve full contact. | ☐ | ☐ | ☐ | ____________________ |
| Check tightness of all shoring wedges and connections. Retighten as needed to maintain proper load transfer. Document wedge torque values. | ☐ | ☐ | ☐ | ____________________ |
| Verify splice connections in shoring members are properly installed and tightened according to manufacturer's specifications. Inspect for slippage or loosening. | ☐ | ☐ | ☐ | ____________________ |
| Monitor ground settlement around shoring footings daily. Maximum allowable settlement: 1/8 inch. Implement corrective action if settlement exceeds this limit. | ☐ | ☐ | ☐ | ____________________ |
| Confirm engineering survey of shoring system is conducted daily, prior to commencement of demolition activities. Document survey results. | ☐ | ☐ | ☐ | ____________________ |
| Inspect shoring system for weather-related damage (corrosion, water accumulation). Implement measures to protect shoring from weather exposure. | ☐ | ☐ | ☐ | ____________________ |
| Ensure safe access and egress are maintained around the shoring system. Clear pathways of obstructions and debris. | ☐ | ☐ | ☐ | ____________________ |

Demolition Sequencing & Pre-Weakening

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify demolition sequence adheres strictly to the approved demolition plan and engineering survey recommendations. Document any deviations. | ☐ | ☐ | ☐ | ____________________ |
| Confirm load-bearing walls are clearly identified and marked on site plans and in the field before any pre-weakening or demolition occurs. | ☐ | ☐ | ☐ | ____________________ |
| Ensure pre-weakening operations (e.g., controlled cuts) are performed under direct supervision of a qualified structural engineer. | ☐ | ☐ | ☐ | ____________________ |
| Verify depth and extent of pre-weakening cuts are within specified limits outlined in the demolition plan and engineering calculations. | ☐ | ☐ | ☐ | ____________________ |
| Monitor adjacent structures for signs of distress (cracks, settlement) during demolition activities. Implement corrective action if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Establish and maintain a clearly defined demolition zone with restricted access. Use barricades, signage, and spotters to prevent unauthorized entry. | ☐ | ☐ | ☐ | ____________________ |
| Ensure material removal path is clear of obstructions and adequately supported. Inspect debris chutes for damage and proper installation. | ☐ | ☐ | ☐ | ____________________ |
| Inspect debris chute connections, supports, and discharge point daily. Ensure chute is properly secured and prevents uncontrolled debris release. | ☐ | ☐ | ☐ | ____________________ |
| Verify dust control measures (water suppression, enclosures) are in place and effectively minimizing dust emissions. Monitor air quality. | ☐ | ☐ | ☐ | ____________________ |
| Implement vibration monitoring program if demolition activities may impact adjacent structures. Compare readings to acceptable limits. | ☐ | ☐ | ☐ | ____________________ |

Mobile Equipment & Heavy Machinery

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Conduct daily inspection of all mobile equipment (excavators, loaders, cranes) before operation. Document inspection results. | ☐ | ☐ | ☐ | ____________________ |
| Verify all equipment operators are properly trained and certified to operate the specific equipment being used. Maintain records of certifications. | ☐ | ☐ | ☐ | ____________________ |
| Ensure load charts are readily available and legible for all cranes and lifting equipment. Operators must understand and adhere to load limits. | ☐ | ☐ | ☐ | ____________________ |
| Identify and mitigate proximity hazards (overhead power lines, underground utilities) before operating mobile equipment. Maintain safe clearance distances. | ☐ | ☐ | ☐ | ____________________ |
| Verify backup alarms are functioning properly on all mobile equipment. Repair or replace alarms immediately if defective. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all mobile equipment is equipped with rollover protection (ROPS) and seatbelts. Operators must wear seatbelts at all times. | ☐ | ☐ | ☐ | ____________________ |
| Establish and maintain clear travel paths for mobile equipment. Minimize congestion and potential for collisions. | ☐ | ☐ | ☐ | ____________________ |
| Implement safe refueling procedures for mobile equipment. Prohibit smoking and open flames during refueling. Use spill containment measures. | ☐ | ☐ | ☐ | ____________________ |
| Verify parking brakes are engaged when mobile equipment is parked on slopes or uneven surfaces. | ☐ | ☐ | ☐ | ____________________ |
| Test the horn on all mobile equipment daily to ensure it is functioning correctly. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Ensure all personnel on site are wearing properly fitted hard hats that meet ANSI Z89.1 standards. | ☐ | ☐ | ☐ | ____________________ |
| Verify appropriate eye protection (safety glasses, goggles, face shields) is worn based on the task being performed (e.g., cutting, grinding, demolition). | ☐ | ☐ | ☐ | ____________________ |
| Ensure hearing protection (earplugs, earmuffs) is worn in areas where noise levels exceed 85 dBA. Conduct noise monitoring to determine exposure levels. | ☐ | ☐ | ☐ | ____________________ |
| Verify appropriate hand protection (gloves) is worn based on the task being performed (e.g., handling debris, operating equipment). | ☐ | ☐ | ☐ | ____________________ |
| Ensure all personnel are wearing safety shoes or boots that meet ANSI Z41 standards. | ☐ | ☐ | ☐ | ____________________ |
| Verify all personnel working near mobile equipment or in traffic areas are wearing high-visibility clothing (vests, jackets) that meet ANSI 107 standards. | ☐ | ☐ | ☐ | ____________________ |
| If respiratory hazards are present (dust, fumes), ensure a respiratory protection program is in place, including fit testing, medical evaluations, and proper respirator selection. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all PPE for damage or defects before each use. Replace damaged PPE immediately. Maintain PPE in good working condition. | ☐ | ☐ | ☐ | ____________________ |
| Verify all personnel have received training on the proper use, care, and maintenance of required PPE. | ☐ | ☐ | ☐ | ____________________ |
| Inspect fall protection harnesses daily for rips, tears, fraying, or other damage before each use. | ☐ | ☐ | ☐ | ____________________ |

Documentation & Communication

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the approved demolition plan, including engineering calculations and shoring design, is readily available on site. | ☐ | ☐ | ☐ | ____________________ |
| Confirm daily engineering survey reports are completed and readily available for review. Reports must document shoring system condition and any observed deviations. | ☐ | ☐ | ☐ | ____________________ |
| Verify a pre-demolition survey was conducted to identify potential hazards (asbestos, lead paint, utilities) and documented. | ☐ | ☐ | ☐ | ____________________ |
| Confirm a competent person has been designated and is present on site to oversee demolition activities and ensure compliance with safety regulations. | ☐ | ☐ | ☐ | ____________________ |
| Ensure emergency contact information (fire, police, ambulance, hospital) is readily available and posted in a conspicuous location. | ☐ | ☐ | ☐ | ____________________ |
| Conduct daily toolbox talks to discuss specific hazards and safety procedures for the day's activities. Document attendance and topics covered. | ☐ | ☐ | ☐ | ____________________ |
| Maintain a log of all personnel entering and exiting the demolition site. Restrict access to authorized personnel only. | ☐ | ☐ | ☐ | ____________________ |
| Ensure a hazard communication program is in place, including SDSs for all hazardous materials used on site. Train employees on hazard recognition and safe handling procedures. | ☐ | ☐ | ☐ | ____________________ |
| Maintain a detailed log of all shoring inspections, including dates, times, findings, and corrective actions taken. | ☐ | ☐ | ☐ | ____________________ |
| Ensure incident reporting procedures are clearly defined and understood by all personnel. Investigate all incidents and implement corrective actions to prevent recurrence. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________