InspectionReport

Elevator Shaft Installation & Fall Protection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Hoistway Barricade Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are hoistway openings barricaded with a minimum 42-inch high guardrail system, including mid-rail and toe board? | ☐ | ☐ | ☐ | ____________________ |
| Are barricade materials (wood, metal, etc.) structurally sound and capable of withstanding a 200-pound force in any direction? | ☐ | ☐ | ☐ | ____________________ |
| Are barricades securely attached to the hoistway structure to prevent accidental displacement? | ☐ | ☐ | ☐ | ____________________ |
| Are warning signs (e.g., 'Danger - Open Hoistway') clearly posted at each barricaded opening? | ☐ | ☐ | ☐ | ____________________ |
| Do barricades allow for necessary access while preventing accidental falls into the hoistway? | ☐ | ☐ | ☐ | ____________________ |
| Are 3.5-inch toe boards installed on all open sides of the hoistway to prevent tools and materials from falling? | ☐ | ☐ | ☐ | ____________________ |
| Is a daily inspection of all hoistway barricades conducted and documented? | ☐ | ☐ | ☐ | ____________________ |
| Are all barricades a minimum of 42 inches in height, with a midrail positioned halfway between the top rail and the walking/working surface? | ☐ | ☐ | ☐ | ____________________ |
| Are barricades free from damage, deterioration, or any condition that could compromise their structural integrity? | ☐ | ☐ | ☐ | ____________________ |
| Is adequate lighting (minimum 10 foot-candles) provided at all hoistway entrances and working areas? | ☐ | ☐ | ☐ | ____________________ |

Fall Protection Systems (Personal)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are personal fall arrest systems (PFAS) inspected daily for damage, wear, and proper fit by a competent person? | ☐ | ☐ | ☐ | ____________________ |
| Are anchor points for PFAS capable of supporting at least 5,000 pounds per employee attached? | ☐ | ☐ | ☐ | ____________________ |
| Are tie-off points located above the worker's D-ring to minimize free-fall distance? | ☐ | ☐ | ☐ | ____________________ |
| Are lanyards free from cuts, abrasions, chemical damage, and other signs of deterioration? | ☐ | ☐ | ☐ | ____________________ |
| Do self-retracting lifelines (SRLs) function properly, with smooth extension and retraction? | ☐ | ☐ | ☐ | ____________________ |
| Is there adequate fall clearance below the worker to prevent contact with lower levels or obstructions? | ☐ | ☐ | ☐ | ____________________ |
| Is a rescue plan in place and readily available in case of a fall? | ☐ | ☐ | ☐ | ____________________ |
| Is the full body harness compatible with the lanyard and anchorage connector being used? | ☐ | ☐ | ☐ | ____________________ |
| Have all employees working at height received documented fall protection training? | ☐ | ☐ | ☐ | ____________________ |
| Are car top railings in place and properly secured when performing car top inspections? | ☐ | ☐ | ☐ | ____________________ |

Overhead Protection & Housekeeping

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is overhead protection (e.g., netting, planking) in place to prevent tools and materials from falling into the hoistway? | ☐ | ☐ | ☐ | ____________________ |
| Is the hoistway free from accumulated debris, tools, and materials that could create a hazard? | ☐ | ☐ | ☐ | ____________________ |
| Are tools used at height tethered to prevent them from falling? | ☐ | ☐ | ☐ | ____________________ |
| Are materials stored at least 6 feet away from the hoistway edge? | ☐ | ☐ | ☐ | ____________________ |
| Are warning signs posted below the hoistway indicating potential falling object hazards? | ☐ | ☐ | ☐ | ____________________ |
| If a false car is in use, is it inspected daily for structural integrity and proper operation? | ☐ | ☐ | ☐ | ____________________ |
| Is adequate lighting provided below the hoistway to identify potential falling objects? | ☐ | ☐ | ☐ | ____________________ |
| Is a reliable communication system (e.g., radios) in place between workers in the hoistway and those on other levels? | ☐ | ☐ | ☐ | ____________________ |
| Is an emergency stop device readily accessible within the hoistway? | ☐ | ☐ | ☐ | ____________________ |
| Is a fire extinguisher (minimum 2A:10BC rating) readily accessible near the hoistway? | ☐ | ☐ | ☐ | ____________________ |

Electrical Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all electrical tools and equipment used in or near the hoistway GFCI protected? | ☐ | ☐ | ☐ | ____________________ |
| Are extension cords free from damage, splices, and improper use? | ☐ | ☐ | ☐ | ____________________ |
| Are electrical panels and disconnects readily accessible and clearly labeled? | ☐ | ☐ | ☐ | ____________________ |
| Is a lockout/tagout procedure in place and followed for all electrical work? | ☐ | ☐ | ☐ | ____________________ |
| Are all electrical tools and equipment properly grounded? | ☐ | ☐ | ☐ | ____________________ |
| Are overhead power lines identified and are safe clearance distances maintained? | ☐ | ☐ | ☐ | ____________________ |
| Is temporary wiring installed according to code requirements? | ☐ | ☐ | ☐ | ____________________ |
| Is all electrical work performed by a qualified electrician? | ☐ | ☐ | ☐ | ____________________ |
| Are electrical enclosures weatherproof when exposed to the elements? | ☐ | ☐ | ☐ | ____________________ |
| Has an arc flash hazard assessment been conducted, and are appropriate PPE requirements being followed? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________