InspectionReport

Temporary Bleacher & Grandstand Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Structural Integrity

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify that the foundation or ground supporting the grandstand is level, stable, and capable of bearing the anticipated load. Check for signs of settlement or erosion. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all frame connections (bolts, welds, pins) for tightness, corrosion, and damage. Ensure all locking mechanisms are properly engaged. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all required support bracing (cross-bracing, diagonal bracing) is in place and properly secured. Check for bent, damaged, or missing braces. | ☐ | ☐ | ☐ | ____________________ |
| Examine all planking (seating and walkways) for cracks, splits, rot, or other damage. Replace any damaged planks immediately. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all planking is securely fastened to the frame to prevent movement or displacement. Check for proper spacing between planks. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that the grandstand's load capacity is clearly posted and not exceeded. Ensure event staff are aware of the load limits. | ☐ | ☐ | ☐ | ____________________ |
| Measure aisle widths to ensure they meet minimum code requirements for safe egress. Clear aisles of any obstructions. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all egress pathways are clear, well-lit, and free from obstructions. Ensure adequate signage is in place to guide occupants to exits. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all metal components (bolts, connectors, etc.) for signs of corrosion. Replace corroded hardware immediately. | ☐ | ☐ | ☐ | ____________________ |
| Monitor for any signs of settling or shifting of the structure during and after setup. Re-level and stabilize as needed. | ☐ | ☐ | ☐ | ____________________ |

Guardrails and Fall Protection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Measure guardrail height to ensure it meets minimum code requirements (typically 42 inches). | ☐ | ☐ | ☐ | ____________________ |
| Verify that guardrails are capable of withstanding the required load (typically 200 pounds). Test for stability and rigidity. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that midrails are installed between the top rail and the walking surface to prevent falls. Spacing should not exceed 19 inches. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that toe boards (minimum 4 inches high) are installed where there is a risk of objects falling onto people below. | ☐ | ☐ | ☐ | ____________________ |
| Inspect stair railings for proper height, strength, and secure attachment. Ensure handrails are continuous and graspable. | ☐ | ☐ | ☐ | ____________________ |
| Measure gaps in guardrails and stair railings to ensure they do not exceed code limits (typically 4 inches). | ☐ | ☐ | ☐ | ____________________ |
| If required, verify that fall arrest systems (harnesses, lanyards, anchor points) are in good condition and properly inspected. Ensure users are trained in their use. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that perimeter barriers are in place to prevent unauthorized access to the grandstand structure. | ☐ | ☐ | ☐ | ____________________ |
| Ensure handrails extend beyond the top and bottom risers of stairways as required by code. | ☐ | ☐ | ☐ | ____________________ |
| Verify the guardrail design prevents easy climbing by children (e.g., no horizontal rails spaced closely together). | ☐ | ☐ | ☐ | ____________________ |

Egress and Emergency Preparedness

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify that exit signs are clearly visible, properly illuminated, and indicate the direction of egress. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that emergency lighting is functional and provides adequate illumination in the event of a power outage. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that fire extinguishers are readily accessible, properly charged, and inspected. Verify that personnel are trained in their use. | ☐ | ☐ | ☐ | ____________________ |
| Review the emergency evacuation plan with event staff. Ensure everyone knows the evacuation routes and procedures. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that first aid supplies and trained personnel are available on-site. | ☐ | ☐ | ☐ | ____________________ |
| Verify that communication systems (radios, public address system) are functional and can be used to communicate emergency information. | ☐ | ☐ | ☐ | ____________________ |
| Assess crowd management procedures to ensure orderly entry, seating, and egress. Identify potential bottlenecks and develop strategies to mitigate them. | ☐ | ☐ | ☐ | ____________________ |
| Verify that accessible routes are provided for individuals with disabilities, in accordance with ADA requirements. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all exit doors are equipped with panic hardware that allows for easy egress in an emergency. | ☐ | ☐ | ☐ | ____________________ |
| Identify and clearly mark designated muster points for evacuation. | ☐ | ☐ | ☐ | ____________________ |

Daily Hardware Checks

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Visually inspect and spot-check the tightness of critical bolts and fasteners, especially in high-stress areas. | ☐ | ☐ | ☐ | ____________________ |
| Verify that plank supports are securely in place and show no signs of bending or cracking. | ☐ | ☐ | ☐ | ____________________ |
| Inspect guardrail connections for looseness or damage. Ensure all locking pins or mechanisms are engaged. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all access points (stairs, ramps) are clear of debris and in good repair. | ☐ | ☐ | ☐ | ____________________ |
| Assess the impact of weather conditions (wind, rain) on the stability of the grandstand. Take appropriate action if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Re-inspect the ground conditions around the base of the structure for any signs of erosion or settling. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all safety signage (load limits, exit signs) are clearly visible and legible. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all lighting systems are functioning properly, including emergency lighting. | ☐ | ☐ | ☐ | ____________________ |
| Inspect for any signs of vandalism or tampering with the structure. | ☐ | ☐ | ☐ | ____________________ |
| Maintain an inventory of spare hardware (bolts, pins, etc.) for quick repairs. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________