InspectionReport

Excavator Daily Pre-Start Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

Jurisdiction: Adaptable. Covers OSHA 29 CFR 1926.602 (US) and UK PUWER 1998. Adaptable to other jurisdictions with heavy equipment regulations. Apply only the section and thresholds for the governing jurisdiction; do not combine US and UK requirements as if they were one code.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Inspection header

| Field | Entry |
| --- | --- |
| Excavator make and model | ________________________________________________ |
| Machine ID / serial number | ________________________________________________ |
| Operator name | ________________________________________________ |
| Date and shift | ________________________________________________ |

360-degree walkaround (structure and undercarriage)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Undercarriage and tracks: track tension correct; shoe pads intact; final drive no leaks | ☐ | ☐ | ☐ | ____________________ |
| Boom, stick, and bucket: no hairline structural cracks at weld points | ☐ | ☐ | ☐ | ____________________ |
| Pins and bushings: retaining bolts secure; no excessive play; grease present | ☐ | ☐ | ☐ | ____________________ |
| Teeth and cutting edges: sharp, retention pins intact | ☐ | ☐ | ☐ | ____________________ |

Hydraulics, engine, and fluid systems

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Fluid levels: engine oil, engine coolant, hydraulic fluid (bucket flat, stick vertical per manufacturer) | ☐ | ☐ | ☐ | ____________________ |
| Hydraulic lines and cylinders: no chafing, bubbling, wet spots; chrome cylinders no scoring or leaking seals | ☐ | ☐ | ☐ | ____________________ |
| Engine compartment: fan belt no cracking; air filter dust valve clear; radiator of packed mud and debris | ☐ | ☐ | ☐ | ____________________ |

Critical wear points

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Hydraulic hoses: outer rubber jacket intact; no exposed steel wire braiding or weeping fluid | ☐ | ☐ | ☐ | ____________________ |
| Bucket teeth: sharp, locking rings present, snug fit | ☐ | ☐ | ☐ | ____________________ |
| Track tension: sag between carrier roller and idler within spec; no clanking during tramming | ☐ | ☐ | ☐ | ____________________ |
| Swing bearing: grease distributed; turntable bolts intact; no grinding during rotation | ☐ | ☐ | ☐ | ____________________ |

Operator cab and in-cab safety devices

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Backup alarms and horns: bidirectional, distinguishable from ambient noise; backup alarm automatic in reverse | ☐ | ☐ | ☐ | ____________________ |
| Visibility aids: cab glass clean; mirrors intact and angled; rear-view / 360-degree camera boots without error codes | ☐ | ☐ | ☐ | ____________________ |
| Fire extinguisher: charged, in-date, securely mounted in cab | ☐ | ☐ | ☐ | ____________________ |
| Seat belts: mandatory with ROPS; latches securely | ☐ | ☐ | ☐ | ____________________ |

Regulatory checks (PUWER / LOLER)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Hydraulic deadman / safety lock: left-hand console raised disables all hydraulic joystick inputs | ☐ | ☐ | ☐ | ____________________ |
| If used for lifting (LOLER): hose burst check valves (HBCVs) on boom and dipper cylinders; safe load indicator present; statutory Thorough Examination within 12 months | ☐ | ☐ | ☐ | ____________________ |

Defect action

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Defect description | ☐ | ☐ | ☐ | ____________________ |
| Machine taken out of service: Yes / No | ☐ | ☐ | ☐ | ____________________ |
| Key removed | ☐ | ☐ | ☐ | ____________________ |
| 'Do Not Operate' tag attached | ☐ | ☐ | ☐ | ____________________ |
| Site supervisor notified | ☐ | ☐ | ☐ | ____________________ |
| Operator signature | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________

References consulted

OSHA 29 CFR 1926.602 – Material handling equipment

UK HSE – PUWER 1998 (Provision and Use of Work Equipment Regulations)

UK HSE – LOLER 1998 (Lifting Operations and Lifting Equipment Regulations)

Verify the current edition and local adoption before use.