InspectionReport

Fall Protection General Requirements Daily Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

Jurisdiction: US. OSHA 29 CFR 1926.501 and 1926.502 are US construction standards. Trigger heights and requirements vary by industry.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Inspection header

| Field | Entry |
| --- | --- |
| Site name and location | ________________________________________________ |
| Inspector / worker name | ________________________________________________ |
| Date and shift | ________________________________________________ |
| Work area / height | ________________________________________________ |

Trigger height verification

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Industry / activity identified (general industry: 4 ft, shipyards: 5 ft, construction: 6 ft, scaffolding: 10 ft, steel erection: 15 ft) | ☐ | ☐ | ☐ | ____________________ |
| Fall protection required at this height: Yes / No | ☐ | ☐ | ☐ | ____________________ |

5-Point daily harness inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Webbing: no cuts, fraying, pulled stitches, or chemical damage; bend in inverted 'U' to expose hidden defects | ☐ | ☐ | ☐ | ____________________ |
| D-rings: no distortion, cracks, breaks, rough or sharp edges; D-ring pivots freely | ☐ | ☐ | ☐ | ____________________ |
| Buckles: engage positively; no bent tongues or distorted frames | ☐ | ☐ | ☐ | ____________________ |
| Stitching: load-bearing indicators intact; impact indicator not popped or deployed | ☐ | ☐ | ☐ | ____________________ |
| Labels: manufacturer labels present and legible | ☐ | ☐ | ☐ | ____________________ |

Anchorage point evaluation

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Structural integrity: tied to main building structure (not guardrails, vents, or light fixtures) | ☐ | ☐ | ☐ | ____________________ |
| Capacity: holds 5,000 lbs static load or maintains safety factor of two | ☐ | ☐ | ☐ | ____________________ |
| Positioning: directly above work area (minimize swing fall hazard) | ☐ | ☐ | ☐ | ____________________ |
| Connectors: self-closing and self-locking carabiners / snap hooks | ☐ | ☐ | ☐ | ____________________ |

Defective equipment handling

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Defective equipment removed from service | ☐ | ☐ | ☐ | ____________________ |
| Tagged out | ☐ | ☐ | ☐ | ____________________ |
| Handed to supervisor | ☐ | ☐ | ☐ | ____________________ |
| Inspector signature | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________

References consulted

OSHA 29 CFR 1926.501 – Duty to have fall protection

OSHA 29 CFR 1926.502 – System planning and criteria

OSHA 29 CFR 1926.503 – Training requirements

OSHA – Fall protection eTool

Verify the current edition and local adoption before use.