InspectionReport

Forklift Battery Charging & Change-out Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Battery Charging Area - Site Conditions

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the battery charging area equipped with adequate ventilation to prevent hydrogen gas accumulation (e.g., mechanical exhaust system)? | ☐ | ☐ | ☐ | ____________________ |
| Is a hydrogen gas detector present and calibrated to alarm at or below 1% (25% LEL) hydrogen concentration? | ☐ | ☐ | ☐ | ____________________ |
| Is an eyewash station located within 10 seconds (or 100 feet) of the battery charging area, on the same level, with unobstructed access? | ☐ | ☐ | ☐ | ____________________ |
| Is the eyewash station flushing freely with potable water for a minimum of 15 minutes, and is the water tepid (60-100°F)? | ☐ | ☐ | ☐ | ____________________ |
| Is an acid spill kit readily available and fully stocked with neutralizing agents, PPE, and disposal bags? | ☐ | ☐ | ☐ | ____________________ |
| Are 'No Smoking' signs clearly posted within 50 feet of the battery charging area? | ☐ | ☐ | ☐ | ____________________ |
| Is a Class C fire extinguisher (suitable for electrical fires) readily accessible and fully charged? | ☐ | ☐ | ☐ | ____________________ |
| Is the floor in the battery charging area acid-resistant and free from cracks or damage that could trap spilled acid? | ☐ | ☐ | ☐ | ____________________ |
| Is there adequate lighting (minimum 50 foot-candles) in the battery charging area to safely perform tasks? | ☐ | ☐ | ☐ | ____________________ |
| Are spare batteries stored on designated racks or pallets, preventing direct contact with the floor? | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are acid-resistant aprons available and in good condition for all personnel handling batteries? | ☐ | ☐ | ☐ | ____________________ |
| Are acid-resistant gloves (nitrile or neoprene) available and in good condition for all personnel handling batteries? | ☐ | ☐ | ☐ | ____________________ |
| Are full-face shields or safety glasses with side shields available and in good condition for all personnel handling batteries? | ☐ | ☐ | ☐ | ____________________ |
| Is PPE inspected before each use for tears, punctures, or other damage? | ☐ | ☐ | ☐ | ____________________ |
| Have all personnel been trained on the proper use, maintenance, and disposal of PPE? | ☐ | ☐ | ☐ | ____________________ |
| Are safety shoes with steel toe protection worn by all personnel handling batteries? | ☐ | ☐ | ☐ | ____________________ |
| Is there a written eye protection policy enforced in the battery charging area? | ☐ | ☐ | ☐ | ____________________ |
| Is there a designated area for storing PPE when not in use, away from potential contamination? | ☐ | ☐ | ☐ | ____________________ |
| Are procedures in place for cleaning and decontaminating PPE after each use? | ☐ | ☐ | ☐ | ____________________ |

Battery Handling Equipment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the battery extractor (if used) in good working order, with all safety features functioning correctly? | ☐ | ☐ | ☐ | ____________________ |
| Is the battery extractor rated for the weight of the batteries being handled? | ☐ | ☐ | ☐ | ____________________ |
| Is the battery extractor inspected daily for damage, wear, or missing parts? | ☐ | ☐ | ☐ | ____________________ |
| If a hoist is used, is the chain or strap inspected for wear, damage, or deformation before each use? | ☐ | ☐ | ☐ | ____________________ |
| If a hoist is used, is it rated for the weight of the batteries being handled, with a clear load rating visible? | ☐ | ☐ | ☐ | ____________________ |
| If a pallet jack is used, are the wheels and lifting mechanism in good working order? | ☐ | ☐ | ☐ | ____________________ |
| Are lifting slings (if used) inspected for cuts, abrasions, or chemical damage before each use? | ☐ | ☐ | ☐ | ____________________ |
| Have all personnel been trained on the safe operation of the battery handling equipment? | ☐ | ☐ | ☐ | ____________________ |
| Is there a documented maintenance schedule for the battery handling equipment? | ☐ | ☐ | ☐ | ____________________ |

Battery Charging Procedures

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the battery charger voltage compatible with the battery being charged? | ☐ | ☐ | ☐ | ____________________ |
| Is the battery charger free from damage, with all cables and connectors in good condition? | ☐ | ☐ | ☐ | ____________________ |
| Are all battery cables and connectors properly insulated and free from exposed wires? | ☐ | ☐ | ☐ | ____________________ |
| Are all battery vent caps in place and functioning correctly? | ☐ | ☐ | ☐ | ____________________ |
| Is the area around the battery charger clear of obstructions and flammable materials? | ☐ | ☐ | ☐ | ____________________ |
| Are battery connections clean and tight, preventing arcing or sparking? | ☐ | ☐ | ☐ | ____________________ |
| Is the battery charging process monitored to prevent overcharging or overheating? | ☐ | ☐ | ☐ | ____________________ |
| Is the electrolyte level in the battery checked regularly and maintained at the proper level with distilled water? | ☐ | ☐ | ☐ | ____________________ |
| Is the battery temperature monitored during charging to prevent overheating? | ☐ | ☐ | ☐ | ____________________ |
| Are the charger's operating instructions readily available and followed? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________