InspectionReport

Commercial Gym & Fitness Center Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Cardio Equipment Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Treadmill belt tracking is centered and stable at speeds of 2 mph, 5 mph, and 8 mph. Belt slippage does not exceed 1 inch under load. | ☐ | ☐ | ☐ | ____________________ |
| Treadmill emergency stop lanyard is present, undamaged, and immediately stops the belt within 2 seconds when pulled. | ☐ | ☐ | ☐ | ____________________ |
| Elliptical resistance levels adjust smoothly and accurately across the full range of settings. No unusual noises or vibrations are present. | ☐ | ☐ | ☐ | ____________________ |
| Elliptical foot pedals are securely attached and move freely without excessive play or binding. Check for loose bolts. | ☐ | ☐ | ☐ | ____________________ |
| Stationary bike seat adjustment mechanism functions correctly, locking securely at all height settings. Verify quick-release lever integrity. | ☐ | ☐ | ☐ | ____________________ |
| Stationary bike pedal straps are present, undamaged, and securely fasten to the pedals. Replace if frayed or cracked. | ☐ | ☐ | ☐ | ____________________ |
| All cardio equipment power cords are free from cuts, abrasions, and frays. Grounding prongs are intact and secure. (NFPA 70) | ☐ | ☐ | ☐ | ____________________ |
| All cardio equipment is stable and level. Rubber feet are present and in good condition. Equipment does not rock or move excessively during use. | ☐ | ☐ | ☐ | ____________________ |
| Cardio equipment displays are functioning correctly and providing accurate feedback (speed, distance, heart rate). | ☐ | ☐ | ☐ | ____________________ |
| Cardio equipment is clean and free from excessive sweat or debris. Ensure proper sanitation protocols are followed. | ☐ | ☐ | ☐ | ____________________ |

Strength Training Equipment Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Cable pulleys on pin-loaded machines are free from cracks, chips, and excessive wear. Cables move smoothly without binding or fraying. | ☐ | ☐ | ☐ | ____________________ |
| Cables on pin-loaded machines are properly tensioned and show no signs of fraying, kinking, or corrosion. Replace cables annually or more frequently if needed. | ☐ | ☐ | ☐ | ____________________ |
| Weight stack selector pins on pin-loaded machines are present, undamaged, and securely lock into place. Verify proper weight selection. | ☐ | ☐ | ☐ | ____________________ |
| Frames of all strength training machines are free from cracks, bends, and corrosion. Welds are intact and show no signs of failure. | ☐ | ☐ | ☐ | ____________________ |
| Adjustable benches are stable and do not wobble. Adjustment mechanisms lock securely in all positions. | ☐ | ☐ | ☐ | ____________________ |
| Dumbbell weights are securely attached to the handles and do not spin freely. Handles are not excessively worn or slippery. | ☐ | ☐ | ☐ | ____________________ |
| Barbell bars are straight and free from excessive rust or pitting. Knurling provides adequate grip. | ☐ | ☐ | ☐ | ____________________ |
| Weight racks are stable and securely anchored to the floor or wall. Weight storage is organized and prevents weights from falling. | ☐ | ☐ | ☐ | ____________________ |
| All strength training machines have legible instruction labels showing proper usage and safety precautions. | ☐ | ☐ | ☐ | ____________________ |
| Ensure proper range of motion on all machines. Check for any obstructions or limitations. | ☐ | ☐ | ☐ | ____________________ |

Facility and Environmental Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Floors are clean, dry, and free from slip hazards such as spilled liquids, loose mats, or damaged flooring. (OSHA 1910.22) | ☐ | ☐ | ☐ | ____________________ |
| Mirrors are clean, undamaged, and securely mounted to the wall. Replace any cracked or broken mirrors immediately. | ☐ | ☐ | ☐ | ____________________ |
| Lighting is adequate throughout the facility, providing sufficient illumination for safe equipment operation and movement. (ANSI/IES RP-7) | ☐ | ☐ | ☐ | ____________________ |
| Ventilation system is functioning properly, providing adequate air circulation and preventing the buildup of odors and moisture. | ☐ | ☐ | ☐ | ____________________ |
| Emergency exits are clearly marked, unobstructed, and easily accessible. Ensure exit doors swing freely and are not blocked. (NFPA 101) | ☐ | ☐ | ☐ | ____________________ |
| Fire extinguishers are readily accessible, fully charged, and inspected monthly. Staff are trained in their proper use. (NFPA 10) | ☐ | ☐ | ☐ | ____________________ |
| First-aid kit is fully stocked with necessary supplies, including bandages, antiseptic wipes, pain relievers, and gloves. Check expiration dates. | ☐ | ☐ | ☐ | ____________________ |
| Drinking water is readily available and easily accessible to members and staff. Water fountains are clean and functioning properly. | ☐ | ☐ | ☐ | ____________________ |
| Appropriate safety signage is posted throughout the facility, including warnings about equipment usage, emergency procedures, and hygiene practices. | ☐ | ☐ | ☐ | ____________________ |
| Noise levels are within acceptable limits. Excessive noise can create a hazard. Consider implementing noise reduction strategies if necessary. | ☐ | ☐ | ☐ | ____________________ |

Free Weight Area Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Dumbbells are stored properly on racks, preventing them from rolling or falling. Racks are not overloaded. | ☐ | ☐ | ☐ | ____________________ |
| Barbells are stored securely on racks. Weight clips are available and used to prevent weights from sliding off the bar. | ☐ | ☐ | ☐ | ____________________ |
| Weight plates are free from cracks, chips, and sharp edges. Rubber coating (if present) is intact. | ☐ | ☐ | ☐ | ____________________ |
| Lifting platforms are level and free from cracks or splinters. Rubber mats are securely in place and provide adequate cushioning. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that members are aware of the importance of spotting when lifting heavy weights. Encourage the use of spotters. | ☐ | ☐ | ☐ | ____________________ |
| Barbell collars are in good working order and securely fasten to the barbell, preventing weights from sliding. | ☐ | ☐ | ☐ | ____________________ |
| Benches in the free weight area are stable and do not wobble. Upholstery is free from tears or rips. | ☐ | ☐ | ☐ | ____________________ |
| Mirrors are positioned to allow members to monitor their form without creating a safety hazard (e.g., glare, obstruction). | ☐ | ☐ | ☐ | ____________________ |
| Floor matting is in place to protect the floor and reduce noise. Mats are clean and free from tears or tripping hazards. | ☐ | ☐ | ☐ | ____________________ |
| The free weight area is clean and free from debris. Weights are returned to their proper storage locations after use. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________