InspectionReport

Excavator Daily Pre-Shift Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Mobile Equipment: Undercarriage & Tracks

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Measure track sag. Ensure it's within the manufacturer's specified range (typically 2-4 inches). Record measurement. | ☐ | ☐ | ☐ | ____________________ |
| Inspect track rollers for excessive wear, damage, or leakage. Check for smooth rotation; seized rollers indicate a problem. | ☐ | ☐ | ☐ | ____________________ |
| Verify track pads are securely fastened and not excessively worn or damaged. Replace any missing or severely damaged pads. | ☐ | ☐ | ☐ | ____________________ |
| Examine the drive sprocket teeth for wear or damage. Ensure proper engagement with the track links. | ☐ | ☐ | ☐ | ____________________ |
| Inspect idlers for proper alignment, wear, and lubrication. Check for excessive play or damage. | ☐ | ☐ | ☐ | ____________________ |
| Check final drive oil level. Inspect for leaks around the final drive housing. | ☐ | ☐ | ☐ | ____________________ |
| Listen for unusual noises during swing operation. Check for excessive play or looseness in the swing bearing. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect the excavator frame for any cracks, bends, or signs of structural damage, especially around weld points. | ☐ | ☐ | ☐ | ____________________ |
| Inspect hydraulic lines and fittings in the undercarriage for leaks. Repair any leaks immediately. | ☐ | ☐ | ☐ | ____________________ |
| Test the travel alarm to ensure it is functioning correctly and audible. | ☐ | ☐ | ☐ | ____________________ |

Operator Cab & Controls

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect the ROPS (Rollover Protective Structure) for damage or modifications. Ensure it is securely mounted. | ☐ | ☐ | ☐ | ____________________ |
| Verify the seatbelt is in good working condition and properly latches. Replace if damaged. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all windows are clean and free from cracks or obstructions that impair visibility. Replace damaged glass. | ☐ | ☐ | ☐ | ____________________ |
| Inspect mirrors for damage and ensure they are properly adjusted for optimal visibility. | ☐ | ☐ | ☐ | ____________________ |
| Test all controls (boom, stick, swing, travel) for smooth and responsive operation. Report any sticking or unresponsive controls. | ☐ | ☐ | ☐ | ____________________ |
| Test the horn to ensure it is functioning correctly and audible. | ☐ | ☐ | ☐ | ____________________ |
| Verify all gauges and indicators (fuel, temperature, oil pressure) are functioning correctly and within normal operating ranges. | ☐ | ☐ | ☐ | ____________________ |
| Ensure a fully charged and readily accessible fire extinguisher is present in the cab. Verify it is the correct type (ABC) and has a current inspection tag. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the operator's manual is present and readily accessible in the cab. | ☐ | ☐ | ☐ | ____________________ |
| Ensure three points of contact are maintained when entering and exiting the cab. Check steps and handholds for damage or obstructions. | ☐ | ☐ | ☐ | ____________________ |

Hydraulic System & Boom

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect all hydraulic hoses, cylinders, and fittings on the boom and stick for leaks. Repair any leaks immediately. | ☐ | ☐ | ☐ | ____________________ |
| Examine hydraulic cylinders for dents, scratches, or other damage. Check for smooth cylinder operation. | ☐ | ☐ | ☐ | ____________________ |
| Inspect hydraulic hoses for wear, cracks, abrasions, or kinks. Replace any damaged hoses. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect the boom and stick for any cracks, bends, or signs of structural damage, especially around weld points. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the quick hitch/coupler for cracks, wear, and proper locking mechanism function. Ensure safety pins are in place and secured. | ☐ | ☐ | ☐ | ____________________ |
| Inspect bucket teeth for wear or damage. Replace worn or broken teeth. | ☐ | ☐ | ☐ | ____________________ |
| Verify bucket pins are properly secured and not excessively worn. Replace if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Check the hydraulic oil level and top off as needed. Note any unusual oil consumption. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the hydraulic filter for damage or contamination. Replace as needed according to the maintenance schedule. | ☐ | ☐ | ☐ | ____________________ |
| Test boom-mounted lights to ensure they are functioning correctly, especially if operating in low-light conditions. | ☐ | ☐ | ☐ | ____________________ |

Engine Compartment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Check the engine oil level and top off as needed. Note any unusual oil consumption. | ☐ | ☐ | ☐ | ____________________ |
| Check the coolant level and top off as needed. Inspect for leaks in the cooling system. | ☐ | ☐ | ☐ | ____________________ |
| Check the fuel level and fill as needed. Inspect for fuel leaks. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the battery terminals for corrosion and ensure they are securely connected. Check battery voltage if possible. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the air filter for cleanliness. Clean or replace as needed. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the engine compartment for any oil, fuel, or coolant leaks. Repair any leaks immediately. | ☐ | ☐ | ☐ | ____________________ |
| Inspect belts for wear, cracks, or fraying. Ensure proper tension. | ☐ | ☐ | ☐ | ____________________ |
| Inspect wiring harnesses for damage, loose connections, or exposed wires. Repair as needed. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the exhaust system for leaks or damage. Ensure the exhaust is properly directed away from the operator. | ☐ | ☐ | ☐ | ____________________ |
| Listen for any unusual noises from the engine during startup and operation. Investigate any abnormal sounds. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________