InspectionReport

Critical Heavy Lift Rigging Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Sling Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify each sling has a legible sling tag indicating its rated capacity, sling angle, and serial number. (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |
| Confirm sling material is compatible with the load and environmental conditions (e.g., temperature, chemicals). (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |
| Inspect slings for cuts, abrasions, and other damage that could compromise their strength. (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |
| Examine wire rope slings for kinks, crushing, bird caging, or other distortions. (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |
| Check synthetic slings for chemical damage, burns, or melting. (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |
| Verify sling end fittings (e.g., eyes, hooks) are properly attached and free from cracks or deformation. (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |
| Measure sling stretch/elongation. Remove from service if exceeding manufacturer's recommendations. (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |
| Ensure all sling hardware (shackles, hooks) are of the correct size and rated capacity for the sling. (ASME B30.10) | ☐ | ☐ | ☐ | ____________________ |
| Verify slings are stored properly, protected from damage, moisture, and extreme temperatures. (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |
| Confirm slings are inspected regularly (daily before use) and documented. (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |

Shackle Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify each shackle has a legible WLL (Working Load Limit) stamped on the body. (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the correct type of shackle pin is used (e.g., screw pin, bolt-type with nut and cotter pin). (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |
| Ensure the shackle pin is fully engaged and properly secured. (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |
| Inspect shackle body for deformation, cracks, or wear. (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |
| Verify the shackle pin is straight and not bent or damaged. (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |
| Check shackle threads for damage or corrosion. (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |
| Ensure the shackle is loaded correctly, avoiding side loading or three-point loading. (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |
| Verify shackles are properly sized for the mating hardware (e.g., sling eyes, lifting lugs). (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |
| Inspect shackles for excessive corrosion that could reduce their strength. (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |
| Confirm shackles are from a reputable manufacturer and meet recognized standards. (ASME B30.26) | ☐ | ☐ | ☐ | ____________________ |

Spreader Bar Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the spreader bar has a legible WLL (Working Load Limit) clearly marked. (ASME B30.20) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the spreader bar design is appropriate for the lift (e.g., adjustable, fixed length). (ASME B30.20) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the spreader bar for any signs of bending, buckling, or deformation. (ASME B30.20) | ☐ | ☐ | ☐ | ____________________ |
| Examine all welds on the spreader bar for cracks, porosity, or other defects. (AWS D1.1) | ☐ | ☐ | ☐ | ____________________ |
| Verify the end connections (e.g., lifting lugs, padeyes) are securely attached and free from damage. (ASME B30.20) | ☐ | ☐ | ☐ | ____________________ |
| Ensure all hardware used with the spreader bar (e.g., shackles, slings) are compatible and properly rated. (ASME B30.20) | ☐ | ☐ | ☐ | ____________________ |
| If adjustable, verify the adjustment mechanism is functioning correctly and securely locked in place. (ASME B30.20) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the load is evenly distributed across the spreader bar. (ASME B30.20) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the spreader bar for adequate corrosion protection (e.g., paint, galvanizing). (ASME B30.20) | ☐ | ☐ | ☐ | ____________________ |
| Verify the spreader bar has documentation including load charts and inspection records. (ASME B30.20) | ☐ | ☐ | ☐ | ____________________ |

Load & Lift Planning

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the load weight is accurately determined and documented. (OSHA 1926.753) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the center of gravity (CG) of the load has been accurately calculated and marked. (ASME B30.9) | ☐ | ☐ | ☐ | ____________________ |
| Ensure a detailed rigging plan is available and understood by all personnel involved in the lift. (OSHA 1926.753) | ☐ | ☐ | ☐ | ____________________ |
| Verify the lift radius is clear of obstructions and personnel. (OSHA 1926.753) | ☐ | ☐ | ☐ | ____________________ |
| Assess environmental conditions (wind speed, temperature) and adjust the lift plan accordingly. (ASME B30.5) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the crane is properly leveled and supported. (OSHA 1926.1402) | ☐ | ☐ | ☐ | ____________________ |
| Establish a clear communication protocol between the crane operator, rigger, and signal person. (OSHA 1926.753) | ☐ | ☐ | ☐ | ____________________ |
| Verify all personnel involved in the lift are qualified and trained for their respective roles. (OSHA 1926.753) | ☐ | ☐ | ☐ | ____________________ |
| Confirm emergency procedures are in place and understood by all personnel. (OSHA 1926.753) | ☐ | ☐ | ☐ | ____________________ |
| Assess the ground bearing capacity and ensure it is adequate to support the crane and load. (OSHA 1926.1402) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________