InspectionReport

Home Safety Checklist - Residential Hazard & Fire Safety Check

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Fire Alarm & Detection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Smoke alarms installed on every level of the home, outside each sleeping area, and inside each bedroom, as required by NFPA 71. | ☐ | ☐ | ☐ | ____________________ |
| Each smoke alarm is within its 10-year service life (check the manufacture date on the back). Replace any unit older than 10 years. | ☐ | ☐ | ☐ | ____________________ |
| Press the test button on every smoke alarm. The alarm should sound clearly. Replace any unit that does not activate. | ☐ | ☐ | ☐ | ____________________ |
| Carbon monoxide (CO) alarms installed on every level of the home, outside each sleeping area, as required by NFPA 1. | ☐ | ☐ | ☐ | ____________________ |
| Press the test button on every CO alarm. The alarm should sound. Replace any unit that does not activate or is past its service life. | ☐ | ☐ | ☐ | ____________________ |
| Battery-powered alarms have fresh batteries. Replace batteries at least twice a year and whenever the low-battery chirp sounds. | ☐ | ☐ | ☐ | ____________________ |

Electrical Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| The electrical service panel cover is in place, undamaged, and not obstructed. Keep at least 36 inches of clear working space in front of the panel. | ☐ | ☐ | ☐ | ____________________ |
| Every circuit breaker is labeled so you can identify what it controls. Unlabeled breakers make it hard to isolate a circuit safely. | ☐ | ☐ | ☐ | ____________________ |
| GFCI (ground-fault circuit interrupter) outlets are installed in kitchens, bathrooms, garages, basements, and outdoors. Test the GFCI button; the outlet should cut power. | ☐ | ☐ | ☐ | ____________________ |
| AFCI (arc-fault circuit interrupter) protection is present on bedroom and living-area circuits, as required by the National Electrical Code for most residential wiring. | ☐ | ☐ | ☐ | ____________________ |
| Extension cords and appliance cords are not cut, frayed, or overloaded. Do not use damaged cords. Replace any cord with exposed wiring. | ☐ | ☐ | ☐ | ____________________ |
| Portable space heaters are at least 3 feet away from curtains, bedding, furniture, and other combustibles. Heaters are turned off when leaving the room or going to sleep. | ☐ | ☐ | ☐ | ____________________ |

Gas & Water Heaters

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| No gas odor near the gas meter, furnace, water heater, or gas appliances. If you smell gas, leave the home and call the gas company or 911 from outside. | ☐ | ☐ | ☐ | ____________________ |
| The gas shutoff valve is accessible, labeled, and in the ON position when the appliance is in use. Know where it is before you need it. | ☐ | ☐ | ☐ | ____________________ |
| The water heater temperature and pressure (T&P) relief valve is not capped, blocked, or painted over. The discharge pipe runs to a safe location (floor drain or within 6 feet of a floor). | ☐ | ☐ | ☐ | ____________________ |
| The water heater is within its expected service life (typically 8-12 years for a tank unit). Older units are more likely to leak or fail. | ☐ | ☐ | ☐ | ____________________ |

Water, Drainage & Mold

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| No active leaks at faucets, shower valves, toilet bases, water heater connections, or under sinks. Fix drips promptly to prevent water damage and mold. | ☐ | ☐ | ☐ | ____________________ |
| Ground around the foundation slopes away from the house so rainwater does not collect against the walls. Check for standing water after heavy rain. | ☐ | ☐ | ☐ | ____________________ |
| No visible mold, water stains, or musty odor in bathrooms, kitchens, basements, or around windows. Persistent moisture is a mold risk. | ☐ | ☐ | ☐ | ____________________ |
| Crawlspace or basement is dry and ventilated. Check for condensation on pipes, joists, or foundation walls, which indicates a moisture problem. | ☐ | ☐ | ☐ | ____________________ |

Falls & Physical Hazards

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Stair handrails are secure and continuous on at least one side of every flight. Check that rails do not wobble when you apply pressure. | ☐ | ☐ | ☐ | ____________________ |
| Carpet on stairs is not curled, loose, or worn through. Loose stair carpet is a trip and fall hazard, especially for children and older adults. | ☐ | ☐ | ☐ | ____________________ |
| Bathtubs and shower floors have non-slip surfaces or mats. Check that mats are clean, not mildewed, and do not shift when wet. | ☐ | ☐ | ☐ | ____________________ |
| Walking surfaces in hallways, kitchens, and entryways are clear of clutter, cords, and loose rugs. Rugs that must stay in place should have a non-slip pad. | ☐ | ☐ | ☐ | ____________________ |
| Lighting is adequate in hallways, stairways, kitchens, and bathrooms. Replace burnt-out bulbs. Consider motion-sensor lights for nighttime paths. | ☐ | ☐ | ☐ | ____________________ |

Fire Escape & Preparedness

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Every bedroom has at least two ways to exit (a door and a window that opens). Windows are not painted shut or blocked by furniture. | ☐ | ☐ | ☐ | ____________________ |
| A written or posted fire escape plan exists for the household. Every occupant knows the two exit routes from each room and the meeting point outside. | ☐ | ☐ | ☐ | ____________________ |
| A rated fire extinguisher (at least 2A:10B:C) is available in the kitchen or near the stove. The pin is in place, the gauge is in the green, and the unit is within its service life. | ☐ | ☐ | ☐ | ____________________ |
| Smoke alarms are tested monthly (press the test button) and batteries are replaced at least twice a year. This is a household habit, not a one-time task. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________