InspectionReport

Hydro-Excavation & Vacuum Truck Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Mobile Equipment & Vacuum Unit Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect all tires for proper inflation, cuts, bulges, and adequate tread depth (minimum 4/32 inch). | ☐ | ☐ | ☐ | ____________________ |
| Air brake system reaches 120 psi within 2 minutes at 1200 RPM; low-pressure buzzer sounds correctly @ 60 psi. Inspect air lines for leaks. | ☐ | ☐ | ☐ | ____________________ |
| Verify all lights (headlights, taillights, brake lights, turn signals, hazard lights) are functioning correctly. Check audible backup alarm. | ☐ | ☐ | ☐ | ____________________ |
| Inspect vacuum unit relief valves for proper operation and ensure they are free from obstructions. Test functionality daily. | ☐ | ☐ | ☐ | ____________________ |
| Verify static grounding cables are securely connected to the vacuum unit and the grounding rod. Check for corrosion or damage. | ☐ | ☐ | ☐ | ____________________ |
| Inspect vacuum hose and water wand hose for cuts, abrasions, kinks, and proper connections. Ensure hose clamps are secure. | ☐ | ☐ | ☐ | ____________________ |
| Check engine oil, coolant, hydraulic fluid, and other fluid levels. Replenish as needed. | ☐ | ☐ | ☐ | ____________________ |
| Verify the horn is functioning properly. | ☐ | ☐ | ☐ | ____________________ |
| Ensure windshield wipers are in good condition and functioning properly. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all seatbelts for proper function and condition. | ☐ | ☐ | ☐ | ____________________ |
| Confirm fire extinguisher is fully charged, inspected within the last year, and readily accessible. | ☐ | ☐ | ☐ | ____________________ |

Site Conditions & Utility Locating

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify all underground utilities have been located and clearly marked per 811 or local regulations. Confirm markings match site plans. | ☐ | ☐ | ☐ | ____________________ |
| Confirm a valid excavation permit has been obtained, if required by local regulations. | ☐ | ☐ | ☐ | ____________________ |
| Assess soil type and stability to determine appropriate excavation methods and shoring requirements, if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Ensure adequate traffic control measures are in place to protect workers and the public. Verify signage and barricades are properly positioned. | ☐ | ☐ | ☐ | ____________________ |
| Assess weather conditions (wind, rain, temperature) and adjust work practices accordingly. Suspend operations during severe weather. | ☐ | ☐ | ☐ | ____________________ |
| Evaluate the excavation area for potential confined space hazards. Implement confined space entry procedures if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the stability of excavation slopes. Ensure proper sloping or shoring is in place to prevent cave-ins. | ☐ | ☐ | ☐ | ____________________ |
| Ensure safe access and egress to the excavation area. Provide ladders, ramps, or other means of access as needed. | ☐ | ☐ | ☐ | ____________________ |
| Assess the proximity of nearby structures and utilities. Take precautions to prevent damage during excavation. | ☐ | ☐ | ☐ | ____________________ |
| Maintain a clean and organized work area. Remove debris and tripping hazards. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify hard hat is ANSI Z89.1 compliant, undamaged, and properly fitted. | ☐ | ☐ | ☐ | ____________________ |
| Ensure safety glasses or face shield are ANSI Z87.1 compliant and provide adequate eye protection. | ☐ | ☐ | ☐ | ____________________ |
| Confirm hearing protection (earplugs or earmuffs) is available and used when noise levels exceed 85 dBA. | ☐ | ☐ | ☐ | ____________________ |
| Verify high-visibility clothing (vest or jacket) is ANSI 107 compliant and worn at all times. | ☐ | ☐ | ☐ | ____________________ |
| Ensure appropriate gloves are worn to protect hands from cuts, abrasions, and chemical exposure. | ☐ | ☐ | ☐ | ____________________ |
| Verify steel-toe boots are worn to protect feet from impact and compression hazards. | ☐ | ☐ | ☐ | ____________________ |
| Assess the need for respiratory protection based on air quality. Provide and ensure proper use of respirators if required. | ☐ | ☐ | ☐ | ____________________ |
| Ensure waterproof clothing is available and worn during wet conditions to prevent hypothermia. | ☐ | ☐ | ☐ | ____________________ |
| Verify sunscreen and appropriate clothing are available for sun protection. | ☐ | ☐ | ☐ | ____________________ |

Hydro-Excavation Procedures

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify water pressure is within safe operating limits for the task and equipment. Do not exceed manufacturer's recommendations. | ☐ | ☐ | ☐ | ____________________ |
| Ensure proper water wand operating techniques are used to minimize risk of injury and damage to utilities. Maintain a safe distance from utilities. | ☐ | ☐ | ☐ | ____________________ |
| Position vacuum hose to prevent back-pressure and ensure efficient material removal. Avoid sharp bends or kinks in the hose. | ☐ | ☐ | ☐ | ____________________ |
| Establish clear communication signals between the water wand operator and the vacuum truck operator. | ☐ | ☐ | ☐ | ____________________ |
| Ensure a spotter is present to monitor the excavation area and provide warnings of potential hazards. | ☐ | ☐ | ☐ | ____________________ |
| Adhere to safe digging practices, including slow and controlled excavation, and probing to verify utility locations. | ☐ | ☐ | ☐ | ____________________ |
| Follow proper backfill procedures to ensure adequate compaction and prevent settlement. | ☐ | ☐ | ☐ | ____________________ |
| Dispose of excavated material in accordance with environmental regulations and company policies. | ☐ | ☐ | ☐ | ____________________ |
| Verify emergency shutdown procedures are understood and readily accessible. | ☐ | ☐ | ☐ | ____________________ |
| Identify and respect 'no spray' zones near sensitive equipment or areas. | ☐ | ☐ | ☐ | ____________________ |

Documentation & Training

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify vacuum truck operators are properly trained and certified to operate the equipment. | ☐ | ☐ | ☐ | ____________________ |
| Ensure utility locators have completed required training and are qualified to locate underground utilities. | ☐ | ☐ | ☐ | ____________________ |
| Confirm a site-specific safety plan has been developed and communicated to all workers. | ☐ | ☐ | ☐ | ____________________ |
| Verify Material Safety Data Sheets (MSDS) or Safety Data Sheets (SDS) are readily available for all chemicals used on site. | ☐ | ☐ | ☐ | ____________________ |
| Ensure an emergency contact list is readily available and includes contact information for local emergency services, utility companies, and company personnel. | ☐ | ☐ | ☐ | ____________________ |
| Review daily inspection logs for the vacuum truck and hydro-excavation equipment. Ensure any deficiencies have been addressed. | ☐ | ☐ | ☐ | ____________________ |
| Confirm a Job Hazard Analysis (JHA) or Task Hazard Analysis (THA) has been completed for the hydro-excavation task. | ☐ | ☐ | ☐ | ____________________ |
| Verify regular toolbox talks are conducted to discuss safety topics and reinforce safe work practices. | ☐ | ☐ | ☐ | ____________________ |
| If applicable, confirm a confined space entry permit is completed and posted. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________