InspectionReport

Industrial Trash Compactor & Baler Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Baler/Compactor Mechanical Integrity

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect the Ram for cracks, excessive wear, or deformation. Ensure it moves smoothly without binding. | ☐ | ☐ | ☐ | ____________________ |
| Examine hydraulic hoses and fittings for leaks, abrasions, or damage. Replace any compromised components immediately. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all door interlocks are functioning correctly. The Ram should not operate when any door is open. Test each interlock individually. | ☐ | ☐ | ☐ | ____________________ |
| Test the Emergency Stop (E-stop) button. Ensure it immediately halts all machine functions when activated. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all safety guards and shields are in place and securely fastened. No missing or damaged guards are permitted. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the baling or compaction chamber for structural damage, such as cracks or dents, that could compromise its integrity. | ☐ | ☐ | ☐ | ____________________ |
| For balers, check the wire feed mechanism for proper operation. Ensure wires are feeding smoothly and not tangling. | ☐ | ☐ | ☐ | ____________________ |
| Verify the pressure relief valve is functioning correctly to prevent over-pressurization of the hydraulic system. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the control panel for any damage, loose wiring, or malfunctioning buttons. Ensure all labels are legible. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all lubrication points are properly maintained. Grease all fittings according to the manufacturer's recommendations. | ☐ | ☐ | ☐ | ____________________ |

Lockout/Tagout (LOTO) Procedures

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Confirm a written Lockout/Tagout (LOTO) program is in place and readily accessible. | ☐ | ☐ | ☐ | ____________________ |
| Inspect LOTO devices (locks, tags, etc.) for proper function and availability. Ensure sufficient quantity for all energy sources. | ☐ | ☐ | ☐ | ____________________ |
| Before any maintenance or jam clearing, verify that all energy sources (electrical, hydraulic, etc.) are properly isolated and locked out. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all employees who operate or maintain the baler/compactor have received adequate LOTO training and can demonstrate proficiency. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that a machine-specific LOTO procedure is available at the machine and that employees understand it. | ☐ | ☐ | ☐ | ____________________ |
| Verify that LOTO tags are properly filled out with the date, time, and name of the authorized employee. | ☐ | ☐ | ☐ | ____________________ |
| After energy isolation, verify that any stored energy (e.g., hydraulic pressure) is safely dissipated before commencing work. | ☐ | ☐ | ☐ | ____________________ |
| If multiple employees are working on the equipment, ensure a group LOTO procedure is followed with a designated authorized employee. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that the LOTO devices are removed only by the employee who applied them, following a documented procedure. | ☐ | ☐ | ☐ | ____________________ |
| Verify that the LOTO program is periodically inspected (at least annually) to ensure its effectiveness. | ☐ | ☐ | ☐ | ____________________ |

Operational Safety & Housekeeping

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Ensure the area around the baler/compactor is clear of obstructions, debris, and tripping hazards. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that materials are loaded into the baler/compactor safely and evenly to prevent jams or uneven compression. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all warning signs and safety labels are in place, legible, and in the correct language(s). | ☐ | ☐ | ☐ | ____________________ |
| Ensure adequate lighting is provided around the baler/compactor to allow for safe operation and inspection. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that only authorized and trained personnel are operating the baler/compactor. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that employees know how to report any malfunctions, safety concerns, or incidents related to the baler/compactor. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that measures are in place to prevent unauthorized entry into the baling/compaction chamber during operation. | ☐ | ☐ | ☐ | ____________________ |
| Verify that only approved materials are being processed in the baler/compactor. Avoid flammable or explosive materials. | ☐ | ☐ | ☐ | ____________________ |
| Ensure spill control measures are in place to address any potential leaks or spills from the hydraulic system. | ☐ | ☐ | ☐ | ____________________ |
| Verify that the E-stop button is easily accessible and not obstructed by any materials or equipment. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Ensure that appropriate PPE (safety glasses, gloves, hearing protection, etc.) is readily available and in good condition. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that employees are wearing appropriate eye protection (safety glasses or face shields) when operating or maintaining the baler/compactor. | ☐ | ☐ | ☐ | ____________________ |
| Verify that employees are wearing appropriate gloves to protect their hands from cuts, abrasions, or exposure to hydraulic fluid. | ☐ | ☐ | ☐ | ____________________ |
| Ensure employees are wearing appropriate safety shoes or boots with steel toes to protect their feet from falling objects or crushing hazards. | ☐ | ☐ | ☐ | ____________________ |
| If noise levels exceed 85 dBA, confirm that employees are wearing appropriate hearing protection (earplugs or earmuffs). | ☐ | ☐ | ☐ | ____________________ |
| Ensure that employees have been trained on the proper use, maintenance, and limitations of the required PPE. | ☐ | ☐ | ☐ | ____________________ |
| Inspect PPE for any damage, wear, or defects. Replace any compromised PPE immediately. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that PPE fits properly and is comfortable for employees to wear. | ☐ | ☐ | ☐ | ____________________ |
| If working in areas with mobile equipment, ensure employees are wearing high-visibility clothing. | ☐ | ☐ | ☐ | ____________________ |
| If airborne contaminants are present, verify that appropriate respiratory protection is used and a respiratory protection program is in place. | ☐ | ☐ | ☐ | ____________________ |

Fall Protection (If Applicable)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| If elevated platforms are used, inspect guardrails for proper height (42 inches +/- 3 inches) and structural integrity. | ☐ | ☐ | ☐ | ____________________ |
| Verify that midrails are installed halfway between the top rail and the walking surface and that toe boards are at least 3.5 inches high. | ☐ | ☐ | ☐ | ____________________ |
| If fall arrest systems are used, inspect harnesses for any damage, wear, or defects. Ensure proper fit. | ☐ | ☐ | ☐ | ____________________ |
| Inspect lanyards for any cuts, abrasions, or damage. Ensure the lanyard is compatible with the anchorage point. | ☐ | ☐ | ☐ | ____________________ |
| Verify that anchorage points are capable of supporting at least 5,000 pounds per employee attached. | ☐ | ☐ | ☐ | ____________________ |
| Ensure adequate fall clearance below the anchorage point to prevent the worker from striking a lower level in the event of a fall. | ☐ | ☐ | ☐ | ____________________ |
| If self-retracting lifelines (SRLs) are used, inspect them for proper function and ensure they are properly connected. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that employees have been trained on the proper use of fall protection equipment and procedures. | ☐ | ☐ | ☐ | ____________________ |
| Inspect ladders for damage, missing rungs, or slippery surfaces. Ensure ladders are properly secured. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that portable ladders are set up at the correct angle (4:1 ratio) and extend at least 3 feet above the landing. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________