InspectionReport

Hot Work Permit Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Pre-Work Area Assessment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all combustible materials removed from within a 35-foot radius of the hot work area, as per NFPA 51B? | ☐ | ☐ | ☐ | ____________________ |
| If combustibles cannot be removed, are they protected with fire-resistant covers or shields? | ☐ | ☐ | ☐ | ____________________ |
| Is the flooring in the hot work area non-combustible or protected with fire-resistant materials? | ☐ | ☐ | ☐ | ____________________ |
| Are walls and ceilings within 35 feet of the hot work area made of non-combustible materials or protected with fire-resistant shields? | ☐ | ☐ | ☐ | ____________________ |
| Are all openings (e.g., ducts, conveyors) within the 35-foot radius sealed or covered to prevent the spread of sparks and hot materials? | ☐ | ☐ | ☐ | ____________________ |
| Is the sprinkler system fully operational in the hot work area? | ☐ | ☐ | ☐ | ____________________ |
| Are appropriate fire extinguishers (e.g., ABC type) readily available and fully charged within the hot work area? | ☐ | ☐ | ☐ | ____________________ |
| Is the hot work area clean and free of accumulated dust, debris, and other potential fire hazards? | ☐ | ☐ | ☐ | ____________________ |
| If hydraulic equipment is present, are there any visible leaks that could pose a fire hazard? | ☐ | ☐ | ☐ | ____________________ |
| Are all emergency egress routes clear and unobstructed? | ☐ | ☐ | ☐ | ____________________ |

Fire Watch Requirements

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a dedicated fire watch assigned and trained in fire safety procedures? | ☐ | ☐ | ☐ | ____________________ |
| Does the fire watch have appropriate fire extinguishing equipment (e.g., fire extinguisher, charged hose) readily available? | ☐ | ☐ | ☐ | ____________________ |
| Does the fire watch have a reliable means of communication (e.g., radio, phone) to alert others in case of a fire? | ☐ | ☐ | ☐ | ____________________ |
| Is the fire watch scheduled to remain in the area for at least 30 minutes after the completion of hot work? | ☐ | ☐ | ☐ | ____________________ |
| Does the fire watch understand their responsibilities, including monitoring for smoldering fires and alerting emergency services? | ☐ | ☐ | ☐ | ____________________ |
| Has the fire watch received documented training on fire hazards, fire prevention, and the use of fire extinguishing equipment? | ☐ | ☐ | ☐ | ____________________ |
| Does the fire watch have an unobstructed view of the hot work area? | ☐ | ☐ | ☐ | ____________________ |
| Is the fire watch alert and not engaged in other tasks that could distract them from their fire watch duties? | ☐ | ☐ | ☐ | ____________________ |
| Does the fire watch have clear and safe access to all areas within the 35-foot radius? | ☐ | ☐ | ☐ | ____________________ |

Welding Equipment Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the welding machine properly grounded to prevent electrical shock? | ☐ | ☐ | ☐ | ____________________ |
| Is the electrode holder fully insulated and in good condition? | ☐ | ☐ | ☐ | ____________________ |
| Are welding cables free from cuts, abrasions, and other damage? | ☐ | ☐ | ☐ | ____________________ |
| Are gas cylinders secured in an upright position to prevent tipping? | ☐ | ☐ | ☐ | ____________________ |
| Are gas cylinders free from leaks, and are regulators in good working order? | ☐ | ☐ | ☐ | ____________________ |
| Are welding hoses inspected for leaks, cracks, and proper connections? | ☐ | ☐ | ☐ | ____________________ |
| Are flashback arrestors installed on both the torch and regulator of oxy-fuel welding equipment? | ☐ | ☐ | ☐ | ____________________ |
| Is adequate ventilation provided to remove welding fumes and gases from the work area? | ☐ | ☐ | ☐ | ____________________ |
| Is a Lower Explosive Limit (LEL) monitor present and calibrated if flammable gases or vapors may be present? | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the welder wearing a welding helmet with the appropriate shade lens for the welding process? | ☐ | ☐ | ☐ | ____________________ |
| Is the welder wearing appropriate protective clothing (e.g., leather gloves, sleeves, apron) to protect against burns? | ☐ | ☐ | ☐ | ____________________ |
| Is the welder wearing safety glasses or goggles under the welding helmet to provide additional eye protection? | ☐ | ☐ | ☐ | ____________________ |
| Is hearing protection provided and used if noise levels exceed permissible exposure limits? | ☐ | ☐ | ☐ | ____________________ |
| Is respiratory protection provided and used if ventilation is inadequate to control welding fumes and gases? | ☐ | ☐ | ☐ | ____________________ |
| Is the welder wearing appropriate foot protection (e.g., steel-toed boots)? | ☐ | ☐ | ☐ | ____________________ |
| Is the fire watch wearing fire-resistant clothing? | ☐ | ☐ | ☐ | ____________________ |
| Has all PPE been inspected for damage and is in good working condition? | ☐ | ☐ | ☐ | ____________________ |

Permit and Documentation

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a hot work permit properly authorized by a qualified individual? | ☐ | ☐ | ☐ | ____________________ |
| Is the hot work permit displayed prominently at the work site? | ☐ | ☐ | ☐ | ____________________ |
| Is the hot work permit valid for the specific location, time, and type of work being performed? | ☐ | ☐ | ☐ | ____________________ |
| Has a hazard assessment been completed to identify potential fire and explosion hazards? | ☐ | ☐ | ☐ | ____________________ |
| Are safe work procedures documented and followed for the specific hot work task? | ☐ | ☐ | ☐ | ____________________ |
| Has the emergency action plan been reviewed with all personnel involved in the hot work? | ☐ | ☐ | ☐ | ____________________ |
| Are training records available to verify that all personnel involved in the hot work have received appropriate training? | ☐ | ☐ | ☐ | ____________________ |
| Are records of previous hot work inspections maintained and readily available? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________