InspectionReport

Industrial Laser Cutter & Plasma Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Laser Enclosure & Interlock System

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify laser enclosure is intact and free from damage (holes, cracks) that could allow beam escape. (ANSI Z136.1) | ☐ | ☐ | ☐ | ____________________ |
| Test all safety interlocks on access doors and panels. Ensure laser shuts down immediately upon opening. (BS EN 60825-1) | ☐ | ☐ | ☐ | ____________________ |
| Confirm emergency stop buttons are functional and easily accessible at all operator stations. (OSHA 1910.147) | ☐ | ☐ | ☐ | ____________________ |
| Inspect all warning labels for legibility and proper placement on the laser enclosure. (ANSI Z136.1) | ☐ | ☐ | ☐ | ____________________ |
| Examine viewing windows for damage (scratches, cracks) and ensure they provide adequate laser radiation protection. (BS EN 60825-1) | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect the laser beam path for any obstructions or reflective surfaces that could cause unintended beam redirection. (ANSI Z136.1) | ☐ | ☐ | ☐ | ____________________ |
| Verify all electrical connections are secure and properly grounded to prevent electrical shock hazards. (NFPA 70E) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the laser cooling system (if applicable) for leaks and proper operation to prevent overheating. (Manufacturer's Specs) | ☐ | ☐ | ☐ | ____________________ |
| Confirm that laser control software is functioning correctly and prevents unauthorized operation or parameter changes. (Manufacturer's Specs) | ☐ | ☐ | ☐ | ____________________ |
| Verify that the laser system key is secured and only accessible to authorized personnel. (ANSI Z136.1) | ☐ | ☐ | ☐ | ____________________ |

Fume Extraction System

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify airflow indicator on the fume extraction system shows adequate suction during laser/plasma operation. (ACGIH Industrial Ventilation Manual) | ☐ | ☐ | ☐ | ____________________ |
| Inspect fume extraction filters for clogging or damage. Replace filters according to manufacturer's recommendations. (Manufacturer's Specs) | ☐ | ☐ | ☐ | ____________________ |
| Examine fume extraction ductwork for leaks, corrosion, or blockages. (SMACNA Guidelines) | ☐ | ☐ | ☐ | ____________________ |
| Ensure the fume extraction exhaust is located away from air intakes and occupied areas. (ACGIH Industrial Ventilation Manual) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the fume extraction fan is operating smoothly and without excessive noise or vibration. (Manufacturer's Specs) | ☐ | ☐ | ☐ | ____________________ |
| Verify the fume capture hood is properly positioned to effectively capture fumes generated during cutting. (ACGIH Industrial Ventilation Manual) | ☐ | ☐ | ☐ | ____________________ |
| Confirm grounding continuity of the fume extraction system to prevent static electricity buildup and potential fire hazards. (NFPA 70) | ☐ | ☐ | ☐ | ____________________ |
| Review the fume extraction system maintenance log to ensure regular inspections and maintenance are performed. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |
| Inspect and clean or replace pre-filters on the fume extraction system to extend the life of the main filters. (Manufacturer's Specs) | ☐ | ☐ | ☐ | ____________________ |
| Monitor the differential pressure across the filters to determine when filter replacement is necessary. (Manufacturer's Specs) | ☐ | ☐ | ☐ | ____________________ |

Cutting Bed & Fire Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Ensure the cutting bed is free from combustible materials (scrap metal, paper, oil) that could ignite during laser/plasma operation. (NFPA 51B) | ☐ | ☐ | ☐ | ____________________ |
| Verify the fire suppression system (if installed) is functional and properly charged. (NFPA 13) | ☐ | ☐ | ☐ | ____________________ |
| Confirm a suitable fire extinguisher (Class ABC or D) is readily accessible near the laser/plasma cutting area. (NFPA 10) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the fire extinguisher to ensure it is fully charged and has been inspected within the last year. (NFPA 10) | ☐ | ☐ | ☐ | ____________________ |
| If using a water table, verify the water level is adequate to submerge cut parts and prevent sparks. (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Ensure slag and dross are regularly removed from the cutting bed to prevent buildup and potential fire hazards. (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| If using oxygen assist gas, check for leaks in the oxygen supply lines and connections. (CGA G-4) | ☐ | ☐ | ☐ | ____________________ |
| If using nitrogen assist gas, check for leaks in the nitrogen supply lines and connections. (CGA P-1) | ☐ | ☐ | ☐ | ____________________ |
| Verify the workpiece is properly grounded with a grounding clamp to prevent stray currents and potential fire hazards. (NFPA 70) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the material being cut is compatible with the laser/plasma system and does not produce hazardous byproducts. (SDS Review) | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify appropriate laser protective eyewear is available and in good condition (no scratches or damage) for the specific laser wavelength. (ANSI Z136.1) | ☐ | ☐ | ☐ | ____________________ |
| Confirm laser eyewear is certified and labeled with the correct optical density (OD) for the laser in use. (ANSI Z136.1) | ☐ | ☐ | ☐ | ____________________ |
| Ensure a face shield is worn in addition to laser eyewear to protect against sparks and debris. (OSHA 1910.133) | ☐ | ☐ | ☐ | ____________________ |
| Verify appropriate gloves are worn to protect against burns and cuts from handling materials. (OSHA 1910.138) | ☐ | ☐ | ☐ | ____________________ |
| Ensure a fire-resistant apron is worn to protect clothing from sparks and molten metal. (Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Confirm a properly fitted respirator is worn if fume extraction is inadequate or when cutting materials that produce hazardous fumes. (OSHA 1910.134) | ☐ | ☐ | ☐ | ____________________ |
| Verify hearing protection is available and used if noise levels exceed permissible exposure limits. (OSHA 1910.95) | ☐ | ☐ | ☐ | ____________________ |
| Ensure safety shoes are worn to protect feet from falling objects and sharp materials. (OSHA 1910.136) | ☐ | ☐ | ☐ | ____________________ |
| Confirm all operators have been trained on the proper use and maintenance of required PPE. (OSHA 1910.132) | ☐ | ☐ | ☐ | ____________________ |
| Inspect all PPE for damage and ensure it is properly stored when not in use. (OSHA 1910.132) | ☐ | ☐ | ☐ | ____________________ |

Documentation & Training

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify Standard Operating Procedures (SOPs) for the laser/plasma cutter are readily available and up-to-date. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |
| Confirm all operators have received documented training on laser/plasma safety, operation, and emergency procedures. (ANSI Z136.1) | ☐ | ☐ | ☐ | ____________________ |
| Review maintenance logs to ensure regular maintenance and inspections are performed on the laser/plasma system. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |
| Ensure Safety Data Sheets (SDS) for all materials being cut are readily available and reviewed by operators. (OSHA 1910.1200) | ☐ | ☐ | ☐ | ____________________ |
| Verify a risk assessment has been conducted for the laser/plasma cutting process and control measures are implemented. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |
| Confirm an emergency plan is in place and all operators are familiar with emergency procedures. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |
| Verify a designated Laser Safety Officer (LSO) is responsible for overseeing laser safety and compliance. (ANSI Z136.1) | ☐ | ☐ | ☐ | ____________________ |
| If applicable, ensure a permit-to-work system is in place for high-risk laser/plasma cutting operations. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |
| Confirm that the equipment manuals are available and easily accessible for reference. (Manufacturer's Specs) | ☐ | ☐ | ☐ | ____________________ |
| Verify that a system is in place for reporting and investigating incidents and near misses. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________