InspectionReport

Commercial Bakery Oven Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Burn Protection & Thermal Hazards

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are oven door interlocks functioning correctly, preventing conveyor movement when doors are open? Test each door interlock every shift. | ☐ | ☐ | ☐ | ____________________ |
| Are external oven surfaces insulated and below 140°F (60°C) in accessible areas? Use an infrared thermometer to verify. | ☐ | ☐ | ☐ | ____________________ |
| Are clear 'Burn Hazard' warning signs posted near all hot surfaces and oven doors? | ☐ | ☐ | ☐ | ____________________ |
| Are heat-resistant gloves, aprons, and face shields readily available and in good condition for employees working near the oven? | ☐ | ☐ | ☐ | ____________________ |
| Is the oven's cooling system (if equipped) functioning correctly to prevent overheating? | ☐ | ☐ | ☐ | ____________________ |
| Are steam release valves (if applicable) functioning properly and venting steam safely away from personnel? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency stop buttons easily accessible and functioning correctly? Test weekly. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect for any signs of hot spots or discoloration on the oven exterior. Investigate any anomalies. | ☐ | ☐ | ☐ | ____________________ |
| Are all moving parts (e.g., conveyor chains, rollers) adequately guarded to prevent contact burns? | ☐ | ☐ | ☐ | ____________________ |
| Are lockout/tagout procedures in place and followed during maintenance and cleaning to prevent accidental start-up? | ☐ | ☐ | ☐ | ____________________ |

Gas Pilot & Fuel System Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the pilot light burning with a stable, blue flame? Check visually at the start of each shift. | ☐ | ☐ | ☐ | ____________________ |
| Use a gas leak detector to check for gas leaks around all connections, valves, and burners. Perform weekly. | ☐ | ☐ | ☐ | ____________________ |
| Is the gas supply pressure within the manufacturer's specified range? Check monthly with a manometer. | ☐ | ☐ | ☐ | ____________________ |
| Inspect burner flame patterns for proper combustion (blue flame with no yellow tips). | ☐ | ☐ | ☐ | ____________________ |
| Is the combustion air supply unobstructed and adequate for proper burner operation? | ☐ | ☐ | ☐ | ____________________ |
| Is the manual gas shutoff valve easily accessible and clearly labeled? | ☐ | ☐ | ☐ | ____________________ |
| Test the flame failure device to ensure it shuts off the gas supply if the pilot light goes out. Test monthly. | ☐ | ☐ | ☐ | ____________________ |
| Is the oven's ventilation system functioning correctly to remove combustion byproducts? | ☐ | ☐ | ☐ | ____________________ |
| Inspect the gas pressure regulator for leaks, corrosion, or damage. Replace if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Verify the location and accessibility of the emergency gas shutoff switch/valve. Ensure all personnel know its location. | ☐ | ☐ | ☐ | ____________________ |

Combustible Dust Control

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all surfaces (including ledges, ducts, and equipment) free from excessive flour dust accumulation? Clean daily. | ☐ | ☐ | ☐ | ____________________ |
| Is a regular cleaning schedule in place to remove flour dust, and is it being followed? | ☐ | ☐ | ☐ | ____________________ |
| Is a Class II, Division 2 (or better) vacuum cleaner used for dust removal? No sweeping! | ☐ | ☐ | ☐ | ____________________ |
| Are all potential ignition sources (e.g., open flames, sparks, hot surfaces) controlled in areas where flour dust is present? | ☐ | ☐ | ☐ | ____________________ |
| Is all electrical equipment in areas with flour dust rated for Class II, Division 2 environments? | ☐ | ☐ | ☐ | ____________________ |
| Is the dust collection system (if equipped) functioning correctly and properly maintained? | ☐ | ☐ | ☐ | ____________________ |
| Are explosion venting systems (if equipped) unobstructed and properly sized for the oven enclosure? | ☐ | ☐ | ☐ | ____________________ |
| Are good housekeeping practices followed to minimize flour dust accumulation throughout the bakery? | ☐ | ☐ | ☐ | ____________________ |
| Is the proofing area free from excessive flour dust accumulation? Proofing creates a lot of airborne flour. | ☐ | ☐ | ☐ | ____________________ |
| Inspect ductwork for dust buildup and leaks. Clean and repair as needed. | ☐ | ☐ | ☐ | ____________________ |

Fire Suppression Systems

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the fire suppression system (e.g., CO2, dry chemical) inspected and maintained according to the manufacturer's recommendations? Check monthly. | ☐ | ☐ | ☐ | ____________________ |
| Are fire suppression nozzles free from obstruction and properly aimed at potential fire hazards? | ☐ | ☐ | ☐ | ____________________ |
| Is the manual actuation system for the fire suppression system easily accessible and clearly labeled? | ☐ | ☐ | ☐ | ____________________ |
| Are portable fire extinguishers of the appropriate type (Class ABC or BC) readily available near the oven? | ☐ | ☐ | ☐ | ____________________ |
| Are portable fire extinguishers inspected monthly and serviced annually by a qualified technician? | ☐ | ☐ | ☐ | ____________________ |
| Is the fire alarm system functioning correctly and audible throughout the bakery? | ☐ | ☐ | ☐ | ____________________ |
| If a sprinkler system is present, are sprinkler heads free from obstruction and in good condition? | ☐ | ☐ | ☐ | ____________________ |
| Are oven hoods and ductwork cleaned regularly to prevent grease and dust buildup, which can fuel a fire? | ☐ | ☐ | ☐ | ____________________ |
| Verify adequate water supply pressure for sprinkler systems (if applicable). | ☐ | ☐ | ☐ | ____________________ |
| Have employees been trained on the proper use of fire suppression systems and fire extinguishers? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________