InspectionReport

Industrial Sandblasting & Surface Prep Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Respiratory Protection & Air Quality

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is current fit test documentation (within 12 months) available for each abrasive blasting operator using a supplied air respirator? (OSHA 1910.134(f)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect Supplied Air Respirator (SAR) hoses for cuts, kinks, abrasions, and secure connections before each shift. (ANSI Z9.4) | ☐ | ☐ | ☐ | ____________________ |
| Verify air compressor maintenance records are up-to-date, including filter changes and inspections. (Manufacturer's Recommendations) | ☐ | ☐ | ☐ | ____________________ |
| Has the supplied air been tested within the last 6 months for contaminants (oil, water, carbon monoxide) and is the air Grade D or better? (OSHA 1910.134(i)(1)(ii)) | ☐ | ☐ | ☐ | ____________________ |
| Is a continuous carbon monoxide (CO) monitor with audible alarm functioning correctly and positioned to detect CO in the supplied air? (OSHA 1926.57(g)(1)(i)) | ☐ | ☐ | ☐ | ____________________ |
| Are respirators cleaned and disinfected after each use, and properly stored to prevent damage and contamination? (OSHA 1910.134(h)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Is the air pressure at the respirator sufficient to meet the manufacturer's specifications and the operator's needs? (Check with a pressure gauge at the respirator). | ☐ | ☐ | ☐ | ____________________ |
| Is there a backup air supply available in case of primary compressor failure? (Recommended Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Inspect air-line filters for condition and ensure they are appropriate for removing oil, water, and particulates. (Manufacturer's Specifications) | ☐ | ☐ | ☐ | ____________________ |
| Check the respirator exhalation valve for proper function and cleanliness. (OSHA 1910.134) | ☐ | ☐ | ☐ | ____________________ |

Abrasive Blasting Equipment & Controls

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the deadman switch is functioning correctly: nozzle stops immediately when the handle is released. (OSHA 1926.57(c)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect blast hose for cuts, bulges, abrasions, and secure couplings before each shift. (OSHA 1926.302(b)(4)) | ☐ | ☐ | ☐ | ____________________ |
| Check nozzle for wear and proper size. Replace if worn beyond manufacturer's specifications. (Manufacturer's Recommendations) | ☐ | ☐ | ☐ | ____________________ |
| Ensure the abrasive blasting pot is equipped with a functional pressure relief valve. (ASME Boiler and Pressure Vessel Code) | ☐ | ☐ | ☐ | ____________________ |
| Are hose whip checks installed and properly secured on all blast hoses to prevent whipping in case of coupling failure? (Recommended Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| If using a remote control system, verify it is functioning correctly and properly grounded. (OSHA 1926.302) | ☐ | ☐ | ☐ | ____________________ |
| Verify the abrasive being used is the correct type for the application and does not contain free silica above permissible limits if silica is prohibited. (SDS Review) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the blast pot for leaks, corrosion, and structural integrity. (Manufacturer's Recommendations) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the blast pressure is set according to the manufacturer's recommendations and the specific task requirements. (Manufacturer's Recommendations) | ☐ | ☐ | ☐ | ____________________ |
| Verify grounding continuity between the blast pot, nozzle, and workpiece to prevent static discharge. (NFPA 77) | ☐ | ☐ | ☐ | ____________________ |

Blast Booth & Containment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the blast booth ventilation system is operating effectively and maintaining adequate air flow to remove dust and contaminants. (ACGIH Industrial Ventilation Manual) | ☐ | ☐ | ☐ | ____________________ |
| Check dust collector filters and ensure they are clean and functioning properly. (Manufacturer's Recommendations) | ☐ | ☐ | ☐ | ____________________ |
| Inspect blast booth walls, doors, and seals for damage or leaks that could allow dust to escape. (Good Engineering Practices) | ☐ | ☐ | ☐ | ____________________ |
| Ensure the blast booth is properly grounded to prevent static discharge. (NFPA 77) | ☐ | ☐ | ☐ | ____________________ |
| Verify all lighting within the blast booth is explosion-proof and properly maintained. (NFPA 70) | ☐ | ☐ | ☐ | ____________________ |
| Maintain good housekeeping practices within the blast booth to minimize dust accumulation. (General Industry Standards) | ☐ | ☐ | ☐ | ____________________ |
| Conduct air monitoring outside the blast booth to ensure dust levels are within permissible exposure limits (PELs). (OSHA 1910.1000) | ☐ | ☐ | ☐ | ____________________ |
| Verify proper procedures are in place for the collection, storage, and disposal of spent abrasive material. (Local Environmental Regulations) | ☐ | ☐ | ☐ | ____________________ |
| Ensure appropriate warning signs and barriers are in place to restrict access to the blasting area. (ANSI Z535) | ☐ | ☐ | ☐ | ____________________ |
| Verify the blast booth has a readily accessible emergency shutdown switch. (Recommended Best Practice) | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Confirm all personnel within the blasting area are wearing appropriate PPE, including supplied air respirators, hearing protection, eye protection, and protective clothing. (OSHA 1910.132) | ☐ | ☐ | ☐ | ____________________ |
| Operators perform a positive and negative pressure seal check on their respirators before each use. (OSHA 1910.134) | ☐ | ☐ | ☐ | ____________________ |
| Verify hearing protection has an appropriate Noise Reduction Rating (NRR) for the noise levels in the blasting area. (OSHA 1910.95) | ☐ | ☐ | ☐ | ____________________ |
| Ensure eye protection (safety glasses or face shield) is ANSI Z87.1 compliant and provides adequate protection from abrasive particles. (ANSI Z87.1) | ☐ | ☐ | ☐ | ____________________ |
| Inspect protective clothing (e.g., leather or canvas suits) for damage and ensure it provides adequate coverage. (Recommended Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Inspect gloves for holes, tears, and wear. Ensure they provide adequate protection from abrasion and impact. (OSHA 1910.138) | ☐ | ☐ | ☐ | ____________________ |
| Verify foot protection (safety shoes or boots) meets ANSI Z41 standards and provides adequate protection from impact and compression. (ANSI Z41) | ☐ | ☐ | ☐ | ____________________ |
| Are procedures in place for cleaning and maintaining PPE to ensure its effectiveness? (OSHA 1910.132) | ☐ | ☐ | ☐ | ____________________ |
| PPE is stored properly to prevent damage and contamination. (OSHA 1910.132) | ☐ | ☐ | ☐ | ____________________ |
| If required, is a communication system in place and functioning properly to allow operators to communicate with personnel outside the blasting area? (Recommended Best Practice) | ☐ | ☐ | ☐ | ____________________ |

Documentation & Training

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a written hazard communication program in place, including Safety Data Sheets (SDS) for all materials used? (OSHA 1910.1200) | ☐ | ☐ | ☐ | ____________________ |
| Verify employee training records are up-to-date, including training on abrasive blasting hazards, PPE use, and safe operating procedures. (OSHA 1926.21) | ☐ | ☐ | ☐ | ____________________ |
| Are SDS readily available to all employees in the blasting area? (OSHA 1910.1200) | ☐ | ☐ | ☐ | ____________________ |
| Is a written abrasive blasting procedure available and followed? (Recommended Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Is a written emergency action plan in place, including procedures for fire, medical emergencies, and equipment failure? (OSHA 1910.38) | ☐ | ☐ | ☐ | ____________________ |
| Is a first-aid kit readily available and adequately stocked? (OSHA 1910.151) | ☐ | ☐ | ☐ | ____________________ |
| Are records of equipment inspections and maintenance kept and readily available? (Recommended Best Practice) | ☐ | ☐ | ☐ | ____________________ |
| Are air quality test results documented and readily available? (OSHA 1910.134) | ☐ | ☐ | ☐ | ____________________ |
| If blasting in a confined space, is a confined space entry permit system in place and followed? (OSHA 1910.146) | ☐ | ☐ | ☐ | ____________________ |
| Is a Job Hazard Analysis (JHA) or Task Hazard Assessment (THA) completed for the specific blasting task? (Recommended Best Practice) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________