InspectionReport

Ladder Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

Jurisdiction: Adaptable. Covers OSHA 29 CFR 1926.1053 (US) and UK HSE LA455. Adaptable to other jurisdictions. Apply only the section and thresholds for the governing jurisdiction; do not combine US and UK requirements as if they were one code.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Inspection header

| Field | Entry |
| --- | --- |
| Ladder type (extension / step / platform / fixed) | ________________________________________________ |
| Ladder material (fiberglass / aluminum / wood) | ________________________________________________ |
| Ladder ID / serial number | ________________________________________________ |
| Inspector name and date | ________________________________________________ |
| Inspection type (pre-use / detailed periodic) | ________________________________________________ |

Pre-use visual inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Rungs, cleats, steps: no bent, cracked, or missing rungs; evenly spaced (10–14 inches); of grease, oil, mud | ☐ | ☐ | ☐ | ____________________ |
| Stiles (side rails): no twisting, heavy denting, or structural buckling | ☐ | ☐ | ☐ | ____________________ |
| Feet and base: anti-slip rubber feet intact, metal spades sharp and straight (for soft ground) | ☐ | ☐ | ☐ | ____________________ |
| Hardware and joints: tie rods, rivets, welded joints secure; spreader arms lock securely on stepladders | ☐ | ☐ | ☐ | ____________________ |

Material-specific checks

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Fiberglass: no blooming (UV degradation), chipping, or deep scratches | ☐ | ☐ | ☐ | ____________________ |
| Aluminum: no permanent denting or bending; never used near live wiring | ☐ | ☐ | ☐ | ____________________ |
| Wood: no splintering, moisture rot, loose rivets; no opaque paint coatings hiding cracks | ☐ | ☐ | ☐ | ____________________ |

Setup geometry

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| 4-to-1 ratio: base 1 foot out for every 4 feet of height to support point (~75-degree angle) | ☐ | ☐ | ☐ | ____________________ |
| Upper extension: rails extend at least 3 feet (0.9 m) past upper landing surface | ☐ | ☐ | ☐ | ____________________ |
| Three points of contact: two hands + one foot, or two feet + one hand, at all times | ☐ | ☐ | ☐ | ____________________ |
| Belt buckle rule: user's belt buckle remains within stiles; both feet on same rung | ☐ | ☐ | ☐ | ____________________ |

Tag-out and recordkeeping

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Defective ladder tagged 'Do Not Use' and removed from service | ☐ | ☐ | ☐ | ____________________ |
| Detailed visual inspection by competent person at fixed intervals: recorded | ☐ | ☐ | ☐ | ____________________ |
| Inspector signature | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________

References consulted

OSHA 29 CFR 1926.1053 – Ladders

UK HSE – LA455: Safe use of ladders and stepladders

OSHA 1926.1053(b) – Portable ladder requirements

UK HSE – Working at height regulations 2005

Verify the current edition and local adoption before use.