InspectionReport

Commercial Bridge Lead Paint Removal Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Abrasive Blasting Containment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the containment structure (e.g., tarps, scaffolding enclosure) fully intact and free of visible tears or gaps that could allow lead-containing debris to escape? | ☐ | ☐ | ☐ | ____________________ |
| Is the HEPA-filtered negative pressure system operating continuously and maintaining adequate airflow to prevent dust migration outside the containment area? Verify pressure differential with manometer. | ☐ | ☐ | ☐ | ____________________ |
| Are HEPA filters on the exhaust system clean and properly installed? Check filter pressure drop against manufacturer's specifications. Document filter changes. | ☐ | ☐ | ☐ | ____________________ |
| Are spent abrasive and lead-containing waste materials being collected in sealed, labeled containers for proper disposal as hazardous waste? | ☐ | ☐ | ☐ | ____________________ |
| Are measures in place to prevent lead-contaminated water runoff from entering waterways (e.g., berms, collection systems, proper disposal)? | ☐ | ☐ | ☐ | ____________________ |
| Is perimeter air monitoring being conducted to ensure lead levels outside the containment area are below permissible exposure limits (PEL)? Review air monitoring data. | ☐ | ☐ | ☐ | ____________________ |
| Is equipment being properly decontaminated before removal from the containment area to prevent the spread of lead contamination? | ☐ | ☐ | ☐ | ____________________ |
| Are warning signs posted at all entrances to the work area indicating the presence of lead and required PPE? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency procedures in place and readily available in case of a containment breach or other incident? | ☐ | ☐ | ☐ | ____________________ |
| Is the abrasive being used approved for lead paint removal and minimizing dust generation (e.g., recyclable steel grit)? | ☐ | ☐ | ☐ | ____________________ |

Worker Hygiene Facilities

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are readily accessible hand-washing stations with hot/cold running water, soap, and single-use towels provided near the work area and change rooms? | ☐ | ☐ | ☐ | ____________________ |
| Are separate change rooms provided for street clothes and work clothes to prevent cross-contamination? | ☐ | ☐ | ☐ | ____________________ |
| Are shower facilities with hot/cold running water and soap provided for workers to shower at the end of each shift? | ☐ | ☐ | ☐ | ____________________ |
| Are eating and drinking areas separate from the work area to prevent ingestion of lead? | ☐ | ☐ | ☐ | ____________________ |
| Is the lunchroom clean and free of visible lead dust? Are surfaces easily cleanable? | ☐ | ☐ | ☐ | ____________________ |
| Have workers received training on proper hygiene practices to minimize lead exposure? | ☐ | ☐ | ☐ | ____________________ |
| Are contaminated towels and clothing being disposed of properly as hazardous waste? | ☐ | ☐ | ☐ | ____________________ |
| Is there a strict no-smoking/tobacco policy enforced in the work area to prevent ingestion of lead? | ☐ | ☐ | ☐ | ____________________ |
| Are respirators stored in a clean, dry place away from contamination? | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are workers wearing appropriate respirators (e.g., full-face powered air-purifying respirators - PAPRs) with HEPA filters, properly fitted and maintained? | ☐ | ☐ | ☐ | ____________________ |
| Have workers undergone respirator fit testing within the past year? | ☐ | ☐ | ☐ | ____________________ |
| Are workers wearing disposable protective clothing (e.g., coveralls, gloves, shoe covers) to prevent lead contamination of personal clothing? | ☐ | ☐ | ☐ | ____________________ |
| Are gloves intact and free of tears or punctures? | ☐ | ☐ | ☐ | ____________________ |
| Are workers wearing appropriate eye protection (e.g., safety glasses or goggles) to prevent lead dust from entering the eyes? | ☐ | ☐ | ☐ | ____________________ |
| Are workers following proper procedures for donning and doffing PPE to prevent self-contamination? | ☐ | ☐ | ☐ | ____________________ |
| Is contaminated PPE being disposed of properly as hazardous waste? | ☐ | ☐ | ☐ | ____________________ |
| Are workers inspecting their PPE before each use for damage or defects? | ☐ | ☐ | ☐ | ____________________ |
| Are workers wearing hearing protection if noise levels exceed 85 dBA? | ☐ | ☐ | ☐ | ____________________ |

Documentation and Training

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Has a comprehensive lead hazard assessment been conducted for the project? | ☐ | ☐ | ☐ | ____________________ |
| Is a written lead compliance program in place and being followed? | ☐ | ☐ | ☐ | ____________________ |
| Are worker training records up-to-date and readily available for review? | ☐ | ☐ | ☐ | ____________________ |
| Are workers enrolled in a medical surveillance program, including blood lead level monitoring? | ☐ | ☐ | ☐ | ____________________ |
| Review recent blood lead level results. Are any workers exceeding the action level (30 µg/dL) or permissible exposure limit (50 µg/dL)? | ☐ | ☐ | ☐ | ____________________ |
| Is a written respirator program in place, including procedures for selection, fit testing, maintenance, and training? | ☐ | ☐ | ☐ | ____________________ |
| Are hazardous waste disposal manifests properly completed and retained? | ☐ | ☐ | ☐ | ____________________ |
| Are daily inspection logs being maintained for the containment system and other critical safety equipment? | ☐ | ☐ | ☐ | ____________________ |
| Is a competent person designated to oversee the lead paint removal project and ensure compliance with all applicable regulations? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________