InspectionReport

Masonry Block Wall Bracing & Scaffolding Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Limited Access Zone (LAZ) & Wall Bracing

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a Limited Access Zone (LAZ) established equal to the height of the wall plus 4 feet, on the unscaffolded side of the wall, before wall construction begins? (OSHA 1926.706(b)) | ☐ | ☐ | ☐ | ____________________ |
| Is access to the LAZ restricted to essential personnel only? (OSHA 1926.706(b)) | ☐ | ☐ | ☐ | ____________________ |
| Is a written wall bracing schedule available and followed? (OSHA 1926.706(a)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Is wall bracing installed according to the bracing schedule and manufacturer's specifications? (OSHA 1926.706(a)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Is the horizontal and vertical spacing of wall bracing within the limits specified by the engineer or qualified person? (OSHA 1926.706(a)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Are wall bracing anchors properly installed and secured to the wall and the supporting structure? (OSHA 1926.706(a)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Are all bracing components free from damage, corrosion, and defects? (OSHA 1926.706(a)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Is bracing adjusted or modified as the wall progresses in height to maintain stability? (OSHA 1926.706(a)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Has wind load been considered in the bracing design, especially for walls exceeding 8 feet in height? (ANSI A10.8) | ☐ | ☐ | ☐ | ____________________ |
| Is the LAZ clearly marked with warning signs and barricades to prevent unauthorized entry? (OSHA 1926.200) | ☐ | ☐ | ☐ | ____________________ |

Scaffold Erection & Stability

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Was the scaffold erected, moved, dismantled, or altered under the supervision of a competent person? (OSHA 1926.451(f)(7)) | ☐ | ☐ | ☐ | ____________________ |
| Is the scaffold base level and on a firm foundation, using base plates and mud sills as needed? (OSHA 1926.451(c)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Are all scaffold frames properly braced with cross bracing or diagonal bracing to prevent racking? (OSHA 1926.451(d)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Is the scaffold tied to the wall or structure at intervals not exceeding 30 feet horizontally and 26 feet vertically? (OSHA 1926.451(e)(6)) | ☐ | ☐ | ☐ | ____________________ |
| Are scaffold planks of the correct grade, inspected for defects, and overlapping supports by at least 12 inches? (OSHA 1926.451(b)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are guardrails installed on all open sides and ends of the scaffold platform, with a top rail height of 42 inches +/- 3 inches? (OSHA 1926.451(g)(4)) | ☐ | ☐ | ☐ | ____________________ |
| Are toeboards at least 3.5 inches high installed on all open sides and ends of the scaffold platform? (OSHA 1926.451(h)(2)) | ☐ | ☐ | ☐ | ____________________ |
| Is safe access to the scaffold provided, such as a ladder, stairway, or ramp? (OSHA 1926.451(e)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Is the scaffold load capacity clearly posted and not exceeded? (OSHA 1926.451(b)(7)) | ☐ | ☐ | ☐ | ____________________ |
| Has the scaffold been inspected by a competent person before each work shift? (OSHA 1926.451(f)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Is the scaffold tagged with the appropriate status (e.g., green for safe, yellow for caution, red for unsafe)? | ☐ | ☐ | ☐ | ____________________ |

Mortar Boards & Material Handling

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the mortar board load within the scaffold's rated capacity, considering the weight of the mortar and the board itself? (OSHA 1926.451(b)(7)) | ☐ | ☐ | ☐ | ____________________ |
| Is the mortar board placed securely on the scaffold platform, preventing tipping or sliding? (OSHA 1926.451(b)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Is the mortar board in good condition, free from cracks, rot, or other defects that could compromise its stability? (General Duty Clause) | ☐ | ☐ | ☐ | ____________________ |
| Are CMU blocks and other materials stacked neatly and securely on the scaffold, preventing them from falling? (OSHA 1926.250(a)(1)) | ☐ | ☐ | ☐ | ____________________ |
| If materials are being hoisted to the scaffold, is a tag line used to control the load and prevent swinging? (OSHA 1926.753(d)(2)(ii)) | ☐ | ☐ | ☐ | ____________________ |
| If material chutes are used, are they properly constructed and secured to prevent collapse or uncontrolled discharge? (OSHA 1926.852) | ☐ | ☐ | ☐ | ____________________ |
| Is the scaffold platform kept clear of debris and obstructions to prevent tripping hazards? (OSHA 1926.451(b)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Is overhand bricklaying limited to situations where it is safe and necessary, with appropriate fall protection measures in place? (OSHA 1926.451(b)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Is mortar spillage cleaned up promptly to prevent slip hazards on the scaffold and below? (OSHA 1926.451(b)(3)) | ☐ | ☐ | ☐ | ____________________ |
| Are materials staged in a manner that does not obstruct access to the scaffold or create a hazard for workers below? (OSHA 1926.250(b)(4)) | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all workers wearing hard hats that meet ANSI Z89.1 standards? (OSHA 1926.100(a)) | ☐ | ☐ | ☐ | ____________________ |
| Are workers wearing appropriate eye protection (safety glasses or goggles) when cutting or chipping CMU blocks? (OSHA 1926.102(a)(1)) | ☐ | ☐ | ☐ | ____________________ |
| Are workers wearing gloves appropriate for handling CMU blocks and mortar? (General Duty Clause) | ☐ | ☐ | ☐ | ____________________ |
| Are workers wearing safety shoes or boots that meet ANSI Z41 standards? (OSHA 1926.96(b)) | ☐ | ☐ | ☐ | ____________________ |
| Are workers using appropriate fall protection (harness and lanyard) when working at heights above 6 feet on the scaffold? (OSHA 1926.501(b)(13)) | ☐ | ☐ | ☐ | ____________________ |
| Has the fall protection equipment been inspected before use for any signs of damage or wear? (OSHA 1926.502(d)(21)) | ☐ | ☐ | ☐ | ____________________ |
| Are workers wearing dust masks or respirators when exposed to excessive dust from cutting or mixing mortar? (OSHA 1926.103) | ☐ | ☐ | ☐ | ____________________ |
| Is all PPE properly fitted and maintained? (OSHA 1910.132(d)) | ☐ | ☐ | ☐ | ____________________ |
| Have workers been trained on the proper use and care of their PPE? (OSHA 1910.132(f)) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________