InspectionReport

Boom Lift Daily Inspection Checklist - Interactive MEWP Check

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Mobile Equipment & Structural Components

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect tires for proper inflation, cuts, and wear. Check wheels for damage and secure lug nuts. (ANSI A92.22) | ☐ | ☐ | ☐ | ____________________ |
| Examine the chassis for any signs of structural damage, cracks, or corrosion. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Verify guardrails and toe boards are securely in place and undamaged. Ensure mid-rails are present and intact. (OSHA 1926.451(g)(4)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the platform floor for any holes, cracks, or signs of weakening. Ensure the platform gate latches securely. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| If equipped, test outriggers for proper function and stability. Ensure outrigger pads are used on soft surfaces. (ANSI A92.22) | ☐ | ☐ | ☐ | ____________________ |
| Inspect boom sections for damage, bends, or cracks. Check boom wear pads for excessive wear. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Examine all welds and connections for cracks or signs of failure. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Verify the emergency lowering system is functional and easily accessible. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Test the horn and all audible/visual alarms to ensure they are functioning correctly. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Ensure all control markings are legible and correspond to the correct function. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |

Hydraulic & Mechanical Systems

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect hydraulic hoses, fittings, and cylinders for leaks. Check for signs of hose damage or wear. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Verify the hydraulic fluid level is within the recommended range. (Manufacturer Specification) | ☐ | ☐ | ☐ | ____________________ |
| Observe cylinder operation for smooth and consistent movement. Check for jerky or erratic motion. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Ensure hydraulic hoses are properly routed and secured to prevent chafing or damage. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Verify the engine oil level is within the recommended range. (Manufacturer Specification) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the battery for corrosion, damage, and proper connections. (Manufacturer Specification) | ☐ | ☐ | ☐ | ____________________ |
| Check the fuel level and ensure it is sufficient for the planned operation. (Manufacturer Specification) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the air filter for cleanliness and damage. (Manufacturer Specification) | ☐ | ☐ | ☐ | ____________________ |
| Test all limit switches to ensure they are functioning correctly and prevent over-travel. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the wiring harness for damage, fraying, or loose connections. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |

Safety & Emergency Equipment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect the fall arrest anchor point for damage and secure attachment. (OSHA 1926.502(d)) | ☐ | ☐ | ☐ | ____________________ |
| Examine the lanyard for cuts, abrasions, or damage to the snap hooks. (OSHA 1926.502(d)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the full-body harness for tears, fraying, or damage to the buckles. (OSHA 1926.502(d)) | ☐ | ☐ | ☐ | ____________________ |
| Verify the fire extinguisher is present, charged, and readily accessible. (NFPA 10) | ☐ | ☐ | ☐ | ____________________ |
| Ensure a first-aid kit is available and adequately stocked. (OSHA 1910.151) | ☐ | ☐ | ☐ | ____________________ |
| Test the ground control override function to ensure it operates correctly. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| If equipped, inspect the emergency descent device for proper function. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| If equipped, test proximity alarms to ensure they are functioning correctly. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |

Documentation & Manuals

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the operator's manual is present and legible on the MEWP. (ANSI A92.24) | ☐ | ☐ | ☐ | ____________________ |
| Review maintenance records to ensure the MEWP has been properly serviced. (ANSI A92.24) | ☐ | ☐ | ☐ | ____________________ |
| Ensure the inspection log is up-to-date and reflects recent inspections. (ANSI A92.24) | ☐ | ☐ | ☐ | ____________________ |
| Verify the load chart is present and legible. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Ensure all warning labels are present and legible. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the MEWP has undergone its required annual inspection. (ANSI A92.20) | ☐ | ☐ | ☐ | ____________________ |
| Verify the operator is certified to operate the specific type of MEWP. (ANSI A92.24) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________