InspectionReport

Underground Mine Ventilation & Gas Detection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Ventilation System Integrity

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the approved MSHA ventilation plan readily available and understood by all personnel in the working section? (30 CFR §75.370) | ☐ | ☐ | ☐ | ____________________ |
| Is the required CFM (Cubic Feet per Minute) of airflow being maintained at the working face as specified in the ventilation plan? Measure airflow using an anemometer. (30 CFR §75.360) | ☐ | ☐ | ☐ | ____________________ |
| Inspect brattice cloth for tears, holes, or improper installation. Are all overlaps properly sealed to prevent air leakage? (30 CFR §75.333) | ☐ | ☐ | ☐ | ____________________ |
| Is the line brattice installed within the specified distance from the face as per the ventilation plan (typically within 10 feet)? (30 CFR §75.333(h)) | ☐ | ☐ | ☐ | ____________________ |
| Are all ventilation controls (e.g., stoppings, regulators, overcasts) free from damage and functioning as intended? (30 CFR §75.334) | ☐ | ☐ | ☐ | ____________________ |
| Check stoppings for excessive air leakage. Seal any leaks immediately to maintain proper airflow. (30 CFR §75.334) | ☐ | ☐ | ☐ | ____________________ |
| If booster fans are in use, are they operating correctly and inspected regularly? Are the required safety devices (e.g., methane monitors, automatic shut-off) functioning properly? (30 CFR §75.312) | ☐ | ☐ | ☐ | ____________________ |
| Are auxiliary fans positioned correctly to effectively ventilate the face without creating recirculation? (30 CFR §75.311) | ☐ | ☐ | ☐ | ____________________ |
| Verify air velocity at designated monitoring points aligns with the ventilation plan. Record measurements. (30 CFR §75.360) | ☐ | ☐ | ☐ | ____________________ |
| Are all changes to the ventilation system documented and approved by a qualified person? (30 CFR §75.370) | ☐ | ☐ | ☐ | ____________________ |

Gas Detection and Monitoring

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all methane monitors properly calibrated and functioning correctly? Check calibration dates. (30 CFR §75.340) | ☐ | ☐ | ☐ | ____________________ |
| Monitor methane levels at the working face. If methane levels exceed 1.0%, take immediate corrective action. If levels exceed 1.5%, de-energize equipment and withdraw personnel. (30 CFR §75.323) | ☐ | ☐ | ☐ | ____________________ |
| Monitor methane levels in the return air. If methane levels exceed 2.0%, take immediate corrective action. (30 CFR §75.323) | ☐ | ☐ | ☐ | ____________________ |
| Check for adequate oxygen levels (minimum 19.5%). Use a multi-gas meter to monitor oxygen, carbon monoxide, and hydrogen sulfide. (30 CFR §75.321) | ☐ | ☐ | ☐ | ____________________ |
| Monitor for carbon monoxide (CO) levels. Take action if levels exceed permissible limits. (30 CFR §75.321) | ☐ | ☐ | ☐ | ____________________ |
| Verify that gas monitor alarms are functioning correctly and audible in the working area. (30 CFR §75.342) | ☐ | ☐ | ☐ | ____________________ |
| Are all personnel trained in the proper use and maintenance of gas monitoring equipment? (30 CFR §75.1501) | ☐ | ☐ | ☐ | ____________________ |
| If methane drainage systems are in place, are they operating effectively and monitored regularly? (30 CFR §75.335) | ☐ | ☐ | ☐ | ____________________ |
| Assess the effectiveness of rock dusting in preventing coal dust explosions. Ensure adequate coverage, especially in return airways. (30 CFR §75.400) | ☐ | ☐ | ☐ | ____________________ |
| Are records of methane monitor readings and calibrations maintained and readily available for review? (30 CFR §75.340) | ☐ | ☐ | ☐ | ____________________ |

Equipment and Electrical Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is all electrical equipment used in the working section MSHA-permissible and properly maintained? (30 CFR §75.500) | ☐ | ☐ | ☐ | ____________________ |
| Inspect trailing cables for damage, splices, and proper support. Ensure cables are not lying in water or across walkways. (30 CFR §75.517) | ☐ | ☐ | ☐ | ____________________ |
| Verify grounding continuity of all electrical equipment. (30 CFR §75.600) | ☐ | ☐ | ☐ | ____________________ |
| Ensure all machine guards are in place and functioning correctly to protect workers from moving parts. (30 CFR §75.1725) | ☐ | ☐ | ☐ | ____________________ |
| Is adequate lighting provided in the working section to ensure safe visibility? (30 CFR §75.1719) | ☐ | ☐ | ☐ | ____________________ |
| Are fire suppression systems on mobile equipment inspected and maintained regularly? (30 CFR §75.1101-23) | ☐ | ☐ | ☐ | ____________________ |
| Are battery charging areas properly ventilated to prevent the accumulation of explosive gases? (30 CFR §75.526) | ☐ | ☐ | ☐ | ____________________ |
| Are maintenance records for all equipment readily available and up-to-date? (30 CFR §75.1719) | ☐ | ☐ | ☐ | ____________________ |
| Verify that emergency shutdown devices are accessible and functioning properly on all equipment. (30 CFR §75.1725) | ☐ | ☐ | ☐ | ____________________ |

Emergency Preparedness

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are escapeways clear, unobstructed, and properly marked? (30 CFR §75.380) | ☐ | ☐ | ☐ | ____________________ |
| Is a reliable communication system in place to contact personnel in case of an emergency? Test the system regularly. (30 CFR §75.1600) | ☐ | ☐ | ☐ | ____________________ |
| Are self-rescuers readily available and in good working condition for all personnel? (30 CFR §75.1714) | ☐ | ☐ | ☐ | ____________________ |
| Is the mine's emergency response plan up-to-date and understood by all personnel? (30 CFR §75.1502) | ☐ | ☐ | ☐ | ____________________ |
| Are adequate first-aid supplies readily available and accessible? (30 CFR §75.1713) | ☐ | ☐ | ☐ | ____________________ |
| Are fire extinguishers readily accessible, inspected regularly, and properly charged? (30 CFR §75.1101-17) | ☐ | ☐ | ☐ | ____________________ |
| Are emergency drills conducted regularly to ensure personnel are familiar with evacuation procedures? (30 CFR §75.1502) | ☐ | ☐ | ☐ | ____________________ |
| Verify the availability and readiness of the mine rescue team. (30 CFR Part 49) | ☐ | ☐ | ☐ | ____________________ |
| Are up-to-date mine maps readily available for emergency response purposes? (30 CFR §75.1200) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________