InspectionReport

Rotary Piling Rig Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Working Platform & Ground Stability

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Has a geotechnical engineer certified the working platform's ground bearing pressure is adequate for the rig's maximum operating weight (including Kelly bar, auger, and spoil)? | ☐ | ☐ | ☐ | ____________________ |
| Is a daily visual inspection of the working platform performed to check for settlement, cracks, or water accumulation? Documented in daily log? | ☐ | ☐ | ☐ | ____________________ |
| Is the working platform adequately sloped to ensure proper drainage and prevent water pooling near the rig's tracks or outriggers? | ☐ | ☐ | ☐ | ____________________ |
| Are outriggers (if used) fully extended and placed on stable, load-distributing pads of sufficient size to prevent sinking or movement? | ☐ | ☐ | ☐ | ____________________ |
| Is there a system in place to monitor groundwater levels and their potential impact on platform stability? Are dewatering measures in place if needed? | ☐ | ☐ | ☐ | ____________________ |
| Are slopes surrounding the working platform stable and protected from erosion or collapse that could undermine the platform? | ☐ | ☐ | ☐ | ____________________ |
| Is vibration monitoring being conducted to assess the impact of rig operations on nearby structures or utilities? | ☐ | ☐ | ☐ | ____________________ |
| Are access and egress routes to the working platform clear, stable, and free from obstructions? | ☐ | ☐ | ☐ | ____________________ |
| Is the working platform constructed with properly compacted materials to achieve the required ground bearing pressure? | ☐ | ☐ | ☐ | ____________________ |
| Has the working platform stability been reassessed following significant rainfall or other adverse weather conditions? | ☐ | ☐ | ☐ | ____________________ |

Wire Rope & Lifting Equipment

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are wire ropes inspected daily for wear, corrosion, kinks, and broken wires, with results documented in a logbook? | ☐ | ☐ | ☐ | ____________________ |
| Are wire ropes replaced when they exhibit excessive wear, corrosion, or the number of broken wires exceeds the manufacturer's recommendations? | ☐ | ☐ | ☐ | ____________________ |
| Are sheaves and drums inspected for wear, cracks, and proper lubrication to prevent damage to wire ropes? | ☐ | ☐ | ☐ | ____________________ |
| Are all lifting gear (slings, shackles, hooks) certified and inspected regularly, with valid documentation available on site? | ☐ | ☐ | ☐ | ____________________ |
| Are hook safety latches functioning correctly and in place on all lifting hooks? | ☐ | ☐ | ☐ | ____________________ |
| Is there a documented lubrication program for wire ropes and lifting equipment, using the manufacturer's recommended lubricants? | ☐ | ☐ | ☐ | ____________________ |
| Is the wire rope reeving system properly installed and maintained according to the rig manufacturer's specifications? | ☐ | ☐ | ☐ | ____________________ |
| Has the lifting equipment been load-tested within the past year, and is documentation available? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency stop buttons for the hoisting system readily accessible and functioning correctly? | ☐ | ☐ | ☐ | ____________________ |
| Are tag lines used to control the load during lifting operations, preventing uncontrolled swinging? | ☐ | ☐ | ☐ | ____________________ |

Underground Utilities & Excavation Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Has a comprehensive utility locate request been submitted and reviewed prior to any excavation or drilling? | ☐ | ☐ | ☐ | ____________________ |
| Are all identified underground utilities clearly marked with paint, flags, or other visible means? | ☐ | ☐ | ☐ | ____________________ |
| Are safe digging practices (e.g., hand digging, vacuum excavation) employed when working near marked utilities? | ☐ | ☐ | ☐ | ____________________ |
| Is there a sufficient clearance distance maintained between the drilling operation and any identified underground utilities? | ☐ | ☐ | ☐ | ____________________ |
| Is emergency contact information for utility companies readily available on site? | ☐ | ☐ | ☐ | ____________________ |
| Is a dedicated spotter present during drilling operations near underground utilities to monitor for potential hazards? | ☐ | ☐ | ☐ | ____________________ |
| Are excavations properly shored or sloped to prevent collapse, especially in unstable soil conditions? | ☐ | ☐ | ☐ | ____________________ |
| Are confined space entry procedures followed if personnel are required to enter excavations? | ☐ | ☐ | ☐ | ____________________ |
| Is atmospheric monitoring conducted in excavations to check for hazardous gases or oxygen deficiency? | ☐ | ☐ | ☐ | ____________________ |
| Are trench boxes or shoring systems inspected daily for damage or defects? | ☐ | ☐ | ☐ | ____________________ |
| Is a competent person (as defined by OSHA) on site to inspect excavations and shoring systems? | ☐ | ☐ | ☐ | ____________________ |

Rig Operation & Maintenance

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a daily pre-start inspection of the rotary piling rig performed, including checks of fluid levels, hoses, and structural components? | ☐ | ☐ | ☐ | ____________________ |
| Are maintenance records up-to-date and readily available for review? | ☐ | ☐ | ☐ | ____________________ |
| Is the Kelly bar inspected for wear, cracks, and proper alignment before each shift? | ☐ | ☐ | ☐ | ____________________ |
| Is the auger inspected for damaged or missing teeth, and is it appropriate for the soil conditions? | ☐ | ☐ | ☐ | ____________________ |
| Are hydraulic hoses and fittings inspected for leaks and damage? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency shutdown procedures clearly posted and understood by all operators? | ☐ | ☐ | ☐ | ____________________ |
| Are all rig operators properly trained and certified to operate the specific type of rotary piling rig being used? | ☐ | ☐ | ☐ | ____________________ |
| Are load charts readily available and understood by the operator? | ☐ | ☐ | ☐ | ____________________ |
| Is the rig equipped with a functioning level indicator to ensure it is operating within safe limits? | ☐ | ☐ | ☐ | ____________________ |
| Are noise levels monitored, and are hearing protection measures in place if noise levels exceed permissible limits? | ☐ | ☐ | ☐ | ____________________ |
| Are exhaust emissions properly controlled to prevent exposure to hazardous fumes? | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are hard hats worn by all personnel within the work zone? | ☐ | ☐ | ☐ | ____________________ |
| Are safety glasses or face shields worn to protect against flying debris? | ☐ | ☐ | ☐ | ____________________ |
| Are all personnel wearing high-visibility clothing (e.g., vests, jackets)? | ☐ | ☐ | ☐ | ____________________ |
| Are steel-toe boots worn to protect against foot injuries? | ☐ | ☐ | ☐ | ____________________ |
| Is hearing protection available and used in areas with high noise levels? | ☐ | ☐ | ☐ | ____________________ |
| Are appropriate gloves worn to protect against hand injuries? | ☐ | ☐ | ☐ | ____________________ |
| Is respiratory protection required and used if there is a risk of exposure to dust or fumes? | ☐ | ☐ | ☐ | ____________________ |
| Is PPE inspected regularly for damage or defects? | ☐ | ☐ | ☐ | ____________________ |
| Have all personnel been trained on the proper use and maintenance of PPE? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________