InspectionReport

Powder Coating Booth & Oven Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Booth Ventilation & Airflow

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify booth airflow is within the manufacturer's specified range (typically 100-120 linear feet per minute) using an anemometer. Record readings. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the pressure drop across the filters. Ensure it's within the recommended range indicated by the filter manufacturer. Replace filters if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Examine exhaust ductwork for any signs of dust accumulation, damage, or leaks. Clean or repair as needed. | ☐ | ☐ | ☐ | ____________________ |
| Verify the powder recovery system (e.g., cyclone, baghouse) is functioning correctly and efficiently collecting overspray. Check for blockages. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the compressed air supply for moisture or oil contamination. Ensure air is clean and dry to prevent powder clumping and application issues. | ☐ | ☐ | ☐ | ____________________ |
| Test airflow alarms (if equipped) to ensure they activate when airflow drops below the setpoint. Document test results. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the booth enclosure for any gaps or openings that could allow powder to escape. Seal any openings. | ☐ | ☐ | ☐ | ____________________ |
| Measure static pressure within the booth. Compare to the manufacturer's specifications. Investigate any deviations. | ☐ | ☐ | ☐ | ____________________ |
| Inspect filter seals for proper seating and integrity. Replace damaged seals immediately. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect interior of ductwork (where accessible) for excessive powder buildup. Schedule cleaning if needed. | ☐ | ☐ | ☐ | ____________________ |

Grounding & Bonding

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify grounding hook continuity to the booth ground using a multimeter. Resistance should be less than 1 ohm. | ☐ | ☐ | ☐ | ____________________ |
| Ensure the electrostatic spray gun is properly grounded. Check the grounding cable for damage and secure connections. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that parts being coated are properly grounded before spraying. Use a grounding clamp or fixture. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the booth's grounding system for corrosion or damage. Ensure all connections are tight and secure. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all solvent containers within the booth are properly grounded and bonded to prevent static buildup. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect all grounding wires for cuts, frays, or loose connections. Repair or replace as needed. | ☐ | ☐ | ☐ | ____________________ |
| Measure the resistance to ground of the booth structure. It should be less than 1 ohm. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the spray gun cable for damage, kinks, or wear. Replace if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all grounding points are clean and free from paint or other coatings that could impede conductivity. | ☐ | ☐ | ☐ | ____________________ |
| Document the date and results of the last grounding continuity test. Ensure tests are performed weekly. | ☐ | ☐ | ☐ | ____________________ |

Oven Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify oven temperature is within the specified range for the powder being cured. Use a calibrated thermometer. | ☐ | ☐ | ☐ | ____________________ |
| Check for gas leaks around the oven using a combustible gas detector. Pay close attention to connections and fittings. | ☐ | ☐ | ☐ | ____________________ |
| Ensure oven ventilation is functioning properly to remove solvent vapors and prevent buildup of combustible gases. | ☐ | ☐ | ☐ | ____________________ |
| Test oven interlocks (e.g., door switches, airflow sensors) to ensure they shut down the oven in case of a malfunction. | ☐ | ☐ | ☐ | ____________________ |
| Verify the emergency shutoff switch for the oven is easily accessible and functioning correctly. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the interior of the oven for powder buildup. Clean regularly to prevent fire hazards. | ☐ | ☐ | ☐ | ____________________ |
| Verify the oven temperature controller is calibrated. Schedule recalibration as needed. | ☐ | ☐ | ☐ | ____________________ |
| Inspect oven door seals for damage or deterioration. Replace if necessary to prevent heat loss and maintain temperature control. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect the burner flame for proper color and stability. Report any unusual flame characteristics. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the oven purge cycle is functioning correctly before each startup to remove any accumulated gases. | ☐ | ☐ | ☐ | ____________________ |

Housekeeping & Combustible Dust Control

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect all surfaces (floors, walls, equipment) for dust accumulation. Remove any visible dust using a vacuum with a HEPA filter. | ☐ | ☐ | ☐ | ____________________ |
| Verify the vacuum system used for dust removal is properly grounded and designed for combustible dust. Empty dust collectors regularly. | ☐ | ☐ | ☐ | ____________________ |
| Minimize the use of compressed air for cleaning, as it can create dust clouds. Use vacuuming or wet wiping methods instead. | ☐ | ☐ | ☐ | ____________________ |
| Ensure spill control materials (e.g., absorbent pads) are readily available for powder spills. Clean up spills immediately. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the dust collection system for leaks or damage. Repair or replace as needed. | ☐ | ☐ | ☐ | ____________________ |
| Verify all electrical equipment in the area is rated for Class II, Division 1 or 2 environments to prevent ignition of combustible dust. | ☐ | ☐ | ☐ | ____________________ |
| Eliminate potential ignition sources (e.g., open flames, smoking, hot surfaces) from the powder coating area. | ☐ | ☐ | ☐ | ____________________ |
| Check light fixtures for dust buildup. Clean regularly to prevent overheating and potential ignition. | ☐ | ☐ | ☐ | ____________________ |
| Review and adhere to the established housekeeping schedule for powder coating areas. Document cleaning activities. | ☐ | ☐ | ☐ | ____________________ |
| Conduct regular dust explosibility testing to determine the Lower Explosive Limit (LEL) and Minimum Ignition Energy (MIE) of the powder being used. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify that all personnel are wearing appropriate respirators (e.g., N95, PAPR) when working in the booth or handling powder. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that all personnel are wearing appropriate eye protection (e.g., safety glasses, goggles) to prevent powder from entering the eyes. | ☐ | ☐ | ☐ | ____________________ |
| Verify that personnel are wearing appropriate gloves to protect their skin from powder and solvents. | ☐ | ☐ | ☐ | ____________________ |
| Ensure personnel are wearing appropriate clothing (e.g., long sleeves, long pants) to minimize skin exposure to powder. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that personnel are wearing static-dissipative clothing to prevent static buildup, especially when using electrostatic spray equipment. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all PPE for damage or wear. Replace damaged PPE immediately. | ☐ | ☐ | ☐ | ____________________ |
| Verify that personnel have been properly fit-tested for their respirators. Document fit-testing results. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that an adequate supply of PPE is readily available for all personnel. | ☐ | ☐ | ☐ | ____________________ |
| Verify that personnel have been trained on the proper use, maintenance, and storage of PPE. | ☐ | ☐ | ☐ | ____________________ |
| Establish a procedure for cleaning and maintaining PPE. Ensure PPE is cleaned regularly. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________