InspectionReport

Tilt-Up Panel Erection Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Lifting Inserts & Rigging Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify lifting inserts are rated for at least 2x the panel's weight, accounting for dynamic loads. Check engineering documentation. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all lifting inserts for cracks, corrosion, or damage before each lift. Discard any suspect inserts. | ☐ | ☐ | ☐ | ____________________ |
| Ensure shackles, slings, and other rigging hardware are of the correct size and capacity for the panel weight. Verify WLL markings are legible. | ☐ | ☐ | ☐ | ____________________ |
| Confirm sling angles do not exceed the manufacturer's recommendations (typically 45 degrees). Use longer slings or a spreader bar if needed. | ☐ | ☐ | ☐ | ____________________ |
| If using a strongback, inspect it for straightness, weld integrity, and proper attachment to the panel. Ensure it's rated for the load. | ☐ | ☐ | ☐ | ____________________ |
| If using a spreader bar, verify its load rating exceeds the panel weight with a safety factor of at least 2. Inspect welds and connections. | ☐ | ☐ | ☐ | ____________________ |
| Verify the number and location of lifting points match the panel's engineered lifting plan. Do not deviate without approval. | ☐ | ☐ | ☐ | ____________________ |
| Confirm lifting inserts were installed per the manufacturer's specifications, including proper embedment depth and concrete consolidation. | ☐ | ☐ | ☐ | ____________________ |
| Review load test records for lifting inserts, if applicable. Ensure tests were performed by a qualified technician. | ☐ | ☐ | ☐ | ____________________ |
| Verify all rigging hardware has current inspection tags. Remove any rigging with expired or missing tags from service. | ☐ | ☐ | ☐ | ____________________ |

Crane & Operator Competency

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the crane operator is certified for the type and capacity of crane being used. Check NCCCO or equivalent certification. | ☐ | ☐ | ☐ | ____________________ |
| Review the crane's daily and annual inspection records. Ensure all maintenance is up-to-date and documented. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the crane's load chart is readily available and legible. The operator must understand how to use it. | ☐ | ☐ | ☐ | ____________________ |
| Verify the crane is properly leveled according to the manufacturer's specifications. Use bubble levels or electronic leveling systems. | ☐ | ☐ | ☐ | ____________________ |
| Ensure outriggers are fully extended and properly supported on suitable pads. Check for ground settlement. | ☐ | ☐ | ☐ | ____________________ |
| Barricade the crane's swing radius to prevent unauthorized personnel from entering the area. | ☐ | ☐ | ☐ | ____________________ |
| Establish clear communication signals between the crane operator, signal person, and ground crew. Use hand signals or radios. | ☐ | ☐ | ☐ | ____________________ |
| Verify the crane maintains a safe clearance distance from overhead power lines (at least 10 feet). | ☐ | ☐ | ☐ | ____________________ |
| Monitor wind speed. Cease lifting operations if wind speeds exceed the crane manufacturer's recommendations. | ☐ | ☐ | ☐ | ____________________ |
| Assess ground conditions for stability. Ensure the ground can support the crane's weight and the panel load. | ☐ | ☐ | ☐ | ____________________ |

Panel Bracing & Stability

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the bracing plan is approved by a qualified engineer and is readily available on site. | ☐ | ☐ | ☐ | ____________________ |
| Ensure bracing is installed according to the approved bracing plan, including proper spacing, angle, and connection details. | ☐ | ☐ | ☐ | ____________________ |
| Inspect bracing hardware (e.g., bolts, welds, anchors) for damage or corrosion. Ensure they are properly tightened. | ☐ | ☐ | ☐ | ____________________ |
| Calculate the wind load on the panels and verify the bracing system is adequate for the expected wind conditions. Re-evaluate daily. | ☐ | ☐ | ☐ | ____________________ |
| Verify bracing connections to the panel and the foundation are secure and properly installed. Check for loose bolts or damaged welds. | ☐ | ☐ | ☐ | ____________________ |
| Inspect ground anchors for proper embedment depth and pull-out resistance. Ensure they are installed in stable soil. | ☐ | ☐ | ☐ | ____________________ |
| Do not adjust or remove any bracing without the approval of a qualified engineer. Document all changes to the bracing system. | ☐ | ☐ | ☐ | ____________________ |
| Conduct daily inspections of the bracing system to identify any signs of damage, loosening, or instability. | ☐ | ☐ | ☐ | ____________________ |
| Verify panels are plumb after erection and before releasing the crane. Adjust bracing as needed to maintain plumbness. | ☐ | ☐ | ☐ | ____________________ |
| Ensure bracing is installed sequentially as panels are erected to maintain stability throughout the process. | ☐ | ☐ | ☐ | ____________________ |

Site Conditions & Housekeeping

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Ensure the work zone is clear of obstructions, including materials, equipment, and personnel not involved in the lifting operation. | ☐ | ☐ | ☐ | ____________________ |
| Establish a controlled access zone around the lifting area to prevent unauthorized pedestrian traffic. | ☐ | ☐ | ☐ | ____________________ |
| Verify the ground is level and stable in the panel staging area. Prevent panels from tipping or shifting. | ☐ | ☐ | ☐ | ____________________ |
| Monitor weather conditions, including wind speed, temperature, and precipitation. Adjust operations as needed to maintain safety. | ☐ | ☐ | ☐ | ____________________ |
| Ensure adequate lighting is provided in the work area, especially during evening or early morning operations. | ☐ | ☐ | ☐ | ____________________ |
| Maintain good housekeeping practices in the work area to prevent slips, trips, and falls. Remove debris and waste materials regularly. | ☐ | ☐ | ☐ | ____________________ |
| Ensure clear access for emergency vehicles and personnel. Keep pathways and roadways unobstructed. | ☐ | ☐ | ☐ | ____________________ |
| Verify communication systems (e.g., radios, cell phones) are functioning properly and readily available for emergency use. | ☐ | ☐ | ☐ | ____________________ |
| Ensure fall protection is in place for workers accessing the top of staged panels. Use guardrails or personal fall arrest systems. | ☐ | ☐ | ☐ | ____________________ |
| Ensure taglines are used to control panel movement during lifting and placement. Tagline operators must be properly trained. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify all personnel in the work area are wearing hard hats that meet ANSI Z89.1 standards. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all personnel are wearing safety glasses with side shields to protect against flying debris. | ☐ | ☐ | ☐ | ____________________ |
| Verify all personnel are wearing high-visibility clothing (e.g., vests, jackets) that meet ANSI 107 standards. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all personnel are wearing steel-toe boots that meet ASTM F2413 standards. | ☐ | ☐ | ☐ | ____________________ |
| Verify personnel are wearing appropriate gloves for the task, such as leather gloves for handling panels or rigging. | ☐ | ☐ | ☐ | ____________________ |
| Ensure hearing protection is available and used in areas with high noise levels (e.g., crane operation, concrete cutting). | ☐ | ☐ | ☐ | ____________________ |
| Inspect fall protection harnesses for damage or wear before each use. Ensure they fit properly and are correctly attached. | ☐ | ☐ | ☐ | ____________________ |
| If dust or other airborne contaminants are present, ensure personnel are using appropriate respiratory protection (e.g., N95 masks). | ☐ | ☐ | ☐ | ____________________ |
| Verify an eyewash station is readily available in case of eye contact with concrete or other irritants. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all PPE is in good condition and properly maintained. Replace damaged or worn PPE immediately. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________