InspectionReport

Roof Safety and Edge Protection Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

Jurisdiction: Adaptable. Covers OSHA 29 CFR 1926.501 (US) and UK Work at Height Regulations 2005 (WAHR). Adaptable to other jurisdictions. Apply only the section and thresholds for the governing jurisdiction; do not combine US and UK requirements as if they were one code.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Inspection header

| Field | Entry |
| --- | --- |
| Site name and location | ________________________________________________ |
| Inspector name and date | ________________________________________________ |
| Roof area / zone inspected | ________________________________________________ |
| Weather conditions | ________________________________________________ |

Edge protection systems

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Guardrail top rail height: OSHA 42 inches (±3) / UK WAHR minimum 950mm | ☐ | ☐ | ☐ | ____________________ |
| Intermediate (mid) rail: installed midway between top rail and walking level | ☐ | ☐ | ☐ | ____________________ |
| Toe boards: minimum 3.5 inches (89mm) high (OSHA) / 150mm (UK) | ☐ | ☐ | ☐ | ____________________ |
| Load resistance: withstands 200-lb downward and outward force (OSHA) | ☐ | ☐ | ☐ | ____________________ |
| Gap between top rail and intermediate rail / toe board: ≤ 470mm (UK) | ☐ | ☐ | ☐ | ____________________ |

Fall arrest systems

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Anchorage points: structurally sound, capable of 5,000 lbs per employee (OSHA) | ☐ | ☐ | ☐ | ____________________ |
| Anchor positioning: directly above work area (minimize swing fall) | ☐ | ☐ | ☐ | ____________________ |
| Connectors: self-closing and self-locking carabiners / snap hooks | ☐ | ☐ | ☐ | ____________________ |

Roof hazards

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Skylights: covered, secured, marked with 'HOLE' or 'COVER' | ☐ | ☐ | ☐ | ____________________ |
| Holes >6 feet above lower level: protected | ☐ | ☐ | ☐ | ____________________ |
| Leading edges: fall protection plan documented if guardrails impossible | ☐ | ☐ | ☐ | ____________________ |
| Roof surface condition: slippery, wet, damaged areas identified | ☐ | ☐ | ☐ | ____________________ |

Access and egress

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Ladders / stairs: secure, properly angled, extending above landing | ☐ | ☐ | ☐ | ____________________ |
| Roof hatches: self-closing, guarded when open | ☐ | ☐ | ☐ | ____________________ |

Inspection frequency and follow-up

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Daily inspection: Yes / No | ☐ | ☐ | ☐ | ____________________ |
| Inspection after high winds / heavy rain: Yes / No | ☐ | ☐ | ☐ | ____________________ |
| Defects noted with location and description | ☐ | ☐ | ☐ | ____________________ |
| Corrective action assigned | ☐ | ☐ | ☐ | ____________________ |
| Inspector signature | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________

References consulted

OSHA 29 CFR 1926.501 – Duty to have fall protection

UK HSE – Work at Height Regulations 2005

OSHA 29 CFR 1926.502 – System planning and criteria

UK HSE – Work at height: the law (INDG401)

Verify the current edition and local adoption before use.