InspectionReport

Smart Meter Installation Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Pre-Installation Assessment & Planning

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Was a pre-pull voltage check performed on the meter base using a calibrated voltage tester (e.g., Fluke T6-1000) to verify absence of voltage? Document readings and socket condition with photos. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect the meter base for signs of corrosion, arcing, or overheating (hot sockets). Document any defects with photos before proceeding. | ☐ | ☐ | ☐ | ____________________ |
| Assess the property for potential animal hazards (dogs, cats, etc.). Have a plan for safe interaction or containment (e.g., customer notification, use of deterrents). | ☐ | ☐ | ☐ | ____________________ |
| Verify that no backup power sources (generators, solar panels) are connected to the customer's electrical system without proper isolation or transfer switches. Confirm with homeowner. | ☐ | ☐ | ☐ | ____________________ |
| Evaluate weather conditions (rain, snow, extreme temperatures). Are conditions safe for electrical work? Postpone if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Establish clear communication protocols with the homeowner regarding the installation process, potential power outages, and safety precautions. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the new smart meter is the correct type and rating for the existing meter base and service. Check AMI system for correct meter number. | ☐ | ☐ | ☐ | ____________________ |
| Verify that emergency contact information (supervisor, utility dispatch) is readily available and accessible. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that a site-specific Lockout/Tagout (LOTO) procedure is in place and understood for disconnecting and reconnecting the meter. | ☐ | ☐ | ☐ | ____________________ |
| If a ladder is needed, is it the correct type (fiberglass, rated for electrical work) and in good condition (no damage, current inspection tag)? | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are properly rated and tested rubber insulating gloves (e.g., Class 0 or Class 00) being worn and inspected for damage before each use? Use leather protectors. | ☐ | ☐ | ☐ | ____________________ |
| Are safety glasses with side shields being worn to protect against arc flash and debris? | ☐ | ☐ | ☐ | ____________________ |
| Is appropriate arc-rated clothing (e.g., shirt, pants) being worn based on the potential arc flash hazard assessment? (Minimum ATPV rating of 4 cal/cm²) | ☐ | ☐ | ☐ | ____________________ |
| Are all hand tools being used (e.g., screwdrivers, pliers) insulated and rated for the voltage being worked on (1000V)? | ☐ | ☐ | ☐ | ____________________ |
| Are steel-toe boots being worn to protect against foot injuries? | ☐ | ☐ | ☐ | ____________________ |
| Is high-visibility clothing (e.g., vest or shirt) being worn to ensure visibility to others? | ☐ | ☐ | ☐ | ____________________ |
| Is hearing protection available and used if noise levels exceed 85 dBA? | ☐ | ☐ | ☐ | ____________________ |
| Is a face shield available and used for additional arc flash protection during meter removal and installation? | ☐ | ☐ | ☐ | ____________________ |

On-Site Safety Procedures

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is a clear and safe work zone established around the meter base to prevent unauthorized access? | ☐ | ☐ | ☐ | ____________________ |
| After LOTO, re-verify absence of voltage at the meter base using a calibrated voltage tester before commencing work. | ☐ | ☐ | ☐ | ____________________ |
| Are proper lifting techniques being used to prevent back injuries when handling the smart meter? | ☐ | ☐ | ☐ | ____________________ |
| Ensure the meter base is securely mounted and in good condition before installing the new smart meter. Repair or replace if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all connections and enclosures are properly weatherproofed to prevent moisture ingress. | ☐ | ☐ | ☐ | ____________________ |
| Verify the meter seal is intact and properly installed to prevent tampering. | ☐ | ☐ | ☐ | ____________________ |
| Maintain a clean and organized work area, free of debris and tripping hazards. | ☐ | ☐ | ☐ | ____________________ |
| Maintain open communication with the homeowner throughout the installation process, addressing any questions or concerns. | ☐ | ☐ | ☐ | ____________________ |
| After installation, verify the smart meter is operating correctly and transmitting data to the AMI system. | ☐ | ☐ | ☐ | ____________________ |
| Conduct a post-installation inspection to ensure all connections are secure, the meter is properly seated, and the area is clean. | ☐ | ☐ | ☐ | ____________________ |

Vehicle & Equipment Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the vehicle inspected daily (tires, lights, brakes, fluid levels) using a pre-trip inspection checklist? | ☐ | ☐ | ☐ | ____________________ |
| Is the vehicle parked safely, avoiding obstructions to traffic or pedestrian walkways? | ☐ | ☐ | ☐ | ____________________ |
| Is the ladder properly secured in the vehicle to prevent shifting or damage during transport? | ☐ | ☐ | ☐ | ____________________ |
| Are tools and equipment properly secured in the vehicle to prevent them from becoming projectiles in the event of an accident? | ☐ | ☐ | ☐ | ____________________ |
| Is a fully stocked first-aid kit available in the vehicle? | ☐ | ☐ | ☐ | ____________________ |
| Is a properly charged and inspected fire extinguisher available in the vehicle? | ☐ | ☐ | ☐ | ____________________ |
| Are traffic control devices (cones, signs) available and used when working near roadways? | ☐ | ☐ | ☐ | ____________________ |
| Is the voltage tester calibrated per manufacturer's recommendations and within the calibration period? Document calibration date. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________