InspectionReport

Haul Truck Pre-Shift Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Mobile Equipment - Brake System

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Air brake system reaches 120 psi within 2 minutes at 1200 RPM; low-pressure warning buzzer sounds correctly at 60 psi. (OSHA 56/57.14200) | ☐ | ☐ | ☐ | ____________________ |
| Brake block thickness meets or exceeds manufacturer's minimum specifications. (SAE J151) | ☐ | ☐ | ☐ | ____________________ |
| Parking brake effectively holds the vehicle on a 15% grade with a full load. (OSHA 56/57.14205) | ☐ | ☐ | ☐ | ____________________ |
| Service brakes provide smooth and even deceleration without pulling to one side. (OSHA 56/57.14205) | ☐ | ☐ | ☐ | ____________________ |
| Inspect all brake lines and connections for leaks or damage. (OSHA 56/57.14210) | ☐ | ☐ | ☐ | ____________________ |
| Retarder system (if equipped) functions correctly and provides adequate speed control on downhill grades. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| Brake cooling system (if equipped) is functioning correctly; coolant levels are adequate and no leaks are present. | ☐ | ☐ | ☐ | ____________________ |
| Emergency brake system is functional and can bring the vehicle to a complete stop in a controlled manner. (OSHA 56/57.14205) | ☐ | ☐ | ☐ | ____________________ |
| Automatic lubrication system is functioning correctly, ensuring proper lubrication of brake components. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| Brake pedal travel is within the manufacturer's specified range. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |

Mobile Equipment - Steering and Suspension

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect steering linkage for excessive wear, looseness, or damage. (OSHA 56/57.14210) | ☐ | ☐ | ☐ | ____________________ |
| Power steering fluid level is within the recommended range; check for leaks. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| Steering wheel free play is within acceptable limits (typically less than 2 inches). (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| Inspect suspension components (springs, shocks, struts) for damage or wear. (OSHA 56/57.14210) | ☐ | ☐ | ☐ | ____________________ |
| Tire condition is acceptable (no cuts, bulges, or excessive wear); tire pressure is within the manufacturer's specifications. (OSHA 56/57.14200) | ☐ | ☐ | ☐ | ____________________ |
| Wheel nuts are properly torqued according to the manufacturer's specifications. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| Articulation points are properly lubricated and show no signs of excessive wear or damage. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| Hydraulic cylinders (steering and suspension) are free from leaks and damage. (OSHA 56/57.14210) | ☐ | ☐ | ☐ | ____________________ |
| Steering stops are functioning correctly, preventing over-steering and damage to components. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect front axle alignment for any signs of misalignment. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |

Mobile Equipment - Visibility and Warning Systems

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| All mirrors are clean, properly adjusted, and free from damage to eliminate blind spots. (OSHA 56/57.14130) | ☐ | ☐ | ☐ | ____________________ |
| Backup alarm is functioning correctly and audible from a safe distance (typically 200 feet). (OSHA 56/57.14130) | ☐ | ☐ | ☐ | ____________________ |
| All lights (headlights, taillights, brake lights, turn signals) are functioning correctly. (OSHA 56/57.14130) | ☐ | ☐ | ☐ | ____________________ |
| Windshield is clean and free from cracks or damage that obstructs visibility. (OSHA 56/57.14130) | ☐ | ☐ | ☐ | ____________________ |
| Windshield wipers are functioning correctly and provide adequate visibility in inclement weather. (OSHA 56/57.14130) | ☐ | ☐ | ☐ | ____________________ |
| Horn is functioning correctly and audible. (OSHA 56/57.14130) | ☐ | ☐ | ☐ | ____________________ |
| Camera systems (if equipped) are functioning correctly and providing clear images. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| Two-way radio is functioning correctly and communication with dispatch is clear. (Site Specific) | ☐ | ☐ | ☐ | ____________________ |
| Fire suppression system (if equipped) is charged and ready for use; inspect nozzles for obstructions. (NFPA 121) | ☐ | ☐ | ☐ | ____________________ |
| Operator seatbelt is in good condition and functions properly. (OSHA 56/57.14206) | ☐ | ☐ | ☐ | ____________________ |

Mobile Equipment - Dump Body and Chassis

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Dump body is free from cracks, dents, or other damage that could compromise its structural integrity. (OSHA 56/57.14210) | ☐ | ☐ | ☐ | ____________________ |
| Dump body hinges and pivot points are properly lubricated and show no signs of excessive wear. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| Dump body hydraulic cylinders are free from leaks and damage. (OSHA 56/57.14210) | ☐ | ☐ | ☐ | ____________________ |
| Dump body locking mechanism is functioning correctly and securely locks the body in the lowered position. (OSHA 56/57.14210) | ☐ | ☐ | ☐ | ____________________ |
| Chassis is free from cracks, corrosion, or other damage that could compromise its structural integrity. (OSHA 56/57.14210) | ☐ | ☐ | ☐ | ____________________ |
| Access ladder to the operator's cab is in good condition and provides safe access. (OSHA 56/57.14206) | ☐ | ☐ | ☐ | ____________________ |
| Mud flaps are in place and in good condition to prevent debris from being thrown. (Local Regulations) | ☐ | ☐ | ☐ | ____________________ |
| Check for any signs of fluid leaks (oil, coolant, hydraulic fluid) and ensure proper spill containment measures are in place. (Environmental Regulations) | ☐ | ☐ | ☐ | ____________________ |
| Wheel chocks are available and in good condition for use during maintenance. (OSHA 56/57.14207) | ☐ | ☐ | ☐ | ____________________ |
| Bed liners (if equipped) are secure and in good condition to protect the dump body. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |

Documentation and PPE

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Review the previous inspection log for any outstanding issues or recurring problems. (OSHA 56/57.14212) | ☐ | ☐ | ☐ | ____________________ |
| Verify that the operator is properly certified and trained to operate the haul truck. (OSHA 56/57.14206) | ☐ | ☐ | ☐ | ____________________ |
| Equipment manual is available and accessible in the cab. (Manufacturer Specs) | ☐ | ☐ | ☐ | ____________________ |
| First-aid kit is present and adequately stocked. (ANSI Z308.1) | ☐ | ☐ | ☐ | ____________________ |
| Ensure required PPE (hard hat, safety glasses, high-visibility vest, steel-toe boots) are available and in good condition. (OSHA 56/57.14206) | ☐ | ☐ | ☐ | ____________________ |
| Review emergency procedures and evacuation routes with the operator. (Site Specific) | ☐ | ☐ | ☐ | ____________________ |
| Ensure the operator is aware of any potential hazards in the work area. (OSHA 1910.1200) | ☐ | ☐ | ☐ | ____________________ |
| Ensure the pre-shift inspection form is completed accurately and thoroughly. (Site Specific) | ☐ | ☐ | ☐ | ____________________ |
| Verify understanding of lockout/tagout procedures for maintenance. (OSHA 1910.147) | ☐ | ☐ | ☐ | ____________________ |
| Spill kit is readily available and properly stocked. (Environmental Regulations) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________