InspectionReport

Telecom Aerial Lift / Bucket Truck Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Mobile Equipment (Bucket Truck) Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Check tire pressure against manufacturer's specifications. All tires must be evenly inflated. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Inspect engine oil, coolant, hydraulic fluid, and brake fluid levels. Refill as needed. (Manufacturer's Specs) | ☐ | ☐ | ☐ | ____________________ |
| Verify all lights (headlights, taillights, brake lights, turn signals, hazard lights) and audible signals (horn, backup alarm) are functioning correctly. (OSHA 1910.67) | ☐ | ☐ | ☐ | ____________________ |
| Inspect steering mechanism for excessive play or binding. Ensure smooth and responsive steering. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Test service brakes and parking brake functionality. Service brakes must provide smooth, even stopping power. Parking brake must hold the truck on an incline. (OSHA 1910.67) | ☐ | ☐ | ☐ | ____________________ |
| Ensure windshield wipers and washers are operational and provide clear visibility. (Local DOT Regulations) | ☐ | ☐ | ☐ | ____________________ |
| Inspect mirrors for damage and ensure they are properly adjusted for clear visibility. (OSHA 1910.67) | ☐ | ☐ | ☐ | ____________________ |
| Verify the backup alarm is functioning and audible. (OSHA 1926.602(a)(9)(ii)) | ☐ | ☐ | ☐ | ____________________ |
| Test the horn to ensure it is functioning properly. (OSHA 1926.602(a)(9)(i)) | ☐ | ☐ | ☐ | ____________________ |
| Confirm a fully charged and inspected fire extinguisher is present and readily accessible. (OSHA 1910.157) | ☐ | ☐ | ☐ | ____________________ |

Outrigger and Stability System Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect outrigger pads for damage or deterioration. Ensure they are of adequate size and load rating for the ground conditions. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Verify smooth and complete extension and retraction of all outriggers. Check for leaks or binding. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Ensure outrigger locking mechanisms are functioning correctly and securely lock the outriggers in the extended position. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Verify the accuracy of the level indicators (bubble level or electronic). Truck must be level before boom operation. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Assess ground conditions for stability. Avoid soft soil, slopes exceeding manufacturer's limits, and uneven surfaces. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the slope is within the manufacturer's specified limits for safe operation. Use a clinometer if necessary. (Manufacturer's Specs) | ☐ | ☐ | ☐ | ____________________ |
| Test outrigger interlocks to ensure boom operation is prevented unless outriggers are properly deployed. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Inspect hydraulic lines and cylinders for leaks around the outriggers. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Identify and mitigate any proximity hazards (e.g., underground utilities, overhead obstructions) before deploying outriggers. (OSHA 1926 Subpart V) | ☐ | ☐ | ☐ | ____________________ |

Insulated Boom and Upper Controls Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Visually inspect the insulated boom sections for cracks, damage, or contamination. Report any defects immediately. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Inspect boom for any signs of bending, buckling, or weld cracks. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Inspect hydraulic hoses and fittings for leaks along the entire boom length. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Verify smooth and responsive operation of all boom controls (elevation, rotation, extension). (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Test the emergency stop button at both the upper and lower controls. Ensure it immediately stops all boom movement. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Confirm that the lower controls can override the upper controls in an emergency. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the bucket for cracks, damage, or debris. Ensure the bucket floor is clean and slip-resistant. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Inspect the bucket liner (if equipped) for damage or contamination. Ensure it is properly installed and provides adequate insulation. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Verify minimum approach distances (MAD) from energized power lines are maintained. Use a spotter if necessary. (OSHA 1926.950) | ☐ | ☐ | ☐ | ____________________ |
| Test boom interlocks to prevent operation outside of safe operating parameters (e.g., exceeding reach limits). (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE) and Fall Protection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect full-body harness for rips, tears, fraying, or damage. Ensure it fits properly. (OSHA 1926.502(d)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect lanyard for cuts, abrasions, or damage to the snap hooks. Use a lanyard of appropriate length. (OSHA 1926.502(d)) | ☐ | ☐ | ☐ | ____________________ |
| Inspect dielectric gloves for holes, tears, or contamination. Test gloves per manufacturer's instructions. (OSHA 1910.137) | ☐ | ☐ | ☐ | ____________________ |
| Ensure appropriate eye protection (safety glasses or face shield) is worn. (OSHA 1926.102) | ☐ | ☐ | ☐ | ____________________ |
| Verify hard hat is in good condition and properly fitted. (OSHA 1926.100) | ☐ | ☐ | ☐ | ____________________ |
| Ensure appropriate footwear (steel-toed boots) is worn. (OSHA 1926.96) | ☐ | ☐ | ☐ | ____________________ |
| Verify that the fall arrest system (harness and lanyard) is properly connected to the designated anchor point in the bucket. (OSHA 1926.502) | ☐ | ☐ | ☐ | ____________________ |
| Ensure all tools used in the bucket are secured with tool lanyards to prevent dropped objects. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |
| Confirm clothing is appropriate for the weather conditions and does not pose a snag hazard. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |

Documentation and Training

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the operator is certified and trained to operate the specific type of bucket truck. (OSHA 1910.67) | ☐ | ☐ | ☐ | ____________________ |
| Review previous inspection records to identify any recurring issues. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Review maintenance records to ensure the bucket truck has been properly maintained according to the manufacturer's recommendations. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Ensure the load chart is present and legible in the bucket truck. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Confirm the operator's manual is available in the bucket truck. (ANSI A92.2) | ☐ | ☐ | ☐ | ____________________ |
| Ensure a daily inspection log is being maintained and completed before each shift. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |
| Verify a rescue plan is in place in case of an emergency or fall from the bucket. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |
| Confirm a reliable method of communication (e.g., radio, cell phone) is available for emergencies. (Company Policy) | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________