InspectionReport

Tower Crane Daily Operator Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Structural Components

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify all mast bolts are present, tightened to the manufacturer's specified torque, and show no signs of corrosion or damage. Use calibrated torque wrench. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the slewing ring for cracks, excessive wear, or loose bolts. Check lubrication levels and ensure proper function. | ☐ | ☐ | ☐ | ____________________ |
| Examine all jib sections for any signs of bending, buckling, or weld cracks. Pay close attention to connection points. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all counterweights are securely attached and properly positioned according to the crane's load chart. Verify locking mechanisms are engaged. | ☐ | ☐ | ☐ | ____________________ |
| Using a level, verify the tower is plumb and within the manufacturer's specified tolerance. Note any deviations. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all tie-in supports to the building structure for proper attachment and condition. Ensure no signs of stress or damage. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect the crane's foundation for any signs of cracking, settling, or water damage. Report any concerns to the site engineer. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the access ladder for damaged rungs, loose connections, and proper fall protection (if applicable). | ☐ | ☐ | ☐ | ____________________ |
| Verify the crane is in weather-vaning mode when out of service and that the slewing brake is released. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all lubrication points are adequately greased according to the manufacturer's recommendations. | ☐ | ☐ | ☐ | ____________________ |

Wire Rope and Hoisting System

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Measure the wire rope diameter at multiple points to check for wear and ensure it meets the minimum allowable diameter specified by the manufacturer. Document measurements. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the wire rope for signs of birdcaging, kinking, crushing, or unstranding. Replace if any of these conditions are present. | ☐ | ☐ | ☐ | ____________________ |
| Examine the wire rope for corrosion, especially at points where it passes over sheaves or drums. Note the severity and location of any corrosion. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the hoist drum grooving for wear or damage that could cause the wire rope to spool unevenly. Ensure proper alignment. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all sheaves are free to rotate and are properly lubricated. Check for cracks, chips, or excessive wear. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the hook latch for proper function and ensure it securely closes. Replace if damaged or missing. | ☐ | ☐ | ☐ | ____________________ |
| Check the hook for any signs of deformation, cracks, or wear. Use a calibrated gauge to measure the throat opening. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that the wire rope is properly reeved according to the crane's load chart and reeving diagram. | ☐ | ☐ | ☐ | ____________________ |
| Inspect rope terminations (e.g., swaged sockets, clips) for proper installation and condition. Ensure they are free from corrosion and damage. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the load block for any signs of damage, including cracks, deformation, or excessive wear. Check the condition of the bearings. | ☐ | ☐ | ☐ | ____________________ |

Operational Controls and Safety Devices

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the Load Moment Indicator (LMI) is functioning correctly by performing a test lift with a known weight. Compare the LMI reading to the actual load. | ☐ | ☐ | ☐ | ____________________ |
| Test the Anti-Two-Block (ATB) system to ensure it activates before the hook block comes into contact with the boom tip. Confirm audible and visual alarms are functional. | ☐ | ☐ | ☐ | ____________________ |
| Test the hoist brake by lifting a load slightly off the ground and observing for any slippage. The brake should hold the load securely. | ☐ | ☐ | ☐ | ____________________ |
| Test the trolley brake (if applicable) to ensure it stops the trolley smoothly and prevents uncontrolled movement. | ☐ | ☐ | ☐ | ____________________ |
| Test the slewing brake to ensure it stops the crane's rotation smoothly and prevents uncontrolled slewing. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all limit switches (e.g., hoist limit, trolley limit, slewing limit) are functioning correctly and prevent movement beyond safe limits. | ☐ | ☐ | ☐ | ____________________ |
| Check the wind speed indicator (anemometer) and ensure it is functioning properly. Know the crane's maximum allowable wind speed. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all control levers for smooth operation and proper return to neutral. Ensure they are clearly labeled. | ☐ | ☐ | ☐ | ____________________ |
| Test the emergency stop button to ensure it immediately shuts down all crane functions. | ☐ | ☐ | ☐ | ____________________ |
| Verify the crane's horn or audible warning device is functioning properly. | ☐ | ☐ | ☐ | ____________________ |

Documentation and Communication

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Ensure the crane's load chart is readily available and legible to the operator. | ☐ | ☐ | ☐ | ____________________ |
| Review the crane's inspection records to verify that all required inspections have been performed and documented. | ☐ | ☐ | ☐ | ____________________ |
| Verify that the crane operator is certified or qualified to operate the specific type of crane. | ☐ | ☐ | ☐ | ____________________ |
| If a signal person is required, verify that they are qualified and understand the hand signals being used. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that a reliable communication system (e.g., radio, hand signals) is in place between the operator and signal person (if applicable). | ☐ | ☐ | ☐ | ____________________ |
| Review the site hazard assessment to identify any potential hazards that could affect crane operations (e.g., power lines, underground utilities). | ☐ | ☐ | ☐ | ____________________ |
| Review the lift plan (if required) to ensure it addresses all critical aspects of the lift, including load weight, rigging, and crane configuration. | ☐ | ☐ | ☐ | ____________________ |
| Ensure the daily inspection log is completed accurately and any defects are reported immediately. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________