InspectionReport

Tower Crane Erection & Section Climbing Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Pre-Erection Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify all mast sections are free from damage (dents, bends, corrosion) prior to assembly. Document findings with photos. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all bolts are of the correct grade and length as specified by the crane manufacturer's manual. Check against parts list. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the slewing ring for any signs of wear, cracks, or damage. Check lubrication points and grease levels. | ☐ | ☐ | ☐ | ____________________ |
| Examine all hydraulic hoses for cuts, abrasions, kinks, or leaks. Ensure hoses are properly rated for the system pressure. | ☐ | ☐ | ☐ | ____________________ |
| Assess the climbing frame for structural integrity, proper alignment, and secure locking mechanisms. Verify all pins and locking devices are in place. | ☐ | ☐ | ☐ | ____________________ |
| Verify the counter-jib is properly secured and that all counterweights are correctly installed and locked in place. Check locking pins. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all wire ropes for wear, broken strands, corrosion, and proper lubrication. Document findings and replace if necessary. | ☐ | ☐ | ☐ | ____________________ |
| Check all electrical connections for tightness, proper insulation, and grounding. Verify wiring diagrams are available and accurate. | ☐ | ☐ | ☐ | ____________________ |
| Confirm all lubrication points are accessible and properly greased according to the manufacturer's specifications. Document lubrication schedule. | ☐ | ☐ | ☐ | ____________________ |
| Test all safety devices, including limit switches, overload protection, and anti-collision systems. Document test results. | ☐ | ☐ | ☐ | ____________________ |

Bolt Torque Verification

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the torque wrench is calibrated and within its calibration period. Check calibration certificate. | ☐ | ☐ | ☐ | ____________________ |
| Confirm the correct bolt tightening sequence is followed as per the manufacturer's instructions. Use a documented procedure. | ☐ | ☐ | ☐ | ____________________ |
| Verify each bolt is torqued to the specified value using a calibrated torque wrench. Record torque values for each bolt. | ☐ | ☐ | ☐ | ____________________ |
| Inspect bolt threads for damage or contamination before tightening. Clean and lubricate threads as necessary. | ☐ | ☐ | ☐ | ____________________ |
| Ensure washers are correctly placed under bolt heads and nuts to distribute load evenly. Verify correct washer type. | ☐ | ☐ | ☐ | ____________________ |
| Implement a re-torque procedure after the initial tightening to account for settling. Document re-torque values. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect all bolts after torquing to ensure proper seating and no signs of thread stripping. Document observations. | ☐ | ☐ | ☐ | ____________________ |
| Verify that locking mechanisms (e.g., lock washers, cotter pins) are properly installed to prevent loosening. Check security. | ☐ | ☐ | ☐ | ____________________ |
| Maintain a detailed record of all torque values for each bolt, including date, time, and personnel performing the task. | ☐ | ☐ | ☐ | ____________________ |
| Check that the bolt protrusion length after tightening meets the manufacturer's specifications. Too little or too much protrusion can indicate issues. | ☐ | ☐ | ☐ | ____________________ |

Hydraulic System Inspection During Climbing

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Monitor hydraulic pressure during the climbing process to ensure it remains within the manufacturer's specified range. Record pressure readings. | ☐ | ☐ | ☐ | ____________________ |
| Verify the cylinder stroke length is consistent and within the specified limits during each climbing cycle. Measure stroke length. | ☐ | ☐ | ☐ | ____________________ |
| Check the hydraulic fluid level in the reservoir before and after each climbing operation. Top off as needed with correct fluid type. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all hydraulic connections, hoses, and cylinders for leaks during and after the climbing process. Repair leaks immediately. | ☐ | ☐ | ☐ | ____________________ |
| Listen for unusual noises from the hydraulic pump during operation. Unusual noises can indicate cavitation or other problems. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all hydraulic valves are operating smoothly and correctly. Check for sticking or binding. | ☐ | ☐ | ☐ | ____________________ |
| Test the emergency stop function of the hydraulic system before each climbing operation. Ensure it immediately halts the process. | ☐ | ☐ | ☐ | ____________________ |
| Monitor for cylinder drift during pauses in the climbing process. Excessive drift indicates internal leakage. | ☐ | ☐ | ☐ | ____________________ |
| Check the condition of the hydraulic filters and replace them according to the manufacturer's recommendations. Document filter changes. | ☐ | ☐ | ☐ | ____________________ |
| Monitor the temperature of the hydraulic fluid during operation. Overheating can damage components. | ☐ | ☐ | ☐ | ____________________ |

Wind Speed Monitoring and Weather Conditions

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the anemometer is calibrated and functioning correctly. Check calibration date. | ☐ | ☐ | ☐ | ____________________ |
| Continuously monitor wind speed at the crane's operating height using a calibrated anemometer. Record wind speed readings. | ☐ | ☐ | ☐ | ____________________ |
| Establish and follow a clear shutdown procedure when wind speeds exceed the crane manufacturer's specified limits. Document procedure. | ☐ | ☐ | ☐ | ____________________ |
| Monitor weather forecasts for impending storms or high winds. Plan work accordingly. | ☐ | ☐ | ☐ | ____________________ |
| Ensure the crane is free to weathervane (rotate freely) when out of service to minimize wind resistance. Check slewing ring brake release. | ☐ | ☐ | ☐ | ____________________ |
| Implement a tie-down procedure for the crane during periods of high winds or when the crane is out of service for extended periods. Document procedure. | ☐ | ☐ | ☐ | ____________________ |
| Assess visibility conditions before starting work. Ensure adequate visibility for safe crane operation. Stop work if visibility is impaired. | ☐ | ☐ | ☐ | ____________________ |
| Verify the crane is properly grounded to protect against lightning strikes. Check grounding connections. | ☐ | ☐ | ☐ | ____________________ |
| Consider the effects of extreme temperatures (hot or cold) on crane components and hydraulic systems. Adjust procedures accordingly. | ☐ | ☐ | ☐ | ____________________ |
| Establish a clear communication protocol for notifying personnel of changing weather conditions and implementing shutdown procedures. | ☐ | ☐ | ☐ | ____________________ |

Personnel and PPE

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify that all personnel involved in the erection and climbing process are properly trained and certified. Check training records. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all fall protection harnesses for damage, wear, and proper fit before each use. Remove damaged harnesses from service. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all lanyards and self-retracting lifelines (SRLs) for damage, wear, and proper functionality. Remove damaged equipment from service. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all personnel are wearing properly fitted hard hats that meet ANSI Z89.1 standards. Check for damage. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all personnel are wearing safety glasses or face shields to protect against flying debris. Verify compliance. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all personnel are wearing high-visibility clothing (e.g., vests, jackets) that meet ANSI 107 standards. Check for cleanliness. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all personnel have reliable communication equipment (e.g., radios) and that communication protocols are established and understood. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all tools and equipment used at height are properly tethered to prevent dropped objects. Inspect tethers for damage. | ☐ | ☐ | ☐ | ____________________ |
| Verify that personnel are wearing appropriate gloves for the task being performed. Check glove condition. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all personnel are wearing appropriate safety footwear that meets ASTM F2413 standards. Check for damage. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________