InspectionReport

High Voltage Transformer Substation Entry Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Pre-Entry Hazard Assessment & Planning

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Review the substation's single-line diagram to confirm equipment status and isolation points. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all relevant equipment is properly locked out and tagged out according to established procedures. | ☐ | ☐ | ☐ | ____________________ |
| Conduct an arc flash risk assessment to determine the incident energy and required PPE. | ☐ | ☐ | ☐ | ____________________ |
| Establish and clearly mark the arc flash boundary based on the arc flash risk assessment. | ☐ | ☐ | ☐ | ____________________ |
| Brief the entry team on the identified hazards, planned work, and emergency procedures. | ☐ | ☐ | ☐ | ____________________ |
| Confirm reliable communication methods (e.g., two-way radios) are available and functioning. | ☐ | ☐ | ☐ | ____________________ |
| Assess weather conditions (e.g., rain, lightning) and postpone entry if unsafe. | ☐ | ☐ | ☐ | ____________________ |
| Review and understand the grounding procedures specific to the substation and equipment. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that the substation emergency plan is readily available and understood. | ☐ | ☐ | ☐ | ____________________ |
| Inspect and clear the escape path of any obstructions. | ☐ | ☐ | ☐ | ____________________ |

Personal Protective Equipment (PPE) Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect arc-rated clothing for tears, burns, or contamination; ensure appropriate arc rating for the task. | ☐ | ☐ | ☐ | ____________________ |
| Test rubber insulating gloves per ASTM D120 standards; inspect for damage and proper voltage rating. | ☐ | ☐ | ☐ | ____________________ |
| Inspect leather protectors for rubber gloves for cuts, abrasions, and proper fit. | ☐ | ☐ | ☐ | ____________________ |
| Verify safety glasses meet ANSI Z87.1 standards and are free from scratches or cracks. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the arc-rated face shield for damage and ensure it is compatible with the arc flash hazard. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the hard hat for cracks, dents, and proper fit; ensure it meets ANSI Z89.1 standards. | ☐ | ☐ | ☐ | ____________________ |
| Inspect safety boots for proper insulation, steel toe protection, and slip resistance. | ☐ | ☐ | ☐ | ____________________ |
| Verify the voltage detector is functioning correctly using a known voltage source before entry. | ☐ | ☐ | ☐ | ____________________ |
| Inspect hearing protection (earplugs or earmuffs) for cleanliness and proper fit. | ☐ | ☐ | ☐ | ____________________ |
| Inspect FR balaclava for any tears or damage that could compromise arc flash protection. | ☐ | ☐ | ☐ | ____________________ |

Grounding & Bonding Verification

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify the grounding switch is in the closed (grounded) position for the equipment being worked on. | ☐ | ☐ | ☐ | ____________________ |
| Inspect grounding cables for damage, corrosion, and proper connection points. | ☐ | ☐ | ☐ | ____________________ |
| Test the continuity of the grounding system using a low-resistance ohmmeter; record readings. | ☐ | ☐ | ☐ | ____________________ |
| Verify that ground clamps are securely attached to clean, unpainted surfaces. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that step potential mitigation measures (e.g., grounding mats) are in place and properly positioned. | ☐ | ☐ | ☐ | ____________________ |
| Establish an equipotential zone around the work area using temporary grounding cables. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the substation grounding bus for corrosion and secure connections. | ☐ | ☐ | ☐ | ____________________ |
| Check the overall ground resistance of the substation grounding system; compare to acceptable limits. | ☐ | ☐ | ☐ | ____________________ |
| Verify that the size of the grounding conductors is adequate for the fault current. | ☐ | ☐ | ☐ | ____________________ |
| Document all grounding measurements and observations in the substation log. | ☐ | ☐ | ☐ | ____________________ |

Substation Environment & Equipment Inspection

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect transformer bushings and other oil-filled equipment for leaks; report any findings immediately. | ☐ | ☐ | ☐ | ____________________ |
| Listen for unusual noises (e.g., arcing, humming) from transformers or other equipment. | ☐ | ☐ | ☐ | ____________________ |
| Inspect transformer cooling fans for proper operation and airflow. | ☐ | ☐ | ☐ | ____________________ |
| Check for signs of overheating on transformers and other equipment (e.g., discolored paint, hot spots). | ☐ | ☐ | ☐ | ____________________ |
| Inspect insulators for cracks, chips, or contamination. | ☐ | ☐ | ☐ | ____________________ |
| Check for signs of animal intrusion (e.g., nests, droppings) and take appropriate measures. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the fire suppression system for proper functionality and charge. | ☐ | ☐ | ☐ | ____________________ |
| Verify adequate yard lighting for safe working conditions, especially during nighttime operations. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the security fencing for breaches or damage that could allow unauthorized entry. | ☐ | ☐ | ☐ | ____________________ |
| Verify that minimum clearance distances are maintained from energized equipment. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________