InspectionReport

Tunnel Boring Machine (TBM) Operations Safety Inspection Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

Gas Monitoring and Ventilation Systems

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all gas monitoring devices (CH4, H2S, CO, O2) calibrated within the last 30 days, with records available? | ☐ | ☐ | ☐ | ____________________ |
| Are gas monitors positioned within 10 feet of the tunnel face and at regular intervals along the tunnel bore? | ☐ | ☐ | ☐ | ____________________ |
| Is the ventilation system delivering the required airflow (CFM) to maintain gas concentrations below permissible exposure limits (PELs)? Verify with anemometer readings. | ☐ | ☐ | ☐ | ____________________ |
| Inspect ventilation ducts for damage (tears, punctures, collapses) and ensure proper sealing at joints. Repair or replace as needed. | ☐ | ☐ | ☐ | ____________________ |
| Verify the emergency ventilation system is functional and can be activated within 60 seconds. Conduct a drill. | ☐ | ☐ | ☐ | ____________________ |
| Are potential ignition sources (e.g., non-intrinsically safe electrical equipment) prohibited within 50 feet of areas where methane concentrations exceed 1%? | ☐ | ☐ | ☐ | ____________________ |
| Is the air velocity at the tunnel face maintained at a minimum of 30 feet per minute to dilute and remove contaminants? | ☐ | ☐ | ☐ | ____________________ |
| Confirm that the gas alarm system is audible and visible throughout the tunnel and that all personnel are trained on alarm response procedures. | ☐ | ☐ | ☐ | ____________________ |
| Verify continuous monitoring of oxygen levels to ensure adequate breathable air (19.5% - 23.5%). | ☐ | ☐ | ☐ | ____________________ |
| Are gas monitoring records maintained and readily available for review, documenting gas levels and ventilation system performance? | ☐ | ☐ | ☐ | ____________________ |

Fire Prevention and Suppression

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are appropriate fire extinguishers (Class A, B, C) readily accessible within 25 feet of potential fire hazards (e.g., welding areas, electrical panels, TBM cutter head)? | ☐ | ☐ | ☐ | ____________________ |
| Are fire extinguishers inspected monthly, with inspection tags indicating the last inspection date and condition? | ☐ | ☐ | ☐ | ____________________ |
| Are hot work permits required for all welding, cutting, and grinding activities, and are they properly completed and authorized? | ☐ | ☐ | ☐ | ____________________ |
| Are flammable liquids stored in approved containers and in designated, well-ventilated areas, away from ignition sources? | ☐ | ☐ | ☐ | ____________________ |
| Is the TBM equipped with a functional fire suppression system (e.g., automatic sprinkler system) that is regularly inspected and maintained? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency egress routes clearly marked and unobstructed, with emergency lighting functional and tested weekly? | ☐ | ☐ | ☐ | ____________________ |
| Verify the fire alarm system is functional and audible throughout the tunnel. Conduct a monthly test. | ☐ | ☐ | ☐ | ____________________ |
| Are designated fire brigade members trained in fire suppression techniques and emergency response procedures? | ☐ | ☐ | ☐ | ____________________ |
| Is the tunnel free of combustible materials (e.g., wood scraps, oily rags) that could contribute to a fire? | ☐ | ☐ | ☐ | ____________________ |
| Are welding blankets or screens used to contain sparks and prevent fires during welding operations? | ☐ | ☐ | ☐ | ____________________ |

Conveyor and Rotating Equipment Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are all moving parts of conveyors (e.g., rollers, belts, pulleys) adequately guarded to prevent contact? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency stop devices (e-stops) readily accessible along the entire length of the conveyor system and tested weekly? | ☐ | ☐ | ☐ | ____________________ |
| Are lockout/tagout procedures in place and followed for all maintenance and repair work on conveyors and rotating equipment? | ☐ | ☐ | ☐ | ____________________ |
| Inspect conveyor belts for proper alignment and tracking. Adjust as needed to prevent material spillage and belt damage. | ☐ | ☐ | ☐ | ____________________ |
| Are all rotating parts of machinery (e.g., pumps, motors, gearboxes) properly guarded to prevent contact? | ☐ | ☐ | ☐ | ____________________ |
| Are nip points on rotating equipment (e.g., belt and pulley interfaces) adequately guarded? | ☐ | ☐ | ☐ | ____________________ |
| Are maintenance records available for all conveyors and rotating equipment, documenting inspections, repairs, and lubrication schedules? | ☐ | ☐ | ☐ | ____________________ |
| Are warning signs posted near conveyors and rotating equipment, alerting personnel to potential hazards? | ☐ | ☐ | ☐ | ____________________ |
| Does the conveyor system have overload protection to prevent material jams and potential equipment damage? | ☐ | ☐ | ☐ | ____________________ |
| Are belt cleaners installed and functioning properly to prevent material buildup on conveyor belts? | ☐ | ☐ | ☐ | ____________________ |

Segment Ring Installation Safety

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Are segment rings handled using appropriate lifting equipment (e.g., vacuum lifters, cranes) with certified lifting capacity? | ☐ | ☐ | ☐ | ____________________ |
| Is lifting equipment inspected daily before use, with records maintained of inspections and maintenance? | ☐ | ☐ | ☐ | ____________________ |
| Are slings inspected before each use for damage (e.g., cuts, abrasions, kinks) and removed from service if necessary? | ☐ | ☐ | ☐ | ____________________ |
| Are segment rings stored on stable, level surfaces to prevent tipping or collapse? | ☐ | ☐ | ☐ | ____________________ |
| Is a personnel exclusion zone established during segment ring lifting and installation operations? | ☐ | ☐ | ☐ | ____________________ |
| Is a clear communication system in place between the crane operator, signal person, and installation crew? | ☐ | ☐ | ☐ | ____________________ |
| Are workers installing segment rings using appropriate fall protection equipment (e.g., harnesses, lanyards, lifelines) when working at heights above 6 feet? | ☐ | ☐ | ☐ | ____________________ |
| Are workers trained to recognize and avoid pinch points during segment ring installation? | ☐ | ☐ | ☐ | ____________________ |
| Are workers using the correct tools for segment ring installation (e.g., alignment pins, hydraulic jacks)? | ☐ | ☐ | ☐ | ____________________ |
| Is a written segment ring installation procedure available and followed by the installation crew? | ☐ | ☐ | ☐ | ____________________ |
| Is adequate lighting provided in the segment ring installation area to ensure safe working conditions? | ☐ | ☐ | ☐ | ____________________ |

Hyperbaric Intervention (If Applicable)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Is the hyperbaric chamber inspected daily for leaks, proper functioning of pressure controls, and communication systems? | ☐ | ☐ | ☐ | ____________________ |
| Are approved decompression tables readily available and followed during decompression procedures? | ☐ | ☐ | ☐ | ____________________ |
| Are all personnel involved in hyperbaric operations trained and certified in hyperbaric procedures and emergency response? | ☐ | ☐ | ☐ | ____________________ |
| Is a standby diver available and equipped to respond to emergencies during hyperbaric interventions? | ☐ | ☐ | ☐ | ____________________ |
| Is a reliable communication system in place between the hyperbaric chamber and the surface? | ☐ | ☐ | ☐ | ____________________ |
| Is medical support readily available for personnel undergoing hyperbaric interventions? | ☐ | ☐ | ☐ | ____________________ |
| Are emergency procedures in place and practiced for hyperbaric incidents (e.g., rapid ascent, chamber fire)? | ☐ | ☐ | ☐ | ____________________ |
| Is the oxygen level within the hyperbaric chamber continuously monitored to prevent oxygen toxicity? | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________