InspectionReport

Wind Turbine Nacelle & Blade Maintenance Safety Checklist

Documentation aid only. Adapt this resource to the site, manufacturer instructions, and applicable local requirements. It does not replace a competent person, professional judgment, legal advice, or a required statutory form.

| Site / Project: ______________________________ | Address: ______________________________ |
| --- | --- |
| Inspector name: ______________________________ | Company: ______________________________ |
| Date: ______________________________ | Time: ______________________________ |

Inspection checklist

I. Rope Access Equipment Inspection (Pre-Use)

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Inspect SPRAT/IRATA-approved harness for tears, cuts, abrasions, chemical damage, and UV degradation. Verify manufacturing date is within the acceptable lifespan. | ☐ | ☐ | ☐ | ____________________ |
| Examine lanyards (energy-absorbing and positioning) for fraying, cuts, burns, and proper stitching. Ensure hardware (snap hooks, D-rings) functions smoothly and locks securely. | ☐ | ☐ | ☐ | ____________________ |
| Visually and tactilely inspect ropes (main and belay lines) for cuts, abrasions, core damage, and excessive wear. Check for proper termination and end-to-end integrity. | ☐ | ☐ | ☐ | ____________________ |
| Verify ascenders and descenders (e.g., Petzl ID, GriGri) are free from damage, corrosion, and debris. Ensure proper cam engagement and smooth operation. Check friction surfaces for wear. | ☐ | ☐ | ☐ | ____________________ |
| Inspect carabiners and connectors for cracks, deformation, and proper gate function. Ensure locking mechanisms engage securely and are free from obstruction. | ☐ | ☐ | ☐ | ____________________ |
| Check helmet (ANSI Z89.1 Type II) for cracks, dents, and proper fit. Ensure chinstrap is functional and adjusted correctly. Verify impact absorption is not compromised. | ☐ | ☐ | ☐ | ____________________ |
| Inspect anchor slings (textile or wire rope) for cuts, abrasions, and proper stitching/termination. Verify load rating is appropriate for the intended use. | ☐ | ☐ | ☐ | ____________________ |
| Inspect rope protectors for damage and proper placement at all edges and potential abrasion points. Ensure adequate coverage to prevent rope damage. | ☐ | ☐ | ☐ | ____________________ |
| Verify all tools are properly tethered to prevent dropped objects. Inspect tethers for wear and tear, and ensure they are securely attached to both the tool and the technician's harness. | ☐ | ☐ | ☐ | ____________________ |
| Inspect emergency descent device (EDD) for proper function and readiness. Ensure technician is trained and competent in its use. | ☐ | ☐ | ☐ | ____________________ |

II. Nacelle Anchor Point Verification

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify that all nacelle anchor points are certified by a qualified engineer and have a valid inspection tag indicating load capacity and last inspection date. | ☐ | ☐ | ☐ | ____________________ |
| Visually inspect anchor points for corrosion, cracks, deformation, and any signs of damage. Ensure they are securely attached to the nacelle structure. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that tie-off points are easily accessible and free from obstructions that could interfere with rope access operations. | ☐ | ☐ | ☐ | ____________________ |
| Ensure adequate edge protection is in place at all anchor points to prevent rope abrasion and damage. | ☐ | ☐ | ☐ | ____________________ |
| Verify that the load rating of each anchor point meets or exceeds the required safety factor for the intended load (typically 5000 lbs or 22kN per person). | ☐ | ☐ | ☐ | ____________________ |
| Confirm that redundant anchor systems are in place for critical operations, providing a backup in case of primary anchor failure. | ☐ | ☐ | ☐ | ____________________ |
| Assess the spacing between anchor points to ensure adequate coverage and minimize swing fall potential. | ☐ | ☐ | ☐ | ____________________ |
| Inspect the yaw deck surface for slip, trip, and fall hazards. Ensure adequate lighting is available, especially during low-light conditions. | ☐ | ☐ | ☐ | ____________________ |
| Verify the lightning protection system is properly grounded and functioning correctly. Check for any signs of damage or corrosion. | ☐ | ☐ | ☐ | ____________________ |
| Inspect all access points to the nacelle (ladders, platforms) for structural integrity and proper safety features (handrails, non-slip surfaces). | ☐ | ☐ | ☐ | ____________________ |

III. Environmental Conditions & Weather Monitoring

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Monitor wind speed using a calibrated anemometer. Cease rope access operations if wind speeds exceed the manufacturer's recommended limits (typically 25-30 mph). | ☐ | ☐ | ☐ | ____________________ |
| Assess temperature conditions. Avoid rope access work in extreme heat or cold that could affect equipment performance or technician safety. | ☐ | ☐ | ☐ | ____________________ |
| Monitor weather forecasts for lightning activity. Suspend rope access operations if lightning is detected within a 10-mile radius. | ☐ | ☐ | ☐ | ____________________ |
| Evaluate precipitation conditions (rain, snow, ice). Ensure surfaces are not slippery and visibility is adequate. Provide appropriate weather protection for technicians. | ☐ | ☐ | ☐ | ____________________ |
| Assess visibility conditions. Ensure adequate lighting is available for safe rope access operations, especially during dawn, dusk, or overcast conditions. | ☐ | ☐ | ☐ | ____________________ |
| Verify that reliable communication systems (radios, cell phones) are available and functioning properly for all technicians and ground support personnel. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that the work area is properly barricaded and access is restricted to authorized personnel only. | ☐ | ☐ | ☐ | ____________________ |
| Assess potential environmental impacts (e.g., noise pollution, wildlife disturbance) and implement appropriate mitigation measures. | ☐ | ☐ | ☐ | ____________________ |
| Check for ice accumulation on blades and nacelle surfaces. De-icing procedures must be in place before work commences. | ☐ | ☐ | ☐ | ____________________ |
| Consider humidity levels, as high humidity can affect the grip and performance of ropes and equipment. | ☐ | ☐ | ☐ | ____________________ |

IV. Emergency Preparedness & Rescue Plan

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify that a written emergency rescue plan is readily available and understood by all personnel involved in the rope access operation. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that all necessary rescue equipment (e.g., rescue descender, hauling system, cutting tools) is readily available, inspected, and in good working order. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all technicians are equipped with suspension trauma straps and are trained in their proper use to mitigate the risk of orthostatic intolerance. | ☐ | ☐ | ☐ | ____________________ |
| Verify that a fully stocked first-aid kit is readily available at the work site, including supplies for treating common injuries such as cuts, burns, and sprains. | ☐ | ☐ | ☐ | ____________________ |
| Establish clear communication protocols with local emergency services (fire department, ambulance) and ensure they are aware of the rope access operation. | ☐ | ☐ | ☐ | ____________________ |
| Ensure that the designated rescue team is adequately trained and competent in performing rope rescue techniques. Conduct regular drills to maintain proficiency. | ☐ | ☐ | ☐ | ____________________ |
| Verify that emergency contact information (e.g., site supervisor, medical personnel, emergency services) is readily available and easily accessible. | ☐ | ☐ | ☐ | ____________________ |
| Identify and clearly mark evacuation routes from the nacelle in case of an emergency. Ensure that all personnel are familiar with these routes. | ☐ | ☐ | ☐ | ____________________ |
| Ensure appropriate fire suppression equipment (fire extinguishers) are readily available and inspected. Personnel must be trained in their use. | ☐ | ☐ | ☐ | ____________________ |
| Implement spill control measures to contain and clean up any potential spills of hazardous materials (e.g., lubricants, hydraulic fluid). | ☐ | ☐ | ☐ | ____________________ |

V. Documentation & Training Records

| Item | Pass | Fail | N/A | Notes |
| --- | --- | --- | --- | --- |
| Verify that all rope access technicians possess valid SPRAT or IRATA certifications and that their certifications are current. | ☐ | ☐ | ☐ | ____________________ |
| Review equipment inspection logs to ensure that all rope access equipment has been inspected regularly and that any defects have been addressed. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that a site-specific risk assessment has been conducted and that all identified hazards have been addressed in the work plan. | ☐ | ☐ | ☐ | ____________________ |
| Review the Job Safety Analysis (JSA) or Task Hazard Analysis (THA) for the specific maintenance task being performed. Ensure all technicians are familiar with the JSA/THA. | ☐ | ☐ | ☐ | ____________________ |
| Verify that all technicians have received adequate training in rope access techniques, fall protection, rescue procedures, and the use of personal protective equipment. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all technicians have undergone medical evaluations to ensure they are physically fit to perform rope access work. | ☐ | ☐ | ☐ | ____________________ |
| Review equipment maintenance records to ensure that all rope access equipment is properly maintained and serviced according to the manufacturer's recommendations. | ☐ | ☐ | ☐ | ____________________ |
| Review incident reports from previous rope access operations to identify any recurring hazards or areas for improvement. | ☐ | ☐ | ☐ | ____________________ |
| Ensure all required permits to work (e.g., hot work, confined space entry) are obtained and properly authorized before commencing work. | ☐ | ☐ | ☐ | ____________________ |
| Confirm that all personnel have completed a site-specific orientation covering hazards, emergency procedures, and site rules. | ☐ | ☐ | ☐ | ____________________ |

Findings

| # | Item | Location | Severity | Action required |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Signature

Inspector signature: ______________________________    Name: ______________________________

Date: ______________________________